Data Sheet

SFP28 25G SR

Features

- Supports 25.78Gb/s bit rate

- Transmission distance up to 70M (OM3),100M(OM4)

- LC optical cable connection

➢ +3.3V single power supply

- Low power consumption ＜1W

- Operating case temperature 0℃~70℃

- RoHS compliant Applications

- 25G Ethernet

- Absolute Maximum Ratings

| Parameters | Symbol | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| Storage Temperature | Tstg | -40 | 85 | ℃ |
| Operating Humidity | RH | 5 | 85 | % |
| Supply Voltage | Vcc | 0 | 3.6 | V |

- Optical transmitter Characteristics

Unless specified else, the specifications below are defined at TC=25±3℃

| Parameters | Symbol | Min. | Typ. | Max. | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Output Optical Power | Pf | -6 | --- | 2.4 | dBm | CW, Iop=6mA |
| Center Wavelength | λc | 840 | 850 | 860 | nm | CW, Iop=6mA |
| RMS Spectral Width | Δ λ | --- | --- | 0.65 | nm | CW, Iop=6mA |
| Extinction Ratio | ER | 2 | --- | --- | dB |  |
| Power Supply | Vcc | 3.14 | 3.3 | 3.47 | V | --- |
| Input Differential Impedance | Zin | 90 | 100 | 110 | Ω | Tc=25℃ |

| Differential Input Voltage | Vppin | 200 | - | 900 | mV | Tc=25℃ |
| --- | --- | --- | --- | --- | --- | --- |
| Bit Rate | BR | - | 25.78 | - | Gbps |  |

- Optical Receiver Characteristics

Unless specified else, the specifications below are defined at TC=25±3℃

| Parameters | Symbol | Min. | Typ. | Max. | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Wavelength | λ | 840 | 850 | 860 | nm | --- |
| Overload Input Optical Power | Pin | 2.4 | --- | --- | dBm | --- |
| Sensitivity | Sen | --- | --- | -10.3 | dBm | PRBS31, BER=5E- 5, @25.78Gbps |
| Power Supply | Vcc | 3.14 | 3.3 | 3.47 | V | --- |
| Output Differential Impedance | Zout | 90 | 100 | 110 | Ω | Tc=25℃ |
| Differential Output Voltage | Vppout | 200 | - | 900 | mV | Tc=25℃ |
| Los Assert | Losa | -30 |  |  | dBm |  |
| Los De-Assert | Losd |  |  | -12 | dBm |  |
| Los Hysteresis | Losh | 0.5 |  |  | dB |  |

- Electrical Connections

PIN Description

| 1 | VEET | Module Transmitter Ground |
| --- | --- | --- |
| 2 | TX_FAULT | Module Transmitter Fault |
| 3 | TX_DISABLE | Transmitter Disable; Turns off transmitter laser output |
| 4 | SDA | 2-Wire Serial Interface Data Line (MOD-DEF2) |
| 5 | SCL | 2-Wire Serial Interface Clock (MOD-DEF1) |
| 6 | MOD_ABS | ModuleAbsent, connected to VEET or VEER in the module |
| 7 | RS0 | Rate Select 0, optionally controls SFP+ module receiver |
| 8 | RX_LOS | Receiver Loss of Signal Indication (In FC designated as Rx_LOS and in Ethernet designated as NOT Signal Detect) |
| 9 | RS1 | Rate Select 1, optionally controls SFP+ module transmitter |
| 10 | VEER | Module Receiver Ground |
| 11 | VEER | Module Receiver Ground |
| 12 | RD- | Receiver Inverted Data Output |
| 13 | RD+ | Receiver Non-Inverted Data Output |
| 14 | VEER | Module Receiver Ground |
| 15 | VCCR | Module Receiver 3.3 V Supply |
| 16 | VCCT | Module Transmitter 3.3 V Supply |
| 17 | VEET | Module Transmitter Ground |
| 18 | TD+ | Transmitter Non-Inverted Data Input |
| 19 | TD- | Transmitter Inverted Data Input |
| 20 | VEET | Module Transmitter Ground |

- Mechanical Outline Dimensions

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,

Donghu High-tech Zone, Wuhan City