Product Specification

25G SFP28 10km Dual LC Transceivers

PRODUCT FEATURES

- Operating data rate up to 25.78Gbps

- Rate Adaptation

- Up to 10km transmission distance

- High sensitivity Pin photodiode and TIA

- LC duplex connector

- Hot pluggable 20pin connector

- Low power consumption <1.2 W

- Single +3.3V±5% power supply

- Compliant with SFF-8472 &IEEE 802.3cc

- Fully RoHS Compliant

- Operating temperature range:

Commercial: 0°C to +70°C

Industrial: -40°C to +85°C

Application

- Data Center Backbone

- Ethernet Switches

- High-speed Servers

- High-performance Computing Clusters

- SAN, Routers, Hubs, Load Balancer

Ordering information

| Part Number | Product Description |
| --- | --- |
| JH-25G-SFP28-LR-10-C | 25G SFP28 10km Transceivers, C-TEMP |
| JH-25G-SFP28-LR-10-I | 25G SFP28 10km Transceivers, I-TEMP |

DESCRIPTION

The JH-25G-SFP28-LR-10 Transceiver is intended for 10km reach service 25.78Gb/s 1310nm single mode high-speed communications equipment where low-cost, extraordinary performance and reliability are essential. It consumes low power, operates base on 3.3V DC power supply and is offered in the industrial temperature range. They are compliant with SFP28 MSA, SFF-8431 and SFF-8432.

The low jitter and low bit error rate optical assembly features a DML laser transmitter and PIN/TIA receiver. It utilizes internal clock and data recovery (CDR) units on transmitter and the receiver chains for low jitter compliance. The differential AC coupled Tx and Rx data interfaces are CML compatible. The device is Class I laser safety compliant.

ABSOLUTE MAXIMUM RATINGS

| Parameter | Symbol | Conditions | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TStorage |  | -40 | +85 | °C |
| Relative Humidity | RH |  | 0 | +85 | % |

RECOMMENDED OPERATING CONDITIONS

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Case Temperature | Tc | JH-25G-SFP28-LR-10-C | 0 |  | 70 | °C |
|  |  | JH-25G-SFP28-LR-10-I | -40 |  | 85 |  |
| Power Supply Voltage | VCC |  | 3.15 | 3.30 | 3.45 | V |
| Bit Rate | BR |  |  | 25.78125 |  | Gbps |
| Bit Error Ratio | BER |  |  |  | 5*10-5 |  |
| Max Supported Link Length | L |  |  |  | 10 | km |

ELECTRICAL CHARACTERISTICS

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Power Consumption |  |  |  |  | 1.2 | W |
| Supply Current | Icc |  |  |  | 350 | mA |

TRANSMITTER CHARACTERISTICS

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength | λ |  | 1290 |  | 1330 | nm |
| Side-mode Suppression Ratio | SMSR |  | 30 |  |  | dB |
| Average Optical Power | Pavg |  | -7 |  | 2.0 | dBm |
| Optical Modulation Amplitude | TxOMA |  | -4 |  |  | dBm |
| Transmitter and Dispersion Penalty | TDP |  |  |  | 1 | dB |
| Average Launch Power of OFF Transmitter | Poff |  |  |  | -30 | dBm |
| Extinction Ratio | ER |  | 3.5 |  |  | dB |
| Optical Return Loss Tolerance |  |  |  |  | 11 | dB |
| Transmitter Reflectance |  |  |  |  | -12 | dB |

RECEIVER CHARACTERISTICS

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength | λ |  | 1260 |  | 1360 | nm |
| Damage Threshold |  |  | 3 |  |  | dBm |
| Receive Power Overload |  |  |  |  | 2 | dBm |
| Receiver Reflectance |  |  |  |  | -26 | dB |
| Receiver Sensitivity | S | Note1 |  |  | -12 | dBm |
| LOS Assert | LOSA |  | -30 |  |  | dBm |
| LOS De-Assert | LOSD |  |  |  | -17 | dBm |
| LOS Hysteresis |  |  | 0.5 |  |  | dB |

Note1: Measured at 25.78Gb/s, ER>3.5dBm, PRBS 231-1 and BER better than or equal to 5E-5;

Electric Ports Definition

| Parameter | Symbol | Unit | Min | Typ | Max | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Input Differential Impedance | RIN | Ω |  | 100 |  |  |
| Single-ended Data Input Swing | VIN | mVp-p | 90 |  | 450 |  |
| Transmit Disable Voltage | VDIS | V | 2 |  | VCCHOST |  |
| Transmit Enable Voltage | VEN | V | VEE |  | VEE+0.8 |  |
| Transmit Fault Assert Voltage | VFA | V | 2 |  | VCCHOST |  |
| Transmit Fault De-Assert Voltage | VFDA | V | VEE |  | VEE+0.4 |  |
| Receiver |  |  |  |  |  |  |
| Single-ended Data Output Swing | VOD | mVp-p | 200 |  | 450 |  |
| LOS Fault | VLOSFT | V | 2 |  | VCCHOST |  |
| LOS Normal | VLOSNR | V | VEE |  | VEE+0.4 |  |

PIN Assignment

Figure 1.Pin function definitions

Table 1: Transceiver pin descriptions

| Pin Number | Symbol | Name | Description |
| --- | --- | --- | --- |
| 1,17,20 | VeeT | Transmitter Signal Ground | Connected to signal ground on the host board. |
| 2 | TX Fault | Transmitter Fault Out (OC) | Module transmitter fault output. |
| 3 | TX Disable | Transmitter Disable In (LVTTL) | Module transmitter disable control. |
| 4 | SDA | Module Definition Identifiers | Serial ID with SFF 8472 Diagnostics Module Definition pins should be pulled up to Host Vcc with 10 kΩ resistors. |
| 5 | SCL |  |  |
| 6 | MOD-ABS |  |  |
| 7 | RS0 | Receiver Rate Select (LVTTL) Transmitter Rate Select (LVTTL) | Rate select 0(Rx):Low=CDR Bypass ; High=CDR Select Rate select 1(Tx):Low=CDR Bypass ; High=CDR Select |
| 9 | RS1 |  |  |
| 8 | LOS | Loss of Signal Out (OC) | Receiver loss of signal. |
| 10,11,14 | VeeR | Receiver Signal Ground | Connected to signal ground on the host board. |
| 12 | RD- | Receiver Negative DATA Out (CML) | Receiver inverted data output, internally AC coupled and terminated |
| 13 | RD+ | Receiver Positive DATA Out (CML) | Receiver non-inverted data output, internally AC coupled and terminated. |
| 15 | VccR | Receiver Power Supply | Receiver Power 3.3V Supply. |
| 16 | VccT | Transmitter Power Supply | Transmitter Power 3.3V Supply. |
| 18 | TD+ | Transmitter Positive DATA In (CML) | Transmitter non-inverted data input, internally AC coupled and terminated. |
| 19 | TD- | Transmitter Negative DATA In (CML) | Transmitter inverted data Input, internally AC coupled and terminated. |

Recommended Interface Circuit

Figure 2. Typical application circuit

Mechanical Dimensions

Figure 3. Module Mechanical Dimensions

Digital Diagnostics Functions

As defined by the SFF-8472, Our SFP28 transceivers provide digital diagnostic functions via a 2-wire serial interface, which allows real-time access to the following operating parameters:

- Transceiver temperature

- Laser bias current

- Transmitted optical power

- Received optical power

- Transceiver supply voltage

It also provides a sophisticated system of alarm and warning flags, which may be used to alert end-users when particular operating parameters are outside of a factory-set normal range. The operating and diagnostics information is monitored and reported by a Digital Diagnostics Transceiver Controller (DDTC) inside the transceiver, which is accessed through the 2-wire serial interface. The memories are organized as a series of 8-bit data words that can be addressed individually or sequentially. The 2-wire serial interface provides sequential or random access to the 8 bit parameters, addressed from 0x00h to the maximum address of the memory. For more detailed information, including memory map definitions, please refer the SFF-8472 documentation.

Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City