QSFP-40G-JH

Bi-Directional, QSFP+ Pluggable, Duplex LC 40 Gigabit Ethernet Fiber-Optics Transceiver

Description

The QSFP-40G-JH is a Four-Channel, Pluggable, LC Duplex, Fiber-Optic QSFP+ Transceiver for 40 Gigabit Ethernet Applications. This transceiver is a high  performance module for short-range duplex data communication and interconnect applications. It integrates four electrical data lanes in each direction into transmission over a single LC duplex fiber optic cable. Each electrical lane operates at 10.3125 Gbps and conforms to the 40GE XLPPI interface.

The QSFP-40G-JH transceiver internally multiplexes an XLPPI 4x10G interface into two 20Gb/s electrical channels, transmitting and receiving each optically over one simplex LC fiber using bi-directional optics. This results in an aggregate bandwidth of 40Gbps into a duplex LC cable. This allows reuse of the installed LC duplex cabling infrastructure for 40GbE application.

shao and 150m using OM4 optical fiber are supported. These modules are designed to operate over multimode fiber systems using a nominal wavelength of 850nm on one end and 900nm on the other end. The electrical interface uses a 38 contact QSFP+ type edge connector. The optical interface uses a conventional LC duplex connector. This module incorporates proven integrated circuit and VCSEL technology to provide reliable long life, high performance, and consistent service.

Features

- Compliant to the 40GbE XLPPI electrical specification per IEEE 802.3ba-2010

- Compliant to QSFP+ SFF-8436 Specification

- Power Level 4: Max Power 3.5W

- Class 1M Eye Safety

- High port density: 21mm horizontal port pitch

- Operates at 10.3125 Gbps per electrical channel with 64b/66b  encoded data

- Links up to 100m using OM3 and 150m using OM4 optical fiber

 +10 to +70ºC case temperature operating range

- Hot pluggable transceiver for ease of installation and servicing

- Two wire Serial (TWS) Management Interface with maskable interrupts for expanded  functionality

- Utilizes a standard LC duplex fiber cable allowing reuse of existing cable  infrastructure

Applications

- 40 Gigabit Ethernet interconnects

- Datacom/Telecom switch & router connections

- Data aggregation and backplane applications

- Proprietary protocol and density applications

QSFP Electrical Connector

4X10G CDR

Din[3:0][p/n] (8)

40GE XLPPI Electrical Interface

SCL	 SDA

Mod Sel

2:4 DeMux

LPMode ModPresL ResetL IntL

Dout[3:0][p/n] (8)

4:2 Mux

RX  λ1

20G Laser Driver + TI

A

20G

20G

20G VCSEL, PIN and Bidirectional optics

20G

Gearbox IC

TX λ2

20G BiDi Optical

TX λ1

LC Duplex Optical Interface

20G BiDi Optical

uC

RX λ2

2X20G CDR

20G Laser Driver + TI

A

20G

20G VCSEL, PIN and Bidirectional optics

Figure 1. Transceiver Block Diagram

Transmitter

The optical transmitter portion of the transceiver (see Fig- ure 1) incorporates a 4-channel XLPPI input buffer, elec- trical multiplexer, two laser drivers and two high speed VCSELs (Vertical Cavity Surface Emitting Laser). The trans- mitter is designed for EN 60825 and CDRH Class 1M eye safety compliance. The Tx Input Buffer provides XLPPI compatible differential inputs presenting a nominal dif- ferential input impedance of 100 Ohms. AC coupling capacitors are located inside the QSFP+ module and are not required on the host board. For module control and interrogation, the control interface (LVTTL compatible) in- corporates a Two Wire Serial (TWS) interface of clock and data signals.

Receiver

The optical receiver portion of the transceiver (see Figure

1) incorporates two high speed PIN photodiodes, TIAs, signal processors, electrical de-multiplexer and 4 chan- nel electrical output buffer blocks. The Rx Output Buffer provides XLPPI compatible differential outputs for the high speed electrical interface presenting nominal single- ended output impedances of 50 Ohms to AC ground and 100 Ohms differentially that should be differentially termi- nated with 100 Ohms. AC coupling capacitors are located inside the QSFP+ module and are not required on the host board.

The electrical output will squelch for loss of input signal (unless squelch is disabled) and channel de-activation through TWS interface. To reduce the need for polling, a hardware interrupt signal INTL is provided to inform hosts of an assertion of LOS or Tx_FAULT.

Host Board

(Only 2 electrical channels/1 optical channel shown for simplicity)

20G

Tx

ASIC (SerDes)

1:2

De -Mux

20G

R x

2:1

Mux

Host Edge Card Connector

Module Card Edge (Host Interface)

Optical Connector/Port (Optical Interface)

Figure 2. Application Reference Diagram

High Speed Electrical Signal Interface

Figure 2 shows the interface between an ASIC/SerDes and the QSFP+ module. For simplicity, only two electrical chan- nels and one optical channel is shown. The high speed signal lines are AC-coupled 100 Ohm differential lines. The AC coupling is inside the QSFP+ module and not required on the host board. The 100 Ohm differential terminations are inside the QSFP+ module for the transmitter lines and at the host ASIC/SerDes for the Receiver lines. All transmit- ter and receiver electrical channels are compliant to mod- ule XLPPI specifications per IEEE 802.3ba.

Control  Signal Interface

The module has the following low speed signals for con- trol and status: ModSelL, LPMode, ResetL, ModPrsL, IntL. In addition, there is an industry standard two wire serial interface scaled for 3.3 volt LVTTL. It is implemented as a slave device. Signal and timing characteristics are further defined in the Control Interface section. The registers of the serial interface memory are defined in the Memory Map section and the corresponding  QSFP+ Memory Map  document.

Regulatory & Compliance Issues

Various standard and regulations apply to the modules. These include eye-safety, EMC, ESD and RoHS. See the Regulatory Section for details regarding these and com- ponent recognition.  Please note the transmitter module  is a Class 1M laser product – DO NOT VIEW RADIATION DIRECTLY  WITH  OPTICAL  INSTRUMENTS.   See Regulatory

Compliance Table  for details.

Package Outline

The module is designed to meet the package outline de- fined in the QSFP+ SFF-8436 Specification. See the pack- age outline and host board footprint figures (Figures 13

- 16) for details.

Handling

The transceiver module can be damaged by exposure to current surges and over voltage events. Care should be taken to restrict exposure to the conditions defined in the Absolute Maximum Ratings. Wave soldering, reflow sol- dering and/or aqueous wash process with the modules on board are not recommended. Normal handling precau- tions for electrostatic discharge sensitive devices should be observed.

Each module is supplied with an inserted port plug for protection of the optical ports. This plug should always be in place whenever a fiber cable is not inserted.

The optical connector includes recessed elements that are exposed whenever a cable or port plug is not inserted. Prior to insertion of a fiber optic cable, it is recommended that the cable end be cleaned to avoid contamination from the cable plug. The port plug ensures the optics re- mains clean and no additional cleaning should be needed. In the event of contamination, standard LC port cleaning method may be used.

Absolute Maximum Ratings

Stress in excess of any of the individual Absolute Maximum Ratings can cause immediate catastrophic damage to the module even if all other parameters are within Recommended Operating Conditions. It should not be assumed that limiting values of more than one parameter can be applied to the module concurrently. Exposure to any of the Absolute Maximum Ratings for extended periods can adversely affect reliability.

| Parameter | Symbol | Min | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -40 | 85 | °C |  |
| 3.3 V Power Supply Voltage | VCC | -0.5 | 3.6 | V |  |
| Data Input Voltage – Single Ended |  | -0.5 | VCC+0.5 | V |  |
| Data Input Voltage – Differential | ¦Vdip - Vdin¦ |  | 1.0 | V |  |
| Control Input Voltage | Vi | -0.5 | VCC+0.5, 3.6 | V |  |
| Control Output Current | IO | -20 | 20 | mA |  |
| Relative Humidity | RH | 5 | 95 | % |  |

Note:

1. This is the maximum voltage that can be applied across the differential inputs without damaging the input circuitry.

Recommended  Operating  Conditions

Recommended Operating Conditions specify parameters for which the optical and electrical characteristics hold un- less otherwise noted. Optical and electrical characteristics are not defined for operation outside the Recommended Operating Conditions where reliability is not implied and damage to the module may occur for such operation over an extended period of time.

| Parameter | Symbol | Min | Typ | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Case Temperature | TC | +10 |  | +70 | °C | 1 |
| 3.3 V Power Supply Voltage | VCC | 3.1 | 3.3 | 3.47 | V |  |
| Signal Rate per Electrical Channel (4 x 10GBd) |  |  | 10.3125 |  | GBd | 2 |
| Signal Rate per Optical Channel (2 x 20GBd) |  |  | 20.625 |  | GBd | 2 |
| Control* Input Voltage High | Vih | 2 |  | VCC+.3 | V |  |
| Control* Input Voltage Low | Vil | -0.3 |  | 0.8 | V |  |
| Two Wire Serial (TWS) Interface Clock Rate |  |  |  | 400 | kHz |  |
| Power Supply Noise |  |  |  | 50 | mVpp | 3 |
| Receiver Differential Data Output Load |  |  | 100 |  |  |  |
| Fiber Length for OM3 |  | 0.5 |  | 100 | m | 4 |
| Fiber Length for OM4 |  | 0.5 |  | 150 | m | 4 |

- Control signals, LVTTL (3.3 V) compatible Note:

- The position of case temperature measurement is shown in Figure 8.

- 64b/66b encoding is  assumed.

- Power Supply Noise is defined as the peak-to-peak noise amplitude over the frequency range at the host supply side of the recommended power supply filter with the module and recommended filter in place. Voltage levels including peak-to-peak noise are limited to the recommended operating range of the associated power supply. See Figure 9 for recommended power supply filter.

- Connection and splice loss of 1.5dB is allocated. Modal Bandwidth for OM3 fiber: 2000 MHz.km for 850nm channel, 1400MHz.km for 900nm channel.  Modal Bandwidth for OM4 fiber: 3700 MHz.km for 850nm channel, 1500MHz.km for 900nm  channel

Transceiver  Electrical  Characteristics*

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted. Typical values are for Tc = 40˚C, Vcc = 3.3 V

| Parameter | Symbols | Min | Typ | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Transceiver Power Consumption |  |  |  | 3.5 | W |  |
| Transceiver Power Supply Current |  |  |  | 1150 | mA |  |
| Transceiver Power On Initialization Time | tPWR INIT |  |  | 2000 | ms | 1 |

- For control signal timing including ModSelL, LPMode, ResetL, ModPrsL, IntL, SCL and SDA see Control Interface Section. Note:

1. Power On Initialization Time is the time from when the supply voltages reach and remain above the minimum Recommended Operating Conditions to the time when the module enables TWS access.  The module at that point is fully functional.

Transmitter Electrical Characteristics

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted. Typical values are for Tc = 40˚C, Vcc = 3.3 V

| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| LOS Assert Threshold: Tx Data Input Differential Peak-to-Peak Voltage Swing | ΔVdi pp los | 40 | 80 | 190 | mVpp |  |
| LOS Hysteresis |  | 0.5 |  | 4 | dB | 1 |
| Parameter (From Table 86A-2 of IEEE 802.3ba) | Test Point* | Min | Typ | Max | Units | Notes/Conditions |
| Single ended input voltage tolerance [2] | TP1a | -0.3 |  | 4 | V | Referred to TP1 signal common |
| AC common mode input voltage tolerance | TP1a | 15 |  |  | mV | RMS |
| Differential input return loss | TP1 | See IEEE 802.3ba 86A.4.1.1 |  |  | dB | 10 MHz to 11.1 GHz |
| Differential to common-mode input return loss | TP1 | 10 |  |  | dB | 10 MHz to 11.1 GHz |
| J2 Jitter tolerance | TP1a | 0.17 |  |  | UI | Defined in IEEE 802.3ba spec |
| J9 Jitter tolerance | TP1a | 0.29 |  |  | UI | Defined in IEEE 802.3ba spec |
| Data Dependent Pulse Width Shrinkage (DDPWS) tolerance | TP1a | 0.07 |  |  | UI |  |
| Eye Mask Coordinates: X1, X2 Y1, Y2 | TP1a | SPECIFICATION VALUES 0.11, 0.31 95, 350 |  |  |  |  |
|  |  |  |  |  | UI mV | Hit Ratio = 5x10-5 |

- See Figure 6 for Test Point definitions.

Y2

Differential amplitude (mV)

Y1

0

-Y1

-Y2

0	X1

X2	1-X2

Time (UI)

1-X1	1

Figure 3. Tx Electrical Eye Mask Coordinates at Hit ratio 5 x 10-5 hits per sample

Receiver Electrical Characteristics

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted. Typical values are for Tc = 40˚C, Vcc = 3.3 V

| Parameter | Test Point* | Min | Typ | Max | Units | Notes/Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Single ended output voltage tolerance | TP4 | -0.3 |  | 4 | V | Referred to signal common |
| AC common mode voltage (RMS) | TP4 |  |  | 7.5 | mV | RMS |
| Termination mismatch at 1MHz | TP4 |  |  | 5 | % |  |
| Differential output return loss | TP4 | See IEEE 802.3ba 86A.4.2.1 |  |  | dB | 10 MHz to 11.1 GHz |
| Common-mode output return loss | TP4 | See IEEE 802.3ba 86A.4.1.2 |  |  | dB | 10 MHz to 11.1 GHz |
| Output transition time 20% to 80% | TP4 | 28 |  |  | ps |  |
| J2 Jitter output | TP4 |  |  | 0.41 | UI |  |
| J9 Jitter output | TP4 |  |  | 0.62 | UI |  |
| Eye Mask coordinates: X1, X2 Y1, Y2 | TP4 | SPECIFICATION VALUES 0.29, 0.5 150, 425 |  |  |  |  |
|  |  |  |  |  | UI mV | Hit Ratio = 5x10-5 |

- See Figure 6 for Test Point definitions.

Y2

Differential Amplitude [mV]

Y1

0

-Y1

-Y2

0	X1	X2	1-X1	1.0

Normalized Time [UI]

Figure 4. Rx Electrical Eye Mask Coordinates (TP4) at Hit ratio 5 x 10-5 hits per sample

Transmitter  Optical  Characteristics

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted. Typical values are for Tc = 40˚C, Vcc = 3.3 V

| Parameter | Test Point* | Min | Typ | Max | Units | Notes/Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Center wavelength 1 | TP2 | 832 | 850 | 868 | nm |  |
| Center wavelength 2 | TP2 | 882 | 900 | 918 | nm |  |
| RMS spectral width | TP2 |  |  | 0.59 | nm | RMS Spectral Width is the standard deviation of the spectrum |
| Average launch power, 850nm lane | TP2 | -4.0 | 0 | 5 | dBm | Max: Set by the lower of Class 1M or Rx overload |
| Average launch power, 900nm lane | TP2 | -4.0 | 0 | 5 | dBm | Max: Set by the lower of Class 1M or Rx overload |
| Optical Modulation Amplitude (OMA) 850nm lane | TP2 | -1.0 | 0 | 5 | dBm |  |
| Optical Modulation Amplitude (OMA) 900nm lane | TP2 | -1.0 | 0 | 5 | dBm |  |
| Peak power, each lane | TP2 |  |  | 7 | dBm |  |
| Extinction ratio, each lane | TP2 | 3 | 4.5 |  | dB |  |
| RIN12OMA | TP2 |  |  | -130 | dB/Hz |  |
| Optical rise and fall time, 20-80% | TP2 |  | 21 |  | ps |  |
| Optical return loss tolerance | TP2 |  |  | 12 | dB |  |
| Encircled flux | TP2 | ≥ 86% at 19 µm, ≤ 30% at 4.5 µm |  |  |  | If measured into type A1a.2 50 µm fiber in accordance with EN 61280-1-4 |
| 20.625 Gb/s Eye Mask: X1, X2, X3, Y1, Y2, Y3 | TP2 | SPECIFICATION VALUES 0.23, 0.34, 0.43, 0.27, 0.35, 0.40 |  |  |  | Scaled IEEE 802.3ba 40GBASE-SR4 TX mask; Hit Ratio = 5x10-5 |
| Average launch power of OFF trans- mitter, each lane | TP2 |  |  | -30 | dBm |  |

- See Figure 6 for Test Point definitions.

1+Y3

Normalizd Amplitude [UA]

1

1-Y1

1-Y2

0.5

Y2 Y1

0

-Y3

0	X1	X2

X3   1-X3

X1 = 0.23

X2 = 0.34

X3 = 0.43

Y1 = 0.27

Y2 = 0.35

Y3 = 0.40

1-X2	1-X1	1.0

Normalized Time [UI]

Figure 5. Transmitter Optical Eye Mask definitions at Hit Ratio 5 x 10-5 hits per sample

Receiver  Optical Characteristics

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted. Typical values are for Tc = 40˚C, Vcc = 3.3 V

| Parameter | Test Point* | Min | Typ | Max | Units | Notes/Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Center wavelength 1 | TP3 |  | 850 |  | nm |  |
| Center wavelength 2 | TP3 |  | 900 |  | nm |  |
| Damage Threshold [1] | TP3 | +7.0 |  |  | dBm |  |
| Maximum Average power at receiver input, each lane (overload) | TP3 |  |  | +5.0 | dBm |  |
| Transceiver Reflectance | TP3 |  |  | -15 | dB |  |
| Peak power, each lane | TP3 |  |  | 7 | dBm |  |
| Optical Modulation Amplitude (OMA), 850nm lane (unstressed sensitivity) | TP3 |  |  | -7.1 | dBm | BER = 1e-12 |
| Optical Modulation Amplitude (OMA), 900nm lane (unstressed sensitivity) | TP3 |  |  | -7.7 | dBm | BER = 1e-12 |
| Stressed receiver sensitivity in OMA, 850nm lane | TP3 |  | -4.5 | -3.83 | dBm | BER = 1e-12 |
| Stressed receiver sensitivity in OMA, 900nm lane | TP3 |  | -4.5 | -3.86 | dBm | BER = 1e-12 |
| Conditions of stressed receiver sensitivity: [2] | TP3 |  |  |  |  |  |
| Vertical Eye Closure Penalty, 850nm | TP3 |  |  | 2.4 | dB |  |
| Vertical Eye Closure Penalty, 900nm | TP3 |  |  | 3.1 | dB |  |
| Stressed eye Jitter, each lane | TP3 |  | 0.13 |  | UI |  |
| LOS Assert | TP3 | -30 |  | -9.1 | dBm |  |
| LOS De-Assert (850nm) | TP3 |  |  | -8.6 | dBm |  |
| LOS De-Assert (900nm) | TP3 |  |  | -8.6 | dBm |  |
| LOS Hysteresis | TP3 | 0.5 |  |  | dB |  |

* See Figure 6 for Test Point definitions. Notes:

- The receiver shall be able to tolerate, without damage, continuous exposure to a modulated optical input signal having this power level on one lane. The receiver does not have to operate correctly at this input power.

ASIC/

SerDes

QSFP + TX

Electrical Connector

Electrical Connector

Vertical eye closure penalty and stressed eye jitter are test conditions for measuring stressed receiver sensitivity. They are not characteristics of the receiver.

Fiber

QSFP + RX

ASIC/

SerDes

TP 0	TP 1

TP 1a

TP 2

TP 3

TP 4a

TP 4

TP 5

Optical Patch Cord

TP0: Host ASIC transmitter output at ASIC package contact on the Host board

TP1: Host ASIC transmitter output across the Host Board at the input side of the Host QSFP+ electrical connector TP2: QSFP+ transmitter LC Duplex optical output at the end of a 2m to 5m patch cord

TP3: QSFP+ receiver LC Duplex optical input at the end of the fiber

TP4a: QSFP+ receiver electrical output at the input side of the Host QSFP+ electrical connector TP4: QSFP+ receiver electrical output at the output side of the Host QSFP+ electrical connector TP5: Host ASIC receiver input at ASIC package contact on the Host board

Figure 6. Test point definitions

Regulatory Compliance Table

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Contacts | JEDEC Human Body Model (HBM) (JESD22-A114-B) | Transceiver module withstands 1kV on high speed pins and 2kV on low speed pins |
| Electrostatic Discharge (ESD) to Optical connector | GR1089 | 10 discharges of 8 kV on the electrical face- plate with device inserted into a panel |
| Electrostatic Discharge (ESD) to Optical connector | Variation of EN 61000-4-2 | Air discharge of 15kV(min) to connector w/o damage |
| Electromagnetic Interference (EMI) | FCC Part 15 CENELEC EN55022 (CISPR 22A) VCCI Class 1 | Typically passes with 10 dB margin. Actual performance dependent on enclosure design |
| Immunity | Variation of EN 61000-4-3 | Typically minimum effect from a 10V/m field swept from 80 MHz to 1 GHz applied to the module without a chassis enclosure |
| Laser Eye Safety and Equipment Type Testing | IEC standard (EN 60825-1:2007 and EN 60825-2:2004+A1+A2:2010) CFR 21 Section 1040 | Pout: EN AEL & US FDA CDRH Class 1M |
| Component Recognition | Underwriters Laboratories and Canadian Stan- dards Association Joint Component Recogni- tion for Information Technology Equipment including Electrical Business Equipment | UL File Number: E173874, Vol. 1 |
| RoHS Compliance | BS EN 1122:2001 Mtd B by ICP for Cadmium, EPA Method 3051A by ICP for Lead and Mer- cury, EPA Method 3060A & 7196A by UV/Vis Spectrophotometry for Hexavalent Chromium. EPA Method 3540C/3550B by GC/MS for PPB and PBDE BS EN method by ICP and EPA methods by ICP, UV/Vis Spectrophotometry and GC/MS. | Less than 100 ppm of cadmium, Less than 1000 ppm of lead, mercury, hexava- lent chromium, polybrominated biphenyls, and polybrominated biphenyl esters. |

38   GND

37   TX1n

36   TX1p

35   GND

34   TX3n

33   TX3p

32   GND

31 LPMode

30  Vcc1

29  VccTx

28   IntL

27  ModPrsL

26   GND

25   RX4p

24   RX4n

23   GND

22   RX2p

21   RX2n

20   GND

GND TX2n TX2p GND TX4n TX4p GND

ModSelL ResetL VccRx SCL

SDA GND RX3p RX3n GND RX1p RX1n GND

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

Card Edge

QSFP+ Transceiver Pad Layout

Top Side Viewed from Top

Figure 7. QSFP+ Transceiver Pad Layout

Pins

| Pin | Logic | Symbol | Description | Plug Sequence | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1 | 1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 3 |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input | 3 |  |
| 4 |  | GND | Ground | 1 | 1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input | 3 |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input | 3 |  |
| 7 |  | GND | Ground | 1 | 1 |
| 8 | LVTTL-I | ModSelL | Module Select | 3 |  |
| 9 | LVTTL-I | ResetL | Module Reset | 3 |  |
| 10 |  | Vcc Rx | +3.3V Power supply receiver | 2 | 2 |
| 11 | LVCMOS-I/O | SCL | 2-wire serial interface clock | 3 |  |
| 12 | LVCMOS-I/O | SDA | 2-wire serial interface data | 3 |  |
| 13 |  | GND | Ground | 1 | 1 |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output | 3 |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output | 3 |  |
| 16 |  | GND | Ground | 1 | 1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output | 3 |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 3 |  |
| 19 |  | GND | Ground | 1 | 1 |
| 20 |  | GND | Ground | 1 | 1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 3 |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output | 3 |  |
| 23 |  | GND | Ground | 1 | 1 |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output | 3 |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output | 3 |  |
| 26 |  | GND | Ground | 1 | 1 |
| 27 | LVTTL-O | ModPrsL | Module Present | 3 |  |
| 28 | LVTTL-O | IntL | Interrupt | 3 |  |
| 29 |  | Vcc Tx | +3.3V Power supply transmitter | 2 | 2 |
| 30 |  | Vcc1 | +3.3V Power Supply | 2 | 2 |
| 31 | LVTTL-I | LPMode | Low Power Mode | 3 |  |
| 32 |  | GND | Ground | 1 | 1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input | 3 |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input | 3 |  |
| 35 |  | GND | Ground | 1 | 1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input | 3 |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input | 3 |  |
| 38 |  | GND | Ground | 1 | 1 |

Note

- GND is the symbol for signal supply (power) common for the QSFP+ module. All are common within the QSFP+ module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane

- Vcc Rx, Vcc1 and Vcc Tx are the receiver and transmitter power supplies and shall be applied concurrently.

1 H

0.1 F

22 F

Vcc_host =

3.3 Volt

1 H

0.1 F

22 F

0.1 F

1 H

0.1 F

22 F

Vcc Tx GND

Vcc Rx GND

Vcc1

GND

QSFP+ Module

22 F

Figure 9. Recommended Power Supply Filter

VCC33

50 

VCC25

DPx

Signal Path (Pos)

VCC33

50 

VCC25

DNx

Signal Path (Neg)

Figure 10. Transmitter Data Input Equivalent Circuit

VCC25

VCC25

VCC33

50 	50 

Doutp

Doutn

Signal Path (Neg)	Signal Path (Pos)

Figure 11. Receiver Data Output Equivalent Circuit

START	ReSTART	STOP

SCL

tSU,SDA

tLOW

tHD,SDA

tR

tHIGH

tHD,DAT

tF    tR

tS U,DAT

tBUF

tF

tSU,STO

tBUF

SDA In

tAA	tDH

SDA Out

Figure 12. TWS Interface Bus Timing

Package Outline, Host PCB Footprint and Bezel Design

All dimensions in mm

Figure 13. Mechanical Package Outline

X

BASIC

Y	37.00 MAX.

Ø1.05 ±0.05

BASIC

|  | Ø0.10 M | A | K S | L S |
| --- | --- | --- | --- | --- |

12 PLC

9.00

7.60

3.10

11.30 MIN.

10.60

6 PLC

3.40

38

1

17.90 REF.

16.80

7.20

22.15

19.00

20

19

1.10

C

Cross-hatched area denotes component and trace keep-out (except chassis ground)

This area denotes component keep-out (traces allowed)

All dimensions in mm

Figure 14. QSFP+ Host Board Mechanical Footprint

7.60

3.10

Notes:

  - Datum X & Y are established by the customer’s fiducial

  - Datum A is the top surface of the host board

  - Location of the edge of PCB is application specific

  - Finished hole size

|  | Ø0.05 | A | X | K S |
| --- | --- | --- | --- | --- |

|  | 0.05 | A | L-K | C |
| --- | --- | --- | --- | --- |

|  | Ø0.05 | A | X | Y |
| --- | --- | --- | --- | --- |

|  | 0.05 | A | C | L-K |
| --- | --- | --- | --- | --- |

4X10G CDR

40GE XLPPI Electrical Interface

VCC33

50 

40GE XLPPI Electrical Interface

4X10G CDR

VCC33

50 

4X10G CDR

40GE XLPPI Electrical Interface

2:4 DeMux

4:2 Mux

RX  λ1

20G Laser Driver + TI

A

20G

20G

20G VCSEL, PIN and Bidirectional optics

Gearbox IC

TX λ2

TX λ1

LC Duplex Optical Interface

1 H

0.1 F

22 F

Vcc_host =

3.3 Volt

1 H

0.1 F

22 F

0.1 F

1 H

0.1 F

22 F

Vcc Tx GND

Vcc Rx GND

Vcc1

GND

QSFP+ Module

Card Edge

LC Duplex Optical Interface

TX λ1

GND TX2n TX2p GND TX4n TX4p GND

ModSelL ResetL VccRx SCL

SDA GND RX3p RX3n GND RX1p RX1n GND

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

TX λ1

38   GND

37   TX1n

36   TX1p

35   GND

34   TX3n

33   TX3p

32   GND

31 LPMode

30  Vcc1

29  VccTx

28   IntL

27  ModPrsL

26   GND

25   RX4p

24   RX4n

23   GND

22   RX2p

21   RX2n

20   GND

Optical Patch Cord

TX λ1

LC Duplex Optical Interface

TX λ2

Fiber

QSFP + RX

ASIC/

SerDes

Electrical Connector

Electrical Connector

TX λ2

LC Duplex Optical Interface

ASIC/

SerDes

QSFP + TX

X1 = 0.23

X2 = 0.34

X3 = 0.43

Y1 = 0.27

Y2 = 0.35

Y3 = 0.40

Normalizd Amplitude [UA]

Differential Amplitude [mV]

Differential amplitude (mV)

Optical Connector/Port (Optical Interface)

Host Edge Card Connector

Module Card Edge (Host Interface)

LC Duplex Optical Interface

TX λ2

TX λ1

Gearbox IC

20G VCSEL, PIN and Bidirectional optics

20G

Ø1.55 ±0.05

2.51

1

5.18

1

3

Datum Axis C

15.02 MAX.

19.20 MAX.

2

22.15

3

C

0.35 ±0.03

Ø1.55 ±0.05

1.80 ±0.03

7.40

7.00

16.80

0.80

0.20

0.20

20

19

Notes:

- Centerline of Pad

- Surface traces permitted within this length

- Indicated holes are  optional

All dimensions in mm

Control Interface

The control interface combines dedicated signal lines for ModSelL, LP Mode, ResetL, ModPrsL, IntL with two-wire serial (TWS), interface clock (SCL) and data (SDA), signals to provide users rich functionality over an efficient and easily used interface. The TWS interface is implemented  as a slave device and compatible with industry standard two-wire serial protocol. It is scaled for 3.3 volt LVTTL. Outputs are high-z in the high state to support busing of these signals. Signal and timing characteristics are further defined in the Control I/O Characteristics  section.

ModSelL

The ModSelL is an input signal. When held low by the host, the module responds to 2-wire serial communica- tion commands. The ModSelL allows the use of multiple QSFP+ modules on a single 2-wire interface bus. When the ModSelL is “High”, the module will not respond to or acknowledge any 2-wire interface communication from the host. ModSelL signal input node is biased to the“High” state in the module. In order to avoid conflicts, the host system shall not attempt 2-wire interface communica- tions within the ModSelL de-assert time after any QSFP+ modules are deselected. Similarly, the host must wait at least for the period of the ModSelL assert time before communicating with the newly selected module. The as- sertion and de-asserting periods of different modules may overlap as long as the above timing requirements are met.

ResetL

The ResetL signal is pulled to Vcc in the QSFP+ module. A low level on the ResetL signal for longer than the minimum pulse length (t_Reset_init) initiates a complete module reset, returning all user module settings to their default state. Module Reset Assert Time (t_init) starts on the rising edge after the low level on the ResetL pin is released. Dur- ing the execution of a reset (t_init) the host shall disregard all status bits until the module indicates a completion of the reset interrupt. The module indicates this by posting an IntL signal with the Data_Not_Ready bit negated.

LPMode

Low power mode. When held high by host, the module is held at low power mode with power consumption meet- ing class 1 power level (1.5W). Under low power mode, high speed operation is not functional and all high speed indicators are not reliable while the TWS management in- terface is functional. When held low by host, the module operates in the normal mode.

ModPrsL

ModPrsL is pulled up to Vcc_Host on the host board and grounded in the module. The ModPrsL is asserted “Low” when inserted and de-asserted “High” when the module   is physically absent from the host  connector.

IntL

IntL is an output signal. When “Low”, it indicates a possible module operational fault or a status critical to the host sys- tem. The host identifies the source of the interrupt using the 2-wire serial interface. The IntL signal is an open collec- tor output and must be pulled to host supply voltage on the host board. A corresponding soft status IntL signal is also available in the transceiver memory page 0 address 2 bit 1.

Soft Status and Control

A number of soft status signals and controls are available in the QSFP-40G-JH transceiver memory and accessible through the TWS interface. Soft status signals include receiv- er LOS, transmitter LOS, and a transmitter fault. Some soft controls include transmitter disable (Tx_Dis), receiver output disable (Rx_Dis), transmitter squelch disable (Tx_SqDis), re- ceiver squelch disable (Rx_SqDis) and masking of status sig- nal in triggering IntL. All soft control entries are volatile.

Receiver LOS

The Receiver LOS status signal is on page 0 address 3 bits 0-3. Reeiver LOS is based on received optical power modula- tion amplitude (OMA). The status register is latched and it is cleared on read. The first 20G lane RX is mapped to the first 2 RX lanes as defined in QSFP, and the second 20G RX lane is mapped to the third and fourth RX lanes as defined in QSFP MSA.

Transmitter LOS

The transmitter LOS status signal is on page 0 address 3 bits 4-7. Transmitter LOS is based on input differential voltage. The status register is latched and it is cleared on read.

Transmitter Fault

Transmitter fault status is reported in page0 register 4. Trans- mitter fault will be asserted when the laser cannot be con- trolled anymore, or it could cause harm to eyes as defined by the eyesafety regulation. The status  is latched  and cleared on read.

Transmitter Disable

The transmitter Disable control is on page 0 address register

86. Because 2 10G input lanes are muxed into one 20G TX optical lane, either one of these lanes getting disabled will disable  the 20G optical signal.

Receiver  Disable

The received disable control is on page 3 address 241 bits 4-7. Asserting any bit would cause the corresponding lane’s RX output to be disabled.

Transmitter Squelch Disable

The Transmitter squelch disable control is on page 3 address

240 bits 0-3. QSFP-40G-JH transceivers have transmitter squelch enabled as default.

Receiver Squelch Disable

The receiver squelch disable control is on page 3 address  240 bits 4-7. QSFP-40G-JH transceivers have receiver output squelch enabled as default.

I/O Timing for Control and Status Functions

The following characteristics are defined over the Recommended Operating Conditions unless otherwise noted.

| Parameter | Symbol | Min | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- |
| Initialization Time | t_init |  | 2000 | ms | Time from power on, hot plug or rising edge of Reset until the module is fully functional. This time does not apply to non Power level 0 modules in the Low Power state |
| LPMode Assert Time | ton_LPMode |  | 100 | µs | Time from assertion of LPMode until the module power con- sumption enters power level 1 |
| Interrupt Assert Time | ton_IntL |  | 200 | ms | Time from occurrence of condition triggering IntL until Vout:IntL=Vol |
| Interrupt De-assert Time | toff_IntL |  | 500 | µs | Time from clear on read operation of associated flag until Vout:IntL=Voh. This includes deassert times for RX LOS, TX Fault and other flag bits |
| Reset Init Assert Time | t_reset_init |  | 2 | µs | A Reset is generated by a low level longer than the minimum reset pulse time present on the ResetL pin |
| Reset Assert Time | t_reset |  | 2000 | ms | Time from rising edge on the ResetL pin until the module is fully functional |
| Serial Bus Hardware Ready Time | t_serial |  | 2000 | ms | Time from power on until module responds to data transmission over the 2-wire serial bus |
| Monitor Data Ready Time | t_data |  | 2000 | ms | Time from power on to data not ready, bit 0 of Byte 2, deasserted and IntL asserted |
| RX LOS Assert Time | ton_los |  | 100 | ms | Time from RX LOS state to RX LOS bit set and IntL asserted |
| TX Fault Assert Time | ton_Txfault |  | 200 | ms | Time from TX Fault state to TX fault bit set and IntL asserted |
| Flag Assert Time | ton_Flag |  | 200 | ms | Time from occurrence of condition triggering flag to associated flag bit set and IntL asserted. |
| Mask Assert Time | ton_Mask |  | 100 | ms | Time from mask bit set until associated IntL assertion is inhibited |
| Mask Deassert Time | toff_Mask |  | 100 | ms | Time from mask bit cleared until associated IntL operation resumes |
| Power Set Assert Time | ton_Pdown |  | 100 | ms | Time from P_Down bit set until module power consumption enters power level 1 |
| Power Set Deassert Time | toff_Pdown |  | 300 | ms | Time from P_Down bit cleared until the module is fully func- tional |
| RX Squelch Assert Time | ton_Rxsq |  | 80 | µs | Time from loss of RX input signal until the squelched output condition is reached |
| RX Squelch Deas- sert Time | toff_Rxsq |  | 1 | ms | Time from resumption of RX input signals until normal RX output condition is reached |
| TX Squelch Assert Time | ton_Txsq |  | 400 | ms | Time from loss of TX input signal until the squelched output condition is reached |
| TX Squelch Deas- sert Time | toff_Txsq |  | 400 | ms | Time from resumption of TX input signals until nominal TX out- put condition is reached |
| TX Disable Assert Time | ton_txdis |  | 100 | ms | Time from TX Disable bit set until optical output falls below 10% of nominal |
| TX Disable Deassert Time | toff_txdis |  | 400 | ms | Time from TX Disable bit cleared until optical output rises above 90% of nominal |
| RX Output Disable Assert Time | ton_rxdis |  | 100 | ms | Time from RX Output Disable bit set until RX output falls below 10% of nominal |
| RX Output Disable Deassert Time | toff_rxdis |  | 100 | ms | Time from RX Output Disable bit cleared until RX output rises above 90% of nominal |
| Squelch Disable Assert Time | ton_sqdis |  | 100 | ms | This applies to RX and TX Squelch and is the time from bit set until squelch functionality is disabled |
| Squelch Disable Deassert Time | toff_sqdis |  | 100 | ms | This applies to RX and TX Squelch and is the time from bit cleared until squelch functionality is enabled |

Memory Map

The memory is structured as a single address, multiple page approach. The address is given as A0xh. The structure of the memory is shown in Figure 18. The memory space is arranged into a lower, single page, address space of 128 bytes and multiple upper address space pages. This structure permits timely access to addresses in the lower page, e.g. Interrupt Flags and Monitors. Less time critical entries, e.g. serial ID information are available with the Page Select function. For a more detailed description of the QSFP+ memory map see the QSFP+ SFF-8436 Specification or the  QSFP+ Memory Map document. DMI is not supported in this product.

2-wire serial address, 1010000x (A0h)"

128

191

223

255

Page 00

Base ID Fields

Extended ID

Vendor Speci c ID

(64 Bytes)

(32 Bytes)

(32 Bytes)

128

128

129

131

Page 01 (Optional)

CC_APPS

AST Table Length (TL)

Application Code Entry 0

Application Code Entry 1

other entries

Application Code Entry TL

(1 Byte)

(1 Byte)

(2 Bytes)

128

255

Page 02 (Optional)

User EEPROM Data

(128 Bytes)

128

175

223

225

Page 03

Module Threshold

Channel Threshold

Reserved

Vendor Speci c Channel Controls

Channel Monitor Masks

Reserved

(48 Bytes)

(48 Bytes)

(2 Bytes)

(2 Bytes)	(16 Bytes)

133	241

255

Figure 17. Two-Wire Serial Address A0h Page Structure

(2 Bytes)

253

255

(12 Bytes)

(2 Bytes)

Figure 18. Transceiver Lane Ordering Limitations

Lane Ordering

Care must be taken to reassemble XLPPI information at the host receiver due to uncertainty of phase alignment of Lane A and B between transmit and receive multiplexer stages. The transceiver will not assure XLPPI lane assignment end-to- end in order to keep complexity and power dissipation in the QSFP+ footprint to a minimum. Once end-to-end com- munication has been established and lane ordering established, lane assignment may change again each time the link     is re-established, such as when TX LOS, RX LOS, or when module power cycling occurs.