JH-40G-QSFP+-ZR4-80

40G QSFP+ ZR4 Optical Transceiver

PRODUCT FEATURES

- Support line rates for 41.25 Gb/s;

- Lane bit rate 10.3125 Gb/s 40GE

- Up to 80km transmission on single mode fiber

- LAN WDM EML laser and APD

- Support Multi-Pin function with IntL/RxLOSL and LPMode/TxDIS

- High speed I/O electrical interface (CAUI-4)

- I2C interface with integrated Digital Diagnostic monitoring

- QSFP+ MSA package with duplex LC connector

- Single +3.3V power supply

- Power dissipation

- Commercial：< 4.5W

- Industrial：< 5.5W

- Complies with EU Directive 2015/863/EU

APPLICATIONS

- Data centers Switches and Routers

- Metro networks

- Switches and Routers

- 40G BASE-ZR4

DESCRIPTIONS

JH-40G-QSFP+-ZR4-80 is designed for 80km optical communication applications. Optical transceiver integrates the transmit and receive path onto one module. On the transmit side, four lanes of serial data streams are recovered, retimed and passed on to four laser drivers, which control four electric-absorption modulated lasers (Lan-WDM) with 1296, 1300, 1305, and 1309 nm center wavelengths. The optical signals are then multiplexed into a single-mode fiber through an industry-standard LC connector. On the receive side, four lanes of optical data streams are optically De multiplexed by an integrated optical De multiplexer. And passed on to an output driver, the receiver adopts APD. This module features a hot-pluggable electrical interface, low power consumption, and 2-wire serial interface.

Module Block Diagram

- Ordering Information

| Part No. | Data Rate(optical) | Laser | Fiber Type | Distance | Optical Interface | Temp | DDMI | Latch Color |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JH-40G-QSFP+-ZR4-80 | 41.25Gbps | EML | SMF | 80km | LC | 0~70℃ | Yes | White |
| JH-40G-QSFP+-ZR4-80-I | 41.25Gbps | EML | SMF | 80km | LC | -40~85℃ | Yes | White |

- Absolute Maximum Ratings

It has to be noted that the operation in excess of any individual absolute maximum ratings might cause permanent damage to this module.

| Parameter | Symbol | Min | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -40 | 85 | degC |  |
| Operating Case Temperature | TOP | -40 | 85 | degC |  |
| Power Supply Voltage | VCC | -0.5 | 3.6 | V |  |
| Damage Threshold, each Lane | THd | -2.5 |  | dBm |  |
| Relative Humidity (non-condensation) | RH | 0 to 85% non-condensing |  | % |  |

The following characteristics are defined over the Recommended Operating Environment unless otherwise specified.

- Recommended Operating Conditions

| Parameter | Symbol | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | Top | 0 |  | +70 | °C | 1 |
|  |  | -40 |  | +85 |  | 2 |
| Power Supply Voltage | VCC | 3.14 | 3.3 | 3.47 | V |  |
| Maximum Power Dissipation | PD |  |  | 4.5 | W | 1 |
| Maximum Power Dissipation | PD |  |  | 5.5 | W | 2 |
| Lane Bit Rate |  |  | 10.3125 |  | Gb/s |  |
| Transmission Distance | TD |  |  | 80 | km |  |
| Coupled fiber | Single mode fiber |  |  |  |  | 3 |

Notes:

  - The product name is JH-40G-QSFP+-ZR4-80.

  - The product name is JH-40G-QSFP+-ZR4-80-I

  - Optical fiber use 9/125um SMF。

- Electrical Characteristics

The following electrical characteristics are defined over the Recommended Operating Environment unless otherwise specified.

| Parameter | Test Point | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter (Module Input) |  |  |  |  |  |  |
| Data Rate, each lane |  |  | 10.3125 |  | Gbps |  |
| Differential Voltage pk-pk | Vpp |  |  | 900 | Mv | 1 |
| Common Mode Voltage | Vcm | -350 |  | 2850 | Mv |  |
| Transition time | Trise/Tfall | 10 |  |  | ps | 2 |
| Receiver (Module Output) |  |  |  |  |  |  |
| Data Rate, each lane |  |  |  |  | 10.3125 |  |
| Common Mode Noise, RMS | Vrms |  |  | 17.5 | Mv |  |
| Differential output voltage swing | Vout, pp |  |  | 900 | Mv |  |
| Eye width | EW15 | 0.57 |  |  | UI |  |
| Eye height | EH15 | 228 |  |  | mV |  |
| Differential Termination Resistance Mismatch |  |  |  | 10 | % | 1 |
| Transition time | Trise/Tfall | 12 |  |  | ps |  |
| Common Mode Noise, RMS | Vrms |  |  | 17.5 | Mv |  |

Notes:

  - At 1 MHz.

  - 20%~80%.

- Optical and Characteristics

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling Speed per Lane |  |  | 10.3125 |  | Gbps |  |
| Center Wavelength Lane 0 | λ0 | 1294.53 | 1295.56 | 1296.59 | nm |  |
| Center Wavelength Lane 1 | λ1 | 1299.02 | 1300.05 | 1301.09 | nm |  |
| Center Wavelength Lane 2 | λ2 | 1303.54 | 1304.58 | 1305.63 | nm |  |
| Center Wavelength Lane 3 | λ3 | 1308.09 | 1309.14 | 1310.19 | nm |  |
| Total Launch Power,40GE | PT |  |  | 12.5 | dBm | 1 |
| Average Launch Power per Lane, | Pavg | 1 |  | 6.5 | dBm | 1 |
| OMA, each Lane | POMA | 2 |  | 6.5 | dBm | 1 |
| Difference in launch power between any two lanes(Average and OMA) between any Two Lanes (OMA) | Ptx,diff |  |  | 3 | Db |  |
| Average Output Power (Laser Turn off) | Poff |  |  | -30 | dBm |  |
| Side Mode Suppression Ratio | SMSR | 30 |  |  | Db |  |
| Extinction Ratio, 100GE | ER | 8.2 |  |  | Db |  |
| RIN20OMA | RIN |  |  | -130 | Db/Hz |  |
| Optical Return Loss Tolerance | TOL |  |  | 20 | Db |  |
| Transmitter Reflectance | RT |  |  | -12 | Db |  |
| Optical Eye Mask | {0.25,0.4, 0.45, 0.25, 0.28, 0.4} |  |  |  | % |  |
| Receiver |  |  |  |  |  |  |
| Signaling rate, each lane |  |  | 10.3125 |  | Gbps |  |
| Center Wavelength Lane 0 | λ0 | 1294.53 | 1295.56 | 1296.59 | nm |  |
| Center Wavelength Lane 1 | λ1 | 1299.02 | 1300.05 | 1301.09 | nm |  |
| Center Wavelength Lane 2 | λ2 | 1303.54 | 1304.58 | 1305.63 | nm |  |
| Center Wavelength Lane 3 | λ3 | 1308.09 | 1309.14 | 1310.19 | nm |  |
| Damage threshold , each lane | Pd amage | -2.5 |  |  | dBm |  |
| Average power at receiver input , each lane | R | -24 |  | -6 | dBm | 2 |
| Los Assert | LosA | -35 |  |  | dBm |  |
| Los De-assert | LosDA |  |  | -25 | dBm |  |
| Los Hysteresis | LosH | 0.5 |  |  | dB |  |

Notes:

- The optical power is launched into SMF。

- Measured with a PRBS 231-1 test pattern @10.3125 Gb/s, PRBS 2^31-1.

- Digital Diagnostics

| Parameter | Unit | Specification |
| --- | --- | --- |
| Temperature Monitor absolute error | ℃ | ± 3.0 |
| Supply Voltage Monitor absolute error | % | ± 5 |
| I_bias Monitor absolute error | % | ± 10 |
| Received Power (Rx) Monitor absolute error | dB | ± 3.0 |
| Transmit Power (Tx) Monitor absolute error | dB | ± 3.0 |

- Pin Diagram

- Pin Definitions

| PIN | Logic | Symbol | Description | Plug Seq. | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1 | 1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 3 |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output | 3 |  |
| 4 |  | GND | Ground | 1 | 1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input | 3 |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output | 3 |  |
| 7 |  | GND | Ground | 1 | 1 |
| 8 | LVTLL-I | ModSelL | Module Select | 3 |  |
| 9 | LVTLL-I | ResetL | Module Reset | 3 |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver | 2 | 2 |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock | 3 |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data | 3 |  |
| 13 |  | GND | Ground | 1 |  |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output | 3 |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output | 3 |  |
| 16 |  | GND | Ground | 1 | 1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output | 3 |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 3 |  |
| 19 |  | GND | Ground | 1 | 1 |
| 20 |  | GND | Ground | 1 | 1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 3 |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output | 3 |  |
| 23 |  | GND | Ground | 1 | 1 |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output | 3 | 1 |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output | 3 |  |
| 26 |  | GND | Ground | 1 | 1 |
| 27 | LVTTL-O | ModPrsL | Module Present | 3 |  |
| 28 | LVTTL-O | IntL/Rx_LOS | Interrupt/Rx_LOS | 3 |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter | 2 | 2 |
| 30 |  | Vcc1 | +3.3 V Power Supply | 2 | 2 |
| 31 | LVTTL-I | LPMode/TxDIS | Low Power Mode/Tx_Disable | 3 |  |
| 32 |  | GND | Ground | 1 | 1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input | 3 |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Output | 3 |  |
| 35 |  | GND | Ground | 1 | 1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input | 3 |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output | 3 |  |
| 38 |  | GND | Ground | 1 | 1 |

Notes:

- GND is the symbol for signal and supply (power) common for the QSFP28 module. All are common within the QSFP28 module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane.

- Vcc Rx, Vcc1 and Vcc Tx are the receiver and transmitter power supplies and shall be applied concurrently. Requirements defined for the host side of the Host Edge Card Connector are listed in MSA. The connector pins are each rated for a maximum current of 1000 Ma

- Recommended Interface Circuit

- Mechanical Diagram

- @

- Revision History

| Version No. | Date | Description |
| --- | --- | --- |
| 1.0 | Dec 24, 2021 | Preliminary datasheet |
| 2.0 | October 11,2023 | Product upgrades |
| 2.1 | July 20, 2024 | Format change |

Wuhan Jiuhong Optics Technology Co., Ltd.

Address: Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,

Donghu High-tech Zone, Wuhan City

Wuhan Jiuhong reserves the right to make changes to the products or information contained herein without notice.

No liability is assumed as a result of their use or application.

No rights under any patent accompany the sale of any such products or information.

Published by Wuhan Jiuhong Optics Technology Co., Ltd. Copyright © Wuhan Jiuhong Optics Technology Co., Ltd. All Rights Reserved.