Product Specification

100G QSFP28 10km PLR4 Transceivers

The  JH-100G-QSFP28-PLR4-10 QSFP28 optical transceiver is intended for up to 10km reach service with four-lane 25.78125G data rate. It is based on 3.3V DC power supply and operates in the commercial temperature range. It is compliant with QSFP MSA 、SFF-8436 、SFF-8636 and PSM4 MSA. Digital diagnostic functions are available via  I2C interface，and the control functions can be  achieved by LVTTL interfaces on the host，mainly including Module Select(ModSelL)、Module Reset(ResetL)、 Low Power Mode(LPMode). The transceiver incorporates a four-laser array which is usually DFB 、four-PIN diode array 、a high performance CDR integrated four drivers and TIAs IC separately. The differential AC coupled Tx and Rx data interfaces are CML compatible.

Applications

- 100G BASE Ethernet

- Infiniband EDR interconnects

- Enterprise networking

Features

- MPO12 optical interface

- Maximum link length up to 10km

- Up to 25.78125Gb/s data links per lane

- +3.3 V power supply

- QSFP MSA compliant package

- Hot Pluggable

- High performance singal mode DML transmitter

- High sensitivity PIN/TIA optical receiver

- Single Mode operation

- BER < 5E-5@-10.5dBm（OMA）

- Built-in CDR

- Case Operating temperature : 0 to 70ºC

- Data and Control Interfaces

- Tx Data CML/AC Coupled

- Rx Data CML/AC Coupled

- ModSelL LVTTL

- ResetL LVTTL

- ModPrsL LVTTL

- LPMode LVTTL

- 2-wire I2C communication bus

- RoHS 6 compliance

# Ordering Code

| Ordering Information |  |
| --- | --- |
| Part Number | Case Operating Temperature |
| JH-100G-QSFP28-PLR4-10 | 100G QSFP28 10km 0 to 70 ºC |

# Absolute maximum parameters

| Absolute Maximum Ratings (EXCEEDING THESE RATINGS MAY CAUSE IRREVERSIBLE DAMAGE TO THE DEVICE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Max | Units | Notes |
| Storage Temperature | Tstg | -40 | +95 | ºC | Exceeding the absolute maximum ratings may cause irreversible damage to the device. The device is not intended to be operated under the condition of simultaneous absolute maximum rat- ings, which may cause irreversible damage to the device. |
| Case Operating Temperature (Commercial ) | TO | 0 | +70 | ºC |  |
| Relative Humidity - Storage | RHS | 0 | 95 | % |  |
| Relative Humidity - Operating | RHO | 0 | 85 | % |  |
| Supply Voltage | VCC | -0.3 | 3.6 | V |  |

# Operating conditions

| Recommended Operating Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| Case Operating Temperature | Tcase | 0 | - | +70 | ºC |  |
| DC Supply Voltage | VCC | 3.135 | - | 3.465 | V |  |
| Module Supply Current | IIN | - | - | 1060 | mA |  |

# Electrical Characteristics

| Transmitter Electrical Characteristics |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Mi | Typ | Max | Units | Notes |
| Differential Data input Swing | VIN | 180 | - | 900 | mV |  |
| Tx Differential Input Impendence | Zin | 90 | 100 | 110 | Ω |  |
| Tx Differential Output Impendence | Zout | 45 | 50 | 55 | Ω |  |
| ResetL Disable Voltage | Vr | 2.0 | - | Vcc+0.3 | V |  |
| ResetL Enable Voltage | VrEN | 0 | - | 0.8 | V |  |
| ModSelL Disable Voltage | Vm | 2.0 | - | Vcc+0.3 | V |  |
| ModSelL Enable Voltage | VmEN | 0 | - | 0.8 | V |  |

| Receiver Electrical Characteristics |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| Differential Data Output Swing | VOUT | 180 | - | 900 | mV |  |
| Rx Differential Output Impendence | ZOUT | 90 | 100 | 110 | Ω |  |
| IntL Assert Voltage | VInt | VCC-0.5 | - | VCC+0.3 | V |  |
| IntL De-assert Voltage | VDInt | 0 | - | +0.4 | V |  |

# Optical  Specification

| Transmitter Optical Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| Signal Rate¦Each Lane |  |  | 25.78125±100ppm |  | Gbps |  |
| Lane Wavelength | L0 | 1295 | 1310 | 1325 | nm |  |
|  | L1 | 1295 | 1310 | 1325 | nm |  |
|  | L2 | 1295 | 1310 | 1325 | nm |  |
|  | L3 | 1295 | 1310 | 1325 | nm |  |
| Side Mode Suppression Ratio | SMSR | 30 |  |  | dB |  |
| Total Average Launch Power | Ptol |  |  | 8.5 | dBm |  |
| Average Launch Power¦Each Lane | Pavg | -6 |  | 2.5 | dBm |  |
| Optical Modulation Amplitude¦Each Lane | OMA |  |  | 2.5 | dBm |  |
| Transmitter and dispersion penalty¦Each Lane | TDP |  |  | 2.2 | dB |  |
| Eye Mask coordinates: X1, X2, X3, Y1, Y2, Y3 |  | {0.31, 0.4, 0.45, 0.34, 0.38, 0.4} |  |  |  | 1 |
| Average launch power of OFF transmitter¦Each Lane |  |  |  | -30 | dBm |  |
| Extinction Ratio | ER | 3.5 |  |  | dB |  |
| Spectral Width¦20dB |  |  |  | 1 | nm |  |
| Transmitter Reflectance |  |  |  | -12 | dB |  |
| Optical return loss tolerance |  |  |  | 20 | dB |  |

Note:

- Hit ratio of 5e-5, per IEEE.

| Receiver Optical Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| Signal Speed Per Lane |  |  | 25.78125±100ppm |  | Gbps |  |
| Lane Wavelength | L0 | 1295 | 1310 | 1325 | nm |  |
|  | L1 | 1295 | 1310 | 1325 | nm |  |
|  | L2 | 1295 | 1310 | 1325 | nm |  |
|  | L3 | 1295 | 1310 | 1325 | nm |  |
| Damage threshold¦Each Lane | THd |  |  | 3.0 | dBm | 1 |
| Average Receive Power¦Each Lane |  | -10.6 |  | 2.5 | dBm | 2 |
| Sensitivity OMA¦Each Lane | Sen |  |  | -10.5 | dBm | 2 |
| Receiver reflectance |  |  |  | -26 | dB |  |
| LOS Assert | LOSA | -24 |  |  |  |  |
| LOS Deassert | LOSD |  |  | -11.6 |  |  |
| LOS Hysteresis | LOSH | 0.5 |  | 6 |  |  |

Note:

1. The receiver shall be able to tolerate, without damage, continuous exposure to a modulated optical input signal having this power level on one lane. The receiver does not have to operate correctly at this input power.

2. Measured with conformance test signal at receiver input for BER = 5e-5 BER.

PIN ASSIGNMENT

PIN DESCRIPTION

| PIN | Logic | Symbol | Name/Description | Note |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground |  |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 |  | GND | Ground |  |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 |  | GND | Ground |  |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver |  |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data |  |
| 13 |  | GND | Ground |  |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground |  |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground |  |
| 20 |  | GND | Ground |  |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground |  |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground |  |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter |  |
| 30 |  | Vcc1 | +3.3 V Power Supply |  |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground |  |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Output |  |
| 35 |  | GND | Ground |  |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output |  |
| 38 |  | GND | Ground |  |

OUTLINE DIMENSIONS

Notes:

1、Tolerance: +/-0.1mm.

2、Others  according to SFF-8661  or customer spec .

3、Optical  port  according  to  fiber  connector spec.

Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |