JH-SW100-SR4C

100G QSFP28 SWDM4 Optical Transceiver

Product Features

Compliant with QSFP28 MSA Compliant with SWDM MSA  Compliant with SFF-8636

Compliant with IEEE 802.3bm CAUI-4

Hot-pluggable QSFP28 form factor

4×25Gb/s 850nm VCSEL-based transmitter Supports 103.1Gbps aggregate bit rate

Power dissipation<3.5W

Maximum link length of 150m on OM5 multimode Fiber Case temperature range of 0℃to 70℃

Duplex LC receptacles

CAUI-4 electrical interface RoHS compliant

Operating case temperature:Standard:0 to  +70℃

Applications

100G Ethernet over Duplex MMF

Standards

Compliant to IEEE 802.3bm    100GBASE-SR4 Ethernet links Compliant to SFF-8436

General Description

The JH 100G QSFP28 SWDM4 transceiver modules are designed for use in 100G Ethernet links over duplex multimode fiber.Four channels/lanes in the 850-940nm region @25.78Gbps to transport the Ethernet signal.   Digital diagnostics functions are available via an I2C interface,as specified by the QSFP28 MSA.

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Units |
| --- | --- | --- | --- | --- |
| Storage Temperature Range | Ts | -40 | +85 | ℃ |
| Supply Voltage | Vcc | -0.5 | 3.6 | V |
| Relative Humidity | RH | 15% | 85% |  |

Operating Conditions

| Parameter | Symbol | Min | Max | Units |
| --- | --- | --- | --- | --- |
| Case Temp-Operating | Tcase | 0 | 70 | ℃ |
| Supply Voltage | Vcc | 3.14 | 3.46 | V |
| Power Consumption | P |  | 3.5 | W |
| Link Distance on OM3 Fiber |  |  | 75 | M |
| Link Distance on OM4 Fiber |  |  | 100 | M |
| Link Distance on OM5 Fiber |  |  | 150 | M |

Optical Characteristics

| Transmitter Parameters | Lane | Min | Typical | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Signaling rate,each lane |  | 25.78125±100ppm |  |  | Gb/s |  |
| Lane Wavelength Range | Lane0 Lane1 Lane2 Lane3 | 844 874 904 934 |  | 858 888 918 948 | nm |  |
| Modulation Format |  | NRZ |  |  |  |  |
| Difference in launch power between any two lanes |  |  |  | 4.5 | dBm |  |
| RMS spectral width |  |  |  | 0.59 | nm | 1 |
| Optical Modulation Amplitude (OMA), each lane |  | -5.5 |  | 3 | dBm | 2 |
| Average Launch Power per Lane @TX Off State |  |  |  | -30 | dBm |  |
| Launch Power in OMA minus TDEC | Lane0 Lane1 Lane2 Lane3 | -7 -7 -7.4 -7.7 |  |  | dBm |  |
| Transmitter and Dispersion Eye Closure | Lane0 Lane1 Lane2 Lane3 |  |  | 4 4 4.4 4.8 | dB | 3 |
| Extinction Ratio |  | 2 |  |  | dB |  |
| Optical Return Loss Tolerance |  |  |  | 12 | dB |  |

| Encircled Flux |  | ≥86% at 19 um ≤30% at 4.5 um | 4 |
| --- | --- | --- | --- |
| Transmitter eye mask definition {X1, X2, X3, Y1, Y2, Y3}Hit ratio 1.5x10-3 hits per sample |  | {0.3,0.38,0.45,0.35,0.41,0.5} |  |
| Notes: 1. RMS spectral width is the standard deviation of the spectrum. 2. The normative lowest value of OMA for a compliant transmitter is 'Launch power in OMA minus TDEC, each lane (min)' plus the actual value of 'TDEC', but with a value of at least 'OMA, each lane (min)'. 3. TDEC is calculated from the measured TDECm using the methods in 3.6. TDECm is measured following the method in IEEE 802.3 clause 95.8.5 using a 12.6 GHz bandwidth reference receiver for all lanes. 4. If measured into type A1a.2 or type A1a.350 um fiber in accordance with IEC61280-1-4. |  |  |  |

| Receiver Parameter | Lane | Min | Typical | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Signaling rate: each lane |  | 25.78125±100ppm |  |  | Gb/s |  |
| Lane Wavelength Range | Lane0 Lane1 Lane2 Lane3 | 844 874 904 934 |  | 858 888 918 948 | nm |  |
| Modulation Format |  | NRZ |  |  |  |  |
| Damage Threshold |  | 4.4 |  |  | dBm |  |
| Average Receive Power, each lane | Lane0 Lane1 Lane2 Lane3 | -9.5 -9.4 -9.4 -9.4 |  | 3.4 | dBm |  |
| Receiver Power, each lane (OMA) |  |  |  | 3 | dBm |  |
| Receiver Reflectance |  |  |  | -12 | dB |  |
| Unstressed Receiver Sensitivity (OMA) | Lane0 Lane1 Lane2 Lane3 |  |  | -8.2 -8.4 -8.6 -8.8 | dBm | 1 |
| RX_Los_Assert |  | -30 |  |  | dBm |  |
| RX_Los_De-ASSERT |  |  |  | -12 | dBm |  |
| RX_Los_Hysteresis |  | 0.5 |  |  | dBm |  |
| 1. unstressed sensitivity at BER of 5E-5 (pre-FEC) |  |  |  |  |  |  |

Digital Diagnostic Monitoring Specifications

| Parameters | Unit | Specification |
| --- | --- | --- |
| Temperature Monitor | ℃ | ±3 |
| Voltage Monitor | V | ±5 % |
| I_bias Monitor | mA | ±10% |
| Received Power(Rx)Monitor | dB | ±3.0 |
| Transmit Power(Tx)Monitor | dB | ±3.0 |

Electrical Characteristics

| Transmitter electrical input signal characteristics(TP1) | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- |
| Signaling rate per lane (range) | 25.78125±100 ppm |  |  | GBd |
| Differential input return loss | Equation (83E-5) |  |  | dB |
| Differential to common mode input return loss | Equation (83 E-6) |  |  | dB |
| Differential termination mismatch |  |  | 10 | % |
| Module stressed input test | See 83E3.4.1 |  |  |  |
| Differential pk-pk input voltage tolerance | 900 |  |  | mV |
| DC common mode voltage | -350 |  | 2850 | mV |
| Single ended voltage tolerance range | -0.4 |  | 3.3 | V |
| Signaling rate per lane(range) | 25.78125±100 ppm |  |  | GBd |
| AC common-mode output voltage(RMS) |  |  | 17.5 | mV |
| Differential output voltage |  |  | 900 | mV |
| Eye width | 0.57 |  |  | UI |
| Eye height,differential | 228 |  |  | mV |
| Vertical eye closure |  |  | 5.5 | dB |
| Differential output return loss | Equation (8 3E-2) |  |  | dB |
| Common to differential mode conversion return loss | Equation (8 3E-3) |  |  | dB |
| Differential termination mismatch |  |  | 10 | % |
| Transition time(20%to 80%) | 12 |  |  | ps |
| DC common mode voltage | -350 |  | 2850 | mV |

QSFP28 Connector and Pinout Description

The electrical interface to the transceiver is a 38-pin edge connector.The 38 pins provide high speed  data, low-speed  monitoring  and  control  signals,I2C  communication, and power  and  ground connectivity.The top and  bottom views of the connector are  provided  below,as well as a table outlining  the  contact  numbering, symbol, and  full  description.

38

36

34

33

32

30

29

28

27

26

25

23

22

21

20

Module Card Edge

GND  Tx1n Tx1p GND  Tx3n Tx3p GND

LPMode/TxDis Vcc1

VccTx

IntL/RxLOSL ModPrsL

GND  Rx4p Rx4n GND  Rx2p Rx2n GND

GND

Tx2n

Tx2p

GND        4

Tx4n

Tx4p         6

GND

ModselL

ResetL

VccRx     10

SCL        11

SDA         12

GND        13

Rx3p          14

Rx3n          15

GND        16

Rx1p          17

Rx1n          18

GND        19

Top Side

Viewed From Top

Bottom Side

Viewed From Bottom

Figure 1.QSFP28-compliant 38-pin connector

| Pin | Symbol | Name/Description | NOTE |
| --- | --- | --- | --- |
| 1 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 | GND | Transmitter Ground(Common with Receiver Ground) | 1 |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | VccRx | 3.3V Power Supply Receiver | 2 |
| 11 | SCL | 2-Wire serial interface clock |  |
| 12 | SDA | 2-Wire serial Interface Data |  |
| 13 | GND | Transmitter Ground (Common with Receiver Ground) |  |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |

| 16 | GND | Transmitter Ground(Common with Receiver Ground) | 1 |
| --- | --- | --- | --- |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output |  |
| 19 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 20 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 | GND | Transmitter Ground(Common with Receiver Ground) | 工 |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Transmitter Ground(Common with Receiver Ground) | 1 |
| 27 | ModPrsl | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | VccTx | 3.3V power supply transmitter | 2 |
| 30 | Vcc1 | 3.3V power supply | 2 |
| 31 | LPMode | Low Power Mode,not connect |  |
| 32 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Output |  |
| 35 | GND | Transmitter Ground (Common with Receiver Ground) | 1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Output |  |
| 38 | GND | Transmitter Ground(Common with Receiver Ground) | 1 |

Notes: 1. GND is the symbol for signal and supply (power) common for QSFP28 modules. All are common within the QSFP28 module, and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal common ground plane.

2. VccRx, Vcc1, and VccTx are the receiving and transmission power suppliers and shall be applied

concurrently. Recommended host board power supply filtering is shown below. Vcc Rx, Vcc1, and Vcc Tx   may be internally connected within the QSFP28 transceiver module in any combination. The connector pins are each rated for a maximum current of 1000 mA.

Memory map

Compatible with SFF-8636

Mechanical Dimensions

Unit: mm

Pull tab color: Gray, Pantone 424U

18.35 ±0.10

4.20±0.10 2-1.10±0.18

18.45max

72 ±0.1

121.4±1

5.20 ±0.15

2.25 ±0.10

0.60 ±0.05

+0.10

3.30-0.350

13.07 ±0.30

53

48.8

3.95

29.60±0.10

Figure 2. Mechanical dimensions

Order Information

| Part Number | Description |
| --- | --- |
| JH-SW100-SR4C | 100G QSFP28 SWDM4 Optical Transceiver, O to +70℃ |