Product Specification

100G QSFP28 2km CWDM4 Transceivers

FEATURES

- Compliant with 100G CWDM4 MSA Specification

- Uncooled 4x25Gb/s CWDM transmitter

- Wide Operating Temperature(0°C~70°C)

- Maximum link length of 2km via Single Mode Fiber (SMF)

APPLICATIONS

- Data Center Backbone

- Ethernet Switches

- High-speed Servers

- High-performance Computing Clusters

- SAN, Routers, Hubs, Load Balancer

ABSOLUTE MAXIMUM RATINGS

| Parameter | Symbol | Conditions | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TStorage |  | -40 | +85 | °C |
| Relative Humidity | RH |  | 0 | +85 | % |

RECOMMENDED OPERATING CONDITIONS (T=25°C, unless noted)

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Case Temperature | TC |  | 0 |  | 70 | °C |
| Power Supply Voltage | VCC |  | 3.135 | 3.3 | 3.465 | V |
| Signaling Rate each Channel |  |  |  | 25.78125 |  | Gbps |
| Supply Noise Rejection |  |  | --- | --- | 100 | mV |
| Receiver Differential Data Output |  |  | --- | 100 |  | Ohm |
| Operating Distance | D |  | --- | --- | 2 | km |

ELECTRICAL CHARACTERISTICS (T=25°C, unless noted)

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Power Consumption |  |  |  |  | 3.5 | W |
| Supply Current | Icc |  |  |  | 1050 | mA |

TRANSMITTER CHARACTERISTICS (T=25°C, unless noted)

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Signaling rate, each lane (range) | GBb |  |  | 25.78125 |  | GBb |
| Center Wavelength | λ0 |  | 1264.5 |  | 1277.5 | nm |
|  | λ1 |  | 1284.5 |  | 1297.5 | nm |
|  | λ2 |  | 1304.5 |  | 1317.5 | nm |
|  | λ3 |  | 1324.5 |  | 1337.5 | nm |
| Side-mode suppression ratio | SMSR |  | 30 |  |  | dB |
| Total average launch power |  |  |  |  | 8.5 | dBm |
| Average launch power, each lane | Pf |  | -6.5 |  | 2.5 | dBm |
| Optical Modulation Amplitude (OMA), each lane | TxOMA |  | -5 |  | 2.5 | dBm |
| Transmitter and Dispersion Penalty | TDP |  |  |  | 3 | dB |
| Launch power in OMA minus TDP, each lane | Tx-TDP |  | -5 |  |  | dBm |
| Average launch power of OFF transmitter, each lane |  |  |  |  | -30 | dBm |
| Extinction ratio | ER |  | 3.5 |  |  | dB |
| Optical return loss tolerance |  |  |  |  | 20 | dB |
| Transmitter reflectance |  |  |  |  | -12 | dB |

RECEIVER CHARACTERISTICS (T=25°C, unless noted)

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Signaling rate, each lane (range) | GBb |  |  | 25.78125 |  | GBb |
| Center Wavelength | λ0 |  | 1264.5 |  | 1277.5 | nm |
|  | λ1 |  | 1284.5 |  | 1297.5 | nm |
|  | λ2 |  | 1304.5 |  | 1317.5 | nm |
|  | λ3 |  | 1324.5 |  | 1337.5 | nm |
| Damage threshold |  |  | 3.5 |  |  | dBm |
| Average power at receiver input, each lane |  |  | -10 |  | 2.5 | dBm |
| Receive power, each lane (OMA) |  |  |  |  | 2.5 | dBm |
| Receiver reflectance |  |  |  |  | -26 | dB |
| Receiver sensitivity (OMA) | SOMA | BER@5e-5 |  |  | -10 | dBm |
| LOS Assert | LOSA |  | -24 |  |  | dBm |
| LOS De-Assert | LOSD |  |  |  | -11.6 | dBm |
| LOS Hysteresis |  |  | 0.5 |  | 6 | dB |

PIN ASSIGNMENT

PIN DESCRIPTION

| PIN | Logic | Symbol | Name/Description | Note |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground |  |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 |  | GND | Ground |  |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 |  | GND | Ground |  |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver |  |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data |  |
| 13 |  | GND | Ground |  |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground |  |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground |  |
| 20 |  | GND | Ground |  |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground |  |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground |  |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter |  |
| 30 |  | Vcc1 | +3.3 V Power Supply |  |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground |  |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Output |  |
| 35 |  | GND | Ground |  |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output |  |
| 38 |  | GND | Ground |  |

OUTLINE DIMENSIONS

Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City