QSFP28-100G-CWDM4+

100G CWDM4+ QSFP28 Transceiver

DESCRIPTION

The CWDM4 QSFP28 optical transceivers are designed for a high-speed link at aggregated data rate of 103.13Gb/s with up to 10km transmission distance of single mode fiber. The transceiver integrates four lasers with center wavelengths of 1271nm, 1291nm, 1311nm and 1331nm to be multiplexed into a single mode fiber. Its receiver side uses an integrated optical demultiplexer to optically demultiplex four lanes of optical data streams.  The transceiver is compliant to QSFP28 MSA, CWDM4 MSA and applicable portions of IEEE802.3bm.

FEATURES

- Hot-pluggable QSFP28 footprint

- Supports 25G Dual-Rate operation

- Supports 100G Dual Mode operation with CDR auto-bypass mode

- Built-in digital diagnostic functions

- 4x25Gb/s CWDM transmitters and PIN receivers

- Duplex LC optical connector

- 10km reach on single mode fiber

- Single +3.3V power supply

- Low power consumption <3.5W

- Operating case temperature: 0~+70°C

APPLICATION

- 100G CWDM4 applications with FEC

- High-density datacenter switches, routers

ABSOLUTE MAXIMUM RATINGS (TC=25°C, UNLESS OTHERWISE NOTED)

Stresses in excess of the absolute maximum ratings can cause permanent damage to the device. These are absolute stress ratings only. Functional operation of the device is not implied at these or any other conditions in excess of those given in the operational sections of the data sheet. Exposure to absolute maximum ratings will cause permanent damage and/or adversely affect device reliability.

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -40 | - | +85 | ºC |  |
| Maximum Supply Voltage | Vcc | -0.3 | - | 3.6 | V |  |
| Operating Relative Humidity | RH | 15 | - | +85 | % |  |

RECOMMENDED OPERATING CONDITIONS

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Data Rate | DR |  | 103.1 |  | Gb/s |  |
| Operating Case Temperature | Top | 0 |  | 70 | oC |  |
| Bit Error Rate | BER |  |  | 5E-5 |  |  |
| Fiber Length on SMF per G.652 | L |  |  | 2000 | m | 1 |
| Loss budget over SMF per G.652 |  |  |  | 5 | dB |  |
| Supply Voltage | Vcc | 3.135 |  | 3.465 | V |  |
| Module Total Power | P |  |  | 3.5 | W |  |

Notes:

1. Tested with PRBS31, 5dB loss budget with FEC on the host.

PIN DESCRIPTIONS

| Pin | Symbol | Name/Description | Ref. |
| --- | --- | --- | --- |
| 1 | GND | Ground | 1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 | GND | Ground | 1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 | GND | Ground | 1 |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | Vcc Rx | +3.3 V Power supply receiver |  |
| 11 | SCL | 2-wire serial interface clock |  |
| 12 | SDA | 2-wire serial interface data |  |
| 13 | GND | Ground | 1 |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |
| 16 | GND | Ground | 1 |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output |  |
| 19 | GND | Ground | 1 |
| 20 | GND | Ground | 1 |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 | GND | Ground | 1 |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Ground | 1 |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | Vcc Tx | +3.3 V Power supply transmitter |  |
| 30 | Vcc1 | +3.3 V Power Supply |  |
| 31 | LPMode | Low Power Mode |  |
| 32 | GND | Ground | 1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground | 1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground | 1 |

1. Circuit ground is internally isolated from chassis ground.

Figure 1 – QSFP28-Compliant 38-Pin Connector (Per SFF-8679)

ELECTRICAL SPECIFICATION

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling rate per lane |  | 25.78125±100ppm |  |  | Gb/s |  |
| Differential input impedance | Zin |  | 100 |  | Ohm |  |
| Differential input voltage amplitude per lane | Vin | 100 |  | 1600 | mVp-p |  |
| Differential termination mismatch |  |  |  | 10 | % |  |
| Receiver |  |  |  |  |  |  |
| Signaling rate per lane |  | 25.78125±100ppm |  |  | Gb/s |  |
| Differential Output impedance | Zout |  | 100 |  | Ohm |  |
| Differential output voltage amplitude per lane | Vout | 100 |  | 400 | mVp-p | 1 |
|  |  | 300 |  | 600 |  |  |
|  |  | 400 |  | 800 |  |  |
|  |  | 600 |  | 1200 |  |  |
| Eye width |  | 0.57 |  |  | UI |  |
| Vertical eye closure |  |  |  | 5.5 | dB |  |
| Differential termination mismatch |  |  |  | 10 | % |  |

Notes:

1. Differential output voltage amplitude is adjustable through  I2C bus. Default value of the differential output voltage amplitude per lane is 400 – 800 mV.

OPTICAL SPECIFICATION

| Parameter | Symbol | Min |  | Typical |  | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |
| Signaling rate, each lane |  | 25.78125±100ppm |  |  |  |  | Gb/s |  |
| Line wavelengths (range) | λ | 1264.5 – 1277.5 |  |  |  |  | nm |  |
|  |  | 1284.5 – 1297.5 |  |  |  |  |  |  |
|  |  | 1304.5 – 1317.5 |  |  |  |  |  |  |
|  |  | 1324.5 – 1337.5 |  |  |  |  |  |  |
| Side-mode Suppression ratio | SMSR | 30 |  |  |  |  | dB |  |
| Total average launch power |  |  |  |  | 8.5 |  | dBm |  |
| Average launch power, each lane |  | -4 |  |  | 2.5 |  | dBm |  |
| Transmit OMA, each lane | TXOMA1 | -4 |  |  | 2.5 |  | dBm |  |
| Launch Power (OMA-TDP), each lane | OMA-TDP | -5 |  |  |  |  | dBm |  |
| Optical Extinction Ratio | ER | 3.5 |  |  |  |  | dB |  |
| Transmitter and dispersion penalty, each lane | TDP |  |  |  | 3 |  | dB | 1 |
| Average launch power of OFF transmitter, each lane |  |  |  |  | -30 |  | dBm |  |
| Transmitter Reflectance | RL |  |  |  | -12 |  | dB |  |
| Transmitter eye mask definition {X1, X2, X3, Y1, Y2, Y3} |  | {0.31, 0.4, 0.45, 0.34, 0.38, 0.4} |  |  |  |  | dB | 2 |
| Receiver |  |  |  |  |  |  |  |  |
| Signaling rate, each lane |  | 25.78125±100ppm |  |  |  |  | Gb/s |  |
| Line wavelengths (range) | λ | 1264.5 – 1277.5 |  |  |  |  | nm |  |
|  |  | 1284.5 – 1297.5 |  |  |  |  |  |  |
|  |  | 1304.5 – 1317.5 |  |  |  |  |  |  |
|  |  | 1324.5 – 1337.5 |  |  |  |  |  |  |
| Damage threshold, each lane |  | 3.5 |  |  |  |  | dBm |  |
| Average receive power, each lane |  | -11.5 |  |  | 2.5 |  | dBm |  |
| Unstressed Receiver Sensitivity (OMA), each lane | RXsens |  |  |  | -10 |  | dBm | 3 |
| Stressed Receiver OMA Sensitivity | SRS |  |  |  | -12 |  | dBm | 4 |
| Conditions of stressed receiver sensitivity test: |  |  |  |  |  |  |  |  |
| Vertical Eye Closure Penalty | VECP |  | 1.9 |  |  |  | dB |  |
| Stressed J2 Jitter, each lane | J2 |  | 0.33 |  |  |  | UI |  |
| Stressed J4 Jitter, each lane | J4 |  | 0.48 |  |  |  | UI |  |
| SRS eye mask definition {X1, X2, X3, Y1, Y2, Y3} |  |  | {0.39, 0.5, 0.5, 0.39, 0.39, 0.4} |  |  |  | dB |  |
| LOS De-Assert | LOSD |  |  |  | -12 |  | dBm |  |
| LOS Assert | LOSA |  | -30 |  |  |  | dBm |  |
| LOS Hysteresis | LOSH |  | 0.5 |  | 5 |  | dB |  |

Notes:

1. TDP does not include a penalty for multi-path interference (MPI).

2. Hit ratio of 5E-5, per IEEE.

3. Sensitivity is specified at the BER of 5E-5.

4. Measured with CWDM4 MSA conformance test signal at TP3 for the BER of 5E-5.

DIGITAL DIAGNOSTIC FUNCTIONS

Figure 2 – Two-Wire Interface Fields

The operating and diagnostics information is monitored and reported by a Digital Diagnostics Transceiver Controller (DDTC) inside the transceiver, which is accessed through a 2-wire serial interface. The 2-wire serial interface shall consist of a master and slave. The fixed side shall be the master and the free side shall be the slave. Control and data are transferred serially. The master shall initiate all data transfers. Data can be transferred from the master to the slave and from the slave to the master. The 2-wire interface shall consist of clock (SCL) and data (SDA) signals. The master utilizes SCL to clock data and control information on the 2-wire bus. The master and slave shall latch the state of SDA on the positive transitioning edge of SCL. The SDA signal is bi-directional. During data transfer, the SDA signal shall transition when SCL is low. A transition on the SDA signal while SCL is high shall indicate a stop or start condition.

MECHANICAL SPECIFICATIONS

ORDERING INFORMATION

QSFP28-100G-CWDM4+

ESD SAFETY CAUTIONS

This transceiver is specified as ESD threshold 1KV for high speed data pins and 2KV for all others electrical input pins, tested per MIL-STD-883, Method 3015.4 /JESD22-A114-A (HBM).  However, normal ESD precautions are still required during the handling of this module. This transceiver is shipped in ESD protective packaging. It should be removed from the packaging and handled only in an ESD protected environment.

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City