# 100G QSFP28 FR1 2KM Optical Transceiver

## Features

l Supports 100GBASE-FR

l Optical port support 106.25Gb/s PAM4

l Electrical port support 4x26.5625Gb/s NRZ

l Up to 2km transmission on SMF

l EML Laser and PIN receiver

l High speed I/O electrical interface

l I2C interface with integrated Digital Diagnostic monitoring

l QSFP28 MSA package with simplex LC connector

l Single +3.3V power supply

l Maximum power consumption 4.5 W

l Operating case temperature: 0 to +70 °C

l Compliant to IEE802.3cu

l Compliant to SFF-8636

l Complies to RoHS compliant

## Applications

l 100GBASE-FR1

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | -0.5 | 4.0 | V |
| Storage Temperature | Tst | -40 | 85 | ºC |
| Humidity(non-condensing) | Rh | 5 | 85 | % |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |
| Power Supply Voltage | VCC | 3.13 | 3.3 | 3.47 | V |  |
| Power Supply Current | ICC | - | - | 1.43 | A |  |
| Maximum Power Dissipation | PD | - | - | 4.5 | W |  |
| Data Rate(optical) | DRo | - | 53.125 | - | GBd | PAM4 |
| Transmission Distance | TD | - | - | 2 | km | Over SMF |

## Electrical Characteristics

High-Speed Signal: Compliant to CAUI-4 (IEEE 802.3cu)

Low-Speed Signal: Compliant to SFF-8679

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter (Module Input) |  |  |  |  |  |  |
| Input Differential Impedance | Rin | - | 100 | - | Ohm |  |
| Differential Data Input Amplitude | VIN,P-P | 80 | - | 900 | mVpp |  |
| Differential termination mismatch (max) | D-mismatch | - | - | 10 | % |  |
| DC common-mode input voltage |  | -0.3 | - | 2.8 | V |  |
| Transition time (20%~80%) | Tr Tf | 10 | - | - | ps |  |
| LPMode, Reset and ModSelL / Tx dis | VIL | -0.3 | - | 0.8 | V |  |
| LPMode, Reset and ModSelL / Tx dis | VIH | 2.0 | - | VCC+0.3 | V |  |
| Receiver (Module Output) |  |  |  |  |  |  |
| Output Differential Impedance | Rout | - | 100 | - | Ohm |  |
| Differential Data Output Amplitude | VOUT,P-P | - | - | 900 | mVpp |  |
| Differential termination mismatch (max) | D-mismatch | - | - | 10 | % |  |
| Transition time, 20% to 80% | Tr Tf | 12 | - |  | ps |  |
| Mod PrsL and IntL/ Rx los | VOL | 0 | - | 0.4 | V |  |
| Mod PrsL and IntL/ Rx los | VOH | VCC-0.5 | - | VCC+0.3 | V |  |

## Optical Characteristics

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Center Wavelength | CW | 1304.5 | - | 1317.5 | nm |  |
| Average Launch Power | PTX | -3.1 | - | 4 | dBm | 1 |
| Outer Optical Modulation Amplitude | OMA | -0.2 | - | 4.2 | dBm | 1 |
| Transmitter and dispersion eye closure for PAM4 (TDECQ) (max) | TDECQ | - | - | 3.4 | dB |  |
| Average Output Power (Laser Turn off) | P0UT-OFF | - | - | -15 | dBm |  |
| Side Mode Suppression Ratio | SMSR | 30 | - | - | dB |  |
| Extinction Ratio | ER | 3.5 | - | - | dB |  |
| Receiver |  |  |  |  |  |  |
| Center Wavelength | CW | 1304.5 | - | 1317.5 | nm |  |
| Damage threshold | PDamage | 5 | - | - | dBm | 2 |
| Average Rx Power | PRX | -7.1 | - | 4 | dBm | 3 |

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Receive power_OMAouter | POMA | - | - | 4.2 | dBm |  |
| Los Assert | LosA | -26 | - | -12 | dBm |  |
| Los De-Assert | LosDA | - | - | -10 | dBm |  |
| Los Hysteresis | LosH | 0.5 | - | - | dB |  |
| Receiver sensitivity (OMAouter) | Sen |  |  | -4.5 | dBm |  |

Notes:

1. The optical power is launched into SMF.

2 . The receiver shall be able to tolerate, without damage, continuous exposure to an optical input signal having this average power level. The receiver does not have to operate correctly at this input power.

3. Average receive power, each lane (min) is informative and not the principal indicator of signal strength.

## Digital Diagnostics

| Parameter | Accuracy | Unit | Calibration |
| --- | --- | --- | --- |
| Temperature | ±3 | ºC | Internal |
| Voltage | ±3 | V | Internal |
| Tx Bias Current | ±10% | mA | Internal |
| Tx Output Power | ±3 | dB | Internal |
| Rx Power | ±3 | dB | Internal |

Pin Descriptions

| PIN | Logic | Symbol | Description | Plug Seq. | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1 | 1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 3 |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output | 3 |  |
| 4 |  | GND | Ground | 1 | 1 |
| 5 |  | NC |  | 3 |  |
| 6 |  | NC |  | 3 |  |
| 7 |  | GND | Ground | 1 | 1 |
| 8 | LVTLL-I | ModSelL | Module Select | 3 |  |
| 9 | LVTLL-I | ResetL | Module Reset | 3 |  |
| 10 |  | VccRx | +3.3V Power Supply Receiver | 2 | 2 |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock | 3 |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data | 3 |  |
| 13 |  | GND | Ground | 1 |  |
| 14 |  | NC |  | 3 |  |
| 15 |  | NC |  | 3 |  |
| 16 |  | GND | Ground | 1 | 1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output | 3 |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 3 |  |
| 19 |  | GND | Ground | 1 | 1 |

| PIN | Logic | Symbol | Description | Plug Seq. | Notes |
| --- | --- | --- | --- | --- | --- |
| 20 |  | GND | Ground | 1 | 1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 3 |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output | 3 |  |
| 23 |  | GND | Ground | 1 | 1 |
| 24 |  | NC |  | 3 |  |
| 25 |  | NC |  | 3 |  |
| 26 |  | GND | Ground | 1 | 1 |
| 27 | LVTTL-O | Mod PrsL | Module Present | 3 |  |
| 28 | LVTTL-O | IntL | Interrupt | 3 |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter | 2 | 2 |
| 30 |  | Vcc1 | +3.3 V Power Supply | 2 | 2 |
| 31 | LVTTL-I | LPMode | Low Power Mode | 3 |  |
| 32 |  | GND | Ground | 1 | 1 |
| 33 |  | NC |  | 3 |  |
| 34 |  | NC |  | 3 |  |
| 35 |  | GND | Ground | 1 | 1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input | 3 |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output | 3 |  |
| 38 |  | GND | Ground | 1 | 1 |
| Note 1: GND is the symbol for signal and supply (power) common for the QSFP28 module. All are common within the QSFP28 module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane. |  |  |  |  |  |
| Note 2: Vcc Rx, Vcc1 and Vcc Tx are the receiver and transmitter power supplies and shall be applied concurrently. Requirements defined for the host side of the Host Edge Card Connector are listed in MSA. The connector pins are each rated for a maximum current of 1000 mA. |  |  |  |  |  |

Mechanical Dimensions

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive

Bonded Zone,Donghu High-tech Zone, Wuhan City