100G QSFP28 BIDI SR Transceiver

# Features

l QSFP28 MSA Specification Compliant, including new functions per SFF-8636

l 100G and 40G  Link  Distances  up to 70m over OM3,  100m  over OM4

l Hot Pluggable

l Dual Wavelength VCSEL Bi-Directional Optical Interface, PAM4 2x50Gbps 850nm/908nm

l Interface with Digital monitoring and maskable Interrupts for expanded functionality

l Case Operating Temperature: Range: 0 to 70ºC

l RoHS II Compliance

l Operating Mode

# Applications

l Data Center

l High performance computing interconnect

# Description

The  JH-100G-QSFP28-SR1.2  is  a  QSFP28 optical transceiver  for  100Gbps  and  40Gbps interconnects.  It  is

compliant with the QSFP28 MSA specifications, operates from a 3.3V DC power supply.  It is fabricated with a

rugged die-cast metal housing and cage assembly.

Absolute Maximum Ratings

| Parameter | Symbol | Min | Typ | Max |
| --- | --- | --- | --- | --- |
| Storage Ambient Temperature | Tstg | -40 | +85 | ºC |
| Relative Humidity - Storage | RHS | 0 | 95 | % |
| Relative Humidity - Operating | RHO | 0 | 85 | % |
| Module Supply Voltage | VCC | -0.5 | 3.6 | V |

Notes:

to  be  operated  under  the  condition  of  simulta-  neous Absolute  Maximum  Ratings,  a  condition  which  may  cause

Exceeding the Absolute Maximum Ratings may cause irreversible damage to the device. The device is not  intended

irreversible damage to the device.RH is Non-condensing condition.

Recommended Operating Conditions    (T=25 ° C, unless noted)

| Parameter | Symbol | Min. | Max. | Unit |  |
| --- | --- | --- | --- | --- | --- |
| Case Operating Temperature | Tcase | 0 | +25 | +70 | ºC |
| Module Supply Voltage | VCC | 3.135 | 3.3 | 3.465 | V |
| Signaling Speed Per electricalchannel (4x25G NRZ) | S | - | 25.78 | - | Gb/s |
| Power Consumption | P | - | - | 4 | W |

Notes

Exceeding the Absolute Maximum Ratings may cause irreversible damage to the device. The device is not intended to be  operated   under  the  condition   of  simultaneous  Absolute   Maximum   Ratings,   a  condition  which   may   cause irreversible damage to the device. RH is a non-condensing condition.

100G QSFP28 BIDI SR Transceiver

Transmitter Electrical Interfaces

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Tx_Data Differential Input Voltage | VIN | 200 | - | 900 | mV |
| Tx_Data Differential Input Impedance | Z IN | - | 100 | - | Ω |

Receiver Electrical Interfaces

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Rx_Data Differential Output Voltage | VOUT | - | - | 1200 | mV |
| Rx_Data Differential Output Impedance | ZOUT | - | 100 | -- | Ω |

Timing requirement of control and status I/O

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Tx Squelch Deassert Time | toff_Txsq | - | - | 1 | s |
| Rx Squelch Deassert Time | toff_Rxsq | - | - | 2 | s |

Non-Volatile Memory Specification

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Complete Single or Sequential Write | twr | - | - | 80 | ms |

Soft Control and Status Timing Requirements

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Application or Rate Select Change Time | t_ratesel | - | - | 600 | ms |

100 G Transmitter Optical Characteristics

| Parameter | Symbol | Test Point | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average Launch Power, each lane | POUT | TP2 | -6.2 | - | 4 | dBm | Average Optical Output |
| Optical modulation Amplitude , each lane | OMA | TP2 | -4.2 | - | 3 | dBm |  |
| Extinction Ratio | ER | TP2 | 3 | - | - | dB |  |
| Launch power in OMAouter minus TDECQ, each lane | OMA TDECQ | TP2 | -5.9 | - | - | dB |  |
| Optical Output with Tx OFF | POFF | TP2 | - | - | -30 | dBm |  |
| Center Wavelength 1 | λ | TP2 | 844 | 850 | 863 | nm |  |
| Center Wavelength 2 | λ | TP2 | 900 | 908 | 918 | nm |  |
| RMS Spectral Width1 | Δλ | TP2 | - | - | 0.6 | nm |  |
| RMS Spectral Width2 | Δλ | TP2 | - | - | 0.65 | nm |  |
| Optical return loss tolerance | ORL | TP2 | - | - | 12 | dB |  |
| Transmitter and dispersion eye closure for PAM4 (TDECQ), each lane | TDECQ | TP2 |  |  | 4.9 | dB |  |

100G QSFP28 BIDI SR Transceiver

100 G Receiver Optical Characteristics

| Parameter | Symbol | Test Point | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average power at receive input, each lane | PIN | TP3 | -7.9 | - | 4 | dBm | Note1 |
| Receive power, each lane (OMAouter) | PIN (OMA) | TP3 | -5.9 | - | 3 | dBm |  |
| Unstressed receiver sensitivity (OMAouter), each lane | Sen | TP3 | - | - | max (-6.6, SECQ-8) | dBm | Note2 |
| Center Wavelength1 | λ | TP3 | 844 | 850 | 863 | nm |  |
| Center Wavelength2 | λ | TP3 | 900 | 908 | 918 | nm |  |
| Receiver Reflectance | RFL | TP3 | - | - | -12 | dB |  |
| Rx_LOS of Signal - Assert | PA | TP3 | -30 | - | - | dBm |  |
| Rx_LOS of Signal - Deassert | PD | TP3 | - | - | -5.9 | dBm |  |
| Rx_LOS of Signal - Hysteresis | PHy | TP3 | 0.5 | - | - | dB |  |

Note 1: A received power below this value cannot be compliant; however, a value above this does not ensure compliance.

Note 2: Sensitivity where the BER=2.4E-4 measured with a PRBS 31Q testpattern@26.56GBaud.

The mode Select description:

The firmware uses only rx2_rate_select and rx1_rate_select to switch between 2 types of modes, and only byte 87, bits 3~0 are used  in  another word.  ● The  host  can  switch  the  module  to  4*25G  NRZ->2*53G  PAM4 for  default  mode  (with standard KP4 FEC) by setting rx2_rate_select to 00b and rx1_rate_select to 11b, byte87=03h in another word. ● The host  can  switch  the  module  to  4*10G   NRZ->2*20G  NRZ  for  40G  mode   by  setting  rx2_rate_select  to  00b  and rx1_rate_select to 01b, byte87 to 01h, in another word. ● The mode select is a volatile operation; after reset, the module is   back    to   default    rate:    4*25G    NRZ->2*53G   PAM4    for     normal   work    mode,    rx2_rate_select=11b   and rx1_rate_select=00b, byte87=03h in another word.

| Type | Mode Description | Host Side | Media Side | Media side FEC | Page00 Byte 87 |
| --- | --- | --- | --- | --- | --- |
| 1 | Default mode | 4*25.78G NRZ | 2*53.125G PAM4 | With standard KP4 | 0x03 |
| 2 | 40G mode | 4*10G NRZ | 2*20G NRZ | No FEC | 0x01 |

Notes:

In 40G mode, we only test the connectivity of 100m OM4 fiber, and do not do other tests, nor do we guarantee other indicators in this mode.

The content of the customer's second-level password is: 0x9715 DAED The range of open register information is as follows  :

page0: byte0

page0: byte105-110

page0: byte113-116

page0: byte128-255

page2: byte128-255

page3: byte128-229

100G QSFP28 BIDI SR Transceiver

| I2C Memory Map (Based on SFF8636 2.10a) Lower Page |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| Byte | Bit |  |  |  |  |  |
| 0 | 7-0 | Identifier |  | 11h | Read-Only | Yes |
| 1 | 7-0 | Revision Compliance |  | 08h | Read-Only | Yes |
| 2 | 7-4 | Reserved |  | status |  |  |
|  | 3 | Reserved |  |  |  |  |
|  | 2 | Flat_mem | 0b |  | Read-Only | Yes |
|  | 1 | IntL |  |  | Read-Only | Yes |
|  | 0 | Data Not Ready |  |  | Read-Only | Yes |
| 3 | 7 | L-Tx4 LOS | status | status | Read-Only | Yes |
|  | 6 | L-Tx3 LOS | status |  | Read-Only | Yes |
|  | 5 | L-Tx2 LOS | status |  | Read-Only | Yes |
|  | 4 | L-Tx1 LOS | status |  | Read-Only | Yes |
|  | 3 | L-Rx4 LOS | status |  | Read-Only | Yes |
|  | 2 | L-Rx3 LOS | status |  | Read-Only | Yes |
|  | 1 | L-Rx2 LOS | status |  | Read-Only | Yes |
|  | 0 | L-Rx1 LOS | status |  | Read-Only | Yes |
| 4 | 7 | L-Tx4 Adapt EQ Fault | status | status | Read-Only | Yes |
|  | 6 | L-Tx3 Adapt EQ Fault | status |  | Read-Only | Yes |
|  | 5 | L-Tx2 Adapt EQ Fault | status |  | Read-Only | Yes |
|  | 4 | L-Tx1 Adapt EQ Fault | status |  | Read-Only | Yes |
|  | 3 | L-Tx4 Fault | status |  | Read-Only | Yes |
|  | 2 | L-Tx3 Fault | status |  | Read-Only | Yes |
|  | 1 | L-Tx2 Fault | status |  | Read-Only | Yes |
|  | 0 | L-Tx1 Fault | status |  | Read-Only | Yes |
| 5 | 7 | L-Tx4 LOL | status | status | Read-Only | Yes |
|  | 6 | L-Tx3 LOL | status |  | Read-Only | Yes |
|  | 5 | L-Tx2 LOL | status |  | Read-Only | Yes |
|  | 4 | L-Tx1 LOL | status |  | Read-Only | Yes |
|  | 3 | L-Rx4 LOL | status |  | Read-Only | Yes |
|  | 2 | L-Rx3 LOL | status |  | Read-Only | Yes |
|  | 1 | L-Rx2 LOL | status |  | Read-Only | Yes |
|  | 0 | L-Rx1 LOL | status |  | Read-Only | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 6 | 7 | L-Temp High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Temp Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Temp High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Temp Low Warning | status |  | Read-Only | Yes |
|  | 3-2 | Reserved | status |  |  |  |
|  | 1 | TC readiness flag | status |  | Read-Only | Yes |
|  | 0 | Initialization complete flag | status |  | Read-Only | Yes |
| 7 | 7 | L-Vcc High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Vcc Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Vcc High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Vcc Low Warning | status |  | Read-Only | Yes |
|  | 3-0 | Reserved | status |  |  |  |
| 8 | All | Vendor-Specific |  |  | Read-Only |  |
| 9 | 7 | L-Rx1 Power High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Rx1 Power Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Rx1 Power High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Rx1 Power Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Rx2 Power High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Rx2 Power Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Rx2 Power High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Rx2 Power Low Warning | status |  | Read-Only | Yes |
| 10 | 7 | L-Rx3 Power High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Rx3 Power Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Rx3 Power High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Rx3 Power Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Rx4 Power High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Rx4 Power Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Rx4 Power High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Rx4 Power Low Warning | status |  | Read-Only | Yes |
| 11 | 7 | L-Tx1 Bias High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Tx1 Bias Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Tx1 Bias High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Tx1 Bias Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Tx2 Bias High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Tx2 Bias Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Tx2 Bias High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Tx2 Bias Low Warning | status |  | Read-Only | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 12 | 7 | L-Tx3 Bias High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Tx3 Bias Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Tx3 Bias High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Tx3 Bias Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Tx4 Bias High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Tx4 Bias Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Tx4 Bias High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Tx4 Bias Low Warning | status |  | Read-Only | Yes |
| 13 | 7 | L-Tx1 Power High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Tx1 Power Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Tx1 Power High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Tx1 Power Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Tx2 Power High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Tx2 Power Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Tx2 Power High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Tx2 Power Low Warning | status |  | Read-Only | Yes |
| 14 | 7 | L-Tx3 Power High Alarm | status | status | Read-Only | Yes |
|  | 6 | L-Tx3 Power Low Alarm | status |  | Read-Only | Yes |
|  | 5 | L-Tx3 Power High Warning | status |  | Read-Only | Yes |
|  | 4 | L-Tx3 Power Low Warning | status |  | Read-Only | Yes |
|  | 3 | L-Tx4 Power High Alarm | status |  | Read-Only | Yes |
|  | 2 | L-Tx4 Power Low Alarm | status |  | Read-Only | Yes |
|  | 1 | L-Tx4 Power High Warning | status |  | Read-Only | Yes |
|  | 0 | L-Tx4 Power Low Warning | status |  | Read-Only | Yes |
| 15-16 | All | Reserved |  |  |  |  |
| 17-18 | All | Reserved |  |  |  |  |
| 19-21 | All | Vendor-Specific |  |  |  |  |
| 22 | 7-0 | Temperature MSB | status | status | Read-Only | Yes |
| 23 | 7-0 | Temperature LSB | status | status | Read-Only | Yes |
| 24-25 | All | Reserved |  |  |  |  |
| 26 | 7-0 | Supply Voltage MSB | status | status | Read-Only | Yes |
| 27 | 7-0 | Supply Voltage LSB | status | status | Read-Only | Yes |
| 28-29 | All | Reserved |  |  |  |  |
| 30-33 | All | Vendor-Specific |  |  |  |  |
| 34 | 7-0 | Rx1 Power MSB | status | status | Read-Only | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 35 | 7-0 | Rx1 Power LSB | status | status | Read-Only | Yes |
| 36 | 7-0 | Rx2 Power MSB | status | status | Read-Only | Yes |
| 37 | 7-0 | Rx2 Power LSB | status | status | Read-Only | Yes |
| 38 | 7-0 | Rx3 Power MSB | status | status | Read-Only | Yes |
| 39 | 7-0 | Rx3 Power LSB | status | status | Read-Only | Yes |
| 40 | 7-0 | Rx4 Power MSB | status | status | Read-Only | Yes |
| 41 | 7-0 | Rx4 Power LSB | status | status | Read-Only | Yes |
| 42 | 7-0 | Tx1 Bias MSB | status | status | Read-Only | Yes |
| 43 | 7-0 | Tx1 Bias LSB | status | status | Read-Only | Yes |
| 44 | 7-0 | Tx2 Bias MSB | status | status | Read-Only | Yes |
| 45 | 7-0 | Tx2 Bias LSB | status | status | Read-Only | Yes |
| 46 | 7-0 | Tx3 Bias MSB | status | status | Read-Only | Yes |
| 47 | 7-0 | Tx3 Bias LSB | status | status | Read-Only | Yes |
| 48 | 7-0 | Tx4 Bias MSB | status | status | Read-Only | Yes |
| 49 | 7-0 | Tx4 Bias LSB | status | status | Read-Only | Yes |
| 50 | 7-0 | Tx1 Power MSB | status | status | Read-Only | Yes |
| 51 | 7-0 | Tx1 Power LSB | status | status | Read-Only | Yes |
| 52 | 7-0 | Tx2 Power MSB | status | status | Read-Only | Yes |
| 53 | 7-0 | Tx2 Power LSB | status | status | Read-Only | Yes |
| 54 | 7-0 | Tx3 Power MSB | status | status | Read-Only | Yes |
| 55 | 7-0 | Tx3 Power LSB | status | status | Read-Only | Yes |
| 56 | 7-0 | Tx4 Power MSB | status | status | Read-Only | Yes |
| 57 | 7-0 | Tx4 Power LSB | status | status | Read-Only | Yes |
| 58-65 |  | Reserved channel monitor set 4 |  |  |  |  |
| 66-73 |  | Reserved channel monitor set 5 |  |  |  |  |
| 74-81 |  | Vendor-Specific |  |  |  |  |
| 82-85 |  | Reserved |  | 00h |  |  |
| 86 | 7-4 | Reserved |  | 00h |  |  |
|  | 3 | Tx4 Disable | 0b |  | Read/Write | Yes |
|  | 2 | Tx3 Disable | 0b |  | Read/Write | Yes |
|  | 1 | Tx2 Disable | 0b |  | Read/Write | Yes |
|  | 0 | Tx1 Disable | 0b |  | Read/Write | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 87 | 7 | Rx4_Rate_select | 0b | 03h | Read/Write | No |
|  | 6 | Rx4_Rate_select | 0b |  | Read/Write | No |
|  | 5 | Rx3_Rate_select | 0b |  | Read/Write | No |
|  | 4 | Rx3_Rate_select | 0b |  | Read/Write | No |
|  | 3 | Rx2_Rate_select | 0b |  | Read/Write | No |
|  | 2 | Rx2_Rate_select | 0b |  | Read/Write | No |
|  | 1 | Rx1_Rate_select | 1b |  | Read/Write | Yes |
|  | 0 | Rx1_Rate_select | 1b |  | Read/Write | Yes |
| 88 | 7 | Tx4_Rate_select | 0b | 00h | Read/Write | No |
|  | 6 | Tx4_Rate_select | 0b |  | Read/Write | No |
|  | 5 | Tx3_Rate_select | 0b |  | Read/Write | No |
|  | 4 | Tx3_Rate_select | 0b |  | Read/Write | No |
|  | 3 | Tx2_Rate_select | 0b |  | Read/Write | No |
|  | 2 | Tx2_Rate_select | 0b |  | Read/Write | No |
|  | 1 | Tx1_Rate_select | 0b |  | Read/Write | No |
|  | 0 | Tx1_Rate_select | 0b |  | Read/Write | No |
| 89-92 | All | Reserved |  | 00h |  |  |
| 93 | 7 | SW Reset | 0b | 00h | Read/Write | Yes |
|  | 6-4 | Reserved |  |  |  |  |
|  | 3 | High Power Class Enable (Class 8) | 0b |  | Read/Write | No |
|  | 2 | High Power Class Enable (Class 5–7) | 0b |  | Read/Write | No |
|  | 1 | Power set | 0b |  | Read/Write | Yes |
|  | 0 | Power override | 0b |  | Read/Write | Yes |
| 94-97 | All | Reserved |  | 00h |  |  |
| 98 | 7 | Tx4_CDR_control | 1b | FFh | Read/Write | No |
|  | 6 | Tx3_CDR_control | 1b |  | Read/Write | No |
|  | 5 | Tx2_CDR_control | 1b |  | Read/Write | No |
|  | 4 | Tx1_CDR_control | 1b |  | Read/Write | No |
|  | 3 | Rx4_CDR_control | 1b |  | Read/Write | No |
|  | 2 | Rx3_CDR_control | 1b |  | Read/Write | No |
|  | 1 | Rx2_CDR_control | 1b |  | Read/Write | No |
|  | 0 | Rx1_CDR_control | 1b |  | Read/Write | No |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 99 | 7-2 | Reserved |  | 00h |  |  |
|  | 1 | LP/TxDis ctrl | 0b | 00h | Read/Write | LP only |
|  | 0 | IntL/LOSL ctrl | 0b |  | Read/Write | IntL only |
| 100 | 7 | M-Tx4 LOS Mask | 0b | 00h | Read/Write | Yes |
|  | 6 | M-Tx3 LOS Mask | 0b |  | Read/Write | Yes |
|  | 5 | M-Tx2 LOS Mask | 0b |  | Read/Write | Yes |
|  | 4 | M-Tx1 LOS Mask | 0b |  | Read/Write | Yes |
|  | 3 | M-Rx4 LOS Mask | 0b |  | Read/Write | Yes |
|  | 2 | M-Rx3 LOS Mask | 0b |  | Read/Write | Yes |
|  | 1 | M-Rx2 LOS Mask | 0b |  | Read/Write | Yes |
|  | 0 | M-Rx1 LOS Mask | 0b |  | Read/Write | Yes |
| 101 | 7 | M-Tx4 Adapt EQ Fault Mask |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx3 Adapt EQ Fault Mask |  |  | Read/Write | Yes |
|  | 5 | M-Tx2 Adapt EQ Fault Mask |  |  | Read/Write | Yes |
|  | 4 | M-Tx1 Adapt EQ Fault Mask |  |  | Read/Write | Yes |
|  | 3 | M-Tx4 Transmitter Fault Mask |  |  | Read/Write | Yes |
|  | 2 | M-Tx3 Transmitter Fault Mask |  |  | Read/Write | Yes |
|  | 1 | M-Tx2 Transmitter Fault Mask |  |  | Read/Write | Yes |
|  | 0 | M-Tx1 Transmitter Fault Mask |  |  | Read/Write | Yes |
| 102 | 7 | M-Tx4 CDR LOL Mask |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx3 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 5 | M-Tx2 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 4 | M-Tx1 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 3 | M-Rx4 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 2 | M-Rx3 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 1 | M-Rx2 CDR LOL Mask |  |  | Read/Write | Yes |
|  | 0 | M-Rx1 CDR LOL Mask |  |  | Read/Write | Yes |
| 103 | 7 | M-Temp High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M - Temp Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Temp High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Temp Low Warning |  |  | Read/Write | Yes |
|  | 3-2 | Reserved |  |  |  |  |
|  | 1 | M-TC readiness flag |  |  | Read/Write | No |
|  | 0 | Reserved |  |  |  |  |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 104 | 7 | M-Vcc High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Vcc Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Vcc High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Vcc Low Warning |  |  | Read/Write | Yes |
|  | 3-0 | Reserved |  |  |  |  |
| 105-106 | All | Vendor-Specific |  | 00h | Read/Write |  |
| 107 | 7-0 | Max Power Consumption |  | 28h | Read/Write | Yes |
| 108 | 7-0 | Propagation Delay MSB |  | 00h | Read/Write | Yes |
| 109 | 7-0 | Propagation Delay LSB |  | 02h | Read/Write | Yes |
| 110 | 7-4 | Advanced Low Power Mode |  | 30h | Read/Write | Yes |
|  | 3 | Far Side Managed |  |  | Read/Write | Yes |
|  | 2-0 | Min Operating Voltage |  |  | Read/Write | Yes |
| 111-112 | All | Assigned for use by PCI Express |  |  | Read/Write | No |
| 113 | 7 | Reserved |  | 00h |  |  |
|  | 6-4 | Far-End Implementation |  |  | Read/Write | Yes |
|  | 3-0 | Near-End Implementation |  |  | Read/Write | Yes |
| 114 | 7-4 | Tx_TurnOn MaxDuration |  | 00h | Read/Write | Yes |
|  | 3-0 | DatapathInit MaxDuration |  |  | Read/Write | Yes |
| 115 | 7-5 | ModeSelL wait time exponent |  | 34h | Read/Write | Yes |
|  | 4-0 | ModSelL wait time mantissa |  |  | Read/Write | Yes |
| 116 | 7-0 | Secondary Extended Spec Compliance |  | 00h | Read/Write | Yes |
| 117-118 | All | Reserved |  | 00h |  |  |
| 119-122 | 7-0 | Password Change Entry Area |  | 00001011h | Read/Write | Yes |
| 123-126 | 7-0 | Password Entry Area |  | 00000000h | Read/Write | Yes |
| 127 | 7-0 | Page Select Byte |  | 00h | Read/Write | Yes |
| 128 | 7-0 | Identifier | - | 11h | Read-Only | Yes |
| 129 | 7-0 | Ext. Identifier | - | CCh | Read-Only | Yes |
| 130 | 7-0 | Connector Type | - | 07h | Read-Only | Yes |
| 131 | 7-0 | Specification compliance | - | 80h | Read-Only | Yes |
| 132 | 7-0 |  | - | 00h | Read-Only | Yes |
| 133 | 7-0 |  | - | 00h | Read-Only | Yes |
| 134 | 7-0 |  | - | 00h | Read-Only | Yes |
| 135 | 7-0 |  | - | 00h | Read-Only | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 136 | 7-0 | Specification compliance | - | 00h | Read-Only | Yes |
| 137 | 7-0 |  | - | 00h | Read-Only | Yes |
| 138 | 7-0 |  | - | 00h | Read-Only | Yes |
| 139 | 7-0 | Encoding | - | 05h | Read-Only | Yes |
| 140 | 7-0 | Singaling rate, nominal | - | FFh | Read-Only | Yes |
| 141 | 7-0 | Extended Rate Select Compliance | - | 02h | Read-Only | Yes |
| 142 | 7-0 | Length (SMF) | - | 00h | Read-Only | Yes |
| 143 | 7-0 | Length (OM3, 50 um) | - | 23h | Read-Only | Yes |
| 144 | 7-0 | Length (OM2, 50 μm) | - | 00h | Read-Only | Yes |
| 145 | 7-0 | Length (OM1: 62.5 um) or Copper Cable Attenuation | - | 00h | Read-Only | Yes |
| 146 | 7-0 | Length (passive copper or active cable or OM4 50 um) | - | 32h | Read-Only | Yes |
| 147 | 7-0 | Device technology | - | 00h | Read-Only | Yes |
| 148 | 7-0 | Vendor name | C | 43h | Read-Only | Yes |
| 149 | 7-0 |  | C | 43h | Read-Only | Yes |
| 150 | 7-0 |  | L | 4Ch | Read-Only | Yes |
| 151 | 7-0 |  | O | 4Fh | Read-Only | Yes |
| 152 | 7-0 |  | U | 4Fh | Read-Only | Yes |
| 153 | 7-0 |  | D | 55h | Read- Only | Yes |
| 154 | 7-0 |  |  | 44h | Read- Only | Yes |
| 155 | 7-0 |  |  | 20h | Read- Only | Yes |
| 156 | 7-0 |  |  | 20h | Read- Only | Yes |
| 157 | 7-0 |  |  | 20h | Read- Only | Yes |
| 158 | 7-0 |  |  | 20h | Read- Only | Yes |
| 159 | 7-0 |  |  | 20h | Read- Only | Yes |
| 160 | 7-0 |  |  | 20h | Read- Only | Yes |
| 161 | 7-0 |  |  | 20h | Read- Only | Yes |
| 162 | 7-0 |  |  | 20h | Read- Only | Yes |
| 163 | 7-0 |  |  | 20h | Read- Only | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 164 | 7-0 | Extender Module | - | 00h | Read-Only | Yes |
| 165 | 7-0 | Vendor OUI | - | 00h | Read-Only | Yes |
| 166 | 7-0 |  | - | 00h | Read-Only | Yes |
| 167 | 7-0 |  | - | 00h | Read-Only | Yes |
| 168 | 7-0 | Vendor PN | C | 43h | Read-Only | Yes |
| 169 | 7-0 |  | C | 43h | Read-Only | Yes |
| 170 | 7-0 |  | - | 2Dh | Read-Only | Yes |
| 171 | 7-0 |  | Q | 51h | Read-Only | Yes |
| 172 | 7-0 |  | K | 4Bh | Read-Only | Yes |
| 173 | 7-0 |  | K | 4Bh | Read-Only | Yes |
| 174 | 7-0 |  | 1 | 31h | Read-Only | Yes |
| 175 | 7-0 |  | 1 | 31h | Read-Only | Yes |
| 176 | 7-0 |  | 0 | 53h | Read-Only | Yes |
| 177 | 7-0 |  | L | 43h | Read-Only | Yes |
| 178 | 7-0 |  | - | 2Dh | Read-Only | Yes |
| 179 | 7-0 |  | S | 53h | Read-Only | Yes |
| 180 | 7-0 |  | D | 44h | Read-Only | Yes |
| 181 | 7-0 |  |  | 20h | Read-Only | Yes |
| 182 | 7-0 |  |  | 20h | Read-Only | Yes |
| 183 | 7-0 |  |  | 20h | Read-Only | Yes |
| 184 | 7-0 | Vendor Rev | 1 | 31h | Read-Only | Yes |
| 185 | 7-0 |  | 1 | 31h | Read-Only | Yes |
| 186 | 7-0 | Wavelength or Copper Cable Attenuation | - | 42h | Read-Only | Yes |
| 187 | 7-0 |  | - | 68h | Read-Only | Yes |
| 188 | 7-0 | Wavelength tolerance or Copper Cable Attenuation | - | 0Ah | Read-Only | Yes |
| 189 | 7-0 |  | - | F0h | Read-Only | Yes |
| 190 | 7-0 | Max case temp. | - | 46h | Read-Only | Yes |
| 191 | 7-0 | CC_BASE | - | Check Sum | Read-Only | Yes |
| 192 | 7-0 | Link codes | - | 21h | Read-Only | Yes |
| 193 | 7-0 | Options | - | 0Bh | Read-Only | Yes |
| 194 | 7-0 |  | - | 35h | Read-Only | Yes |
| 195 | 7-0 |  | - | BAh | Read-Only | Yes |
| 196-211 | All | Vendor SN | - | 20h | Read-Only | Yes |
| 212-219 | All | Date Code | - | 20h | Read-Only | Yes |
| 220 | 7-0 | Diagnostic Monitoring Type | - | 3Ch | Read-Only | Yes |
| 221 | 7-0 | Enhanced Options | - | 19h | Read-Only | Yes |
| 222 | 7-0 | Baud Rate, nominal | - | 67h | Read-Only | Yes |
| 223 | 7-0 | CC_EXT | - | Check | Read-Only | Yes |
| 224-255 | All | Vendor-Specific | - | 00h | Read-Only |  |

100G QSFP28 BIDI SR Transceiver

| I2C Memory Map (Based on SFF8636 2.10a) Page02 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| Byte | Bit |  |  |  |  |  |
| 128-255 | 7-0 | User EEPROM | 00000000b | 00h | Read/Write | Yes |

| I2C Memory Map (Based on SFF8636 2.10a) Page 03 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| Byte | Bit |  |  |  |  |  |
| 128-129 | All | Temp High Alarm | - | 50h 00h | Read-Only | Yes |
| 130-131 | All | Temp Low Alarm | - | F6h 00h | Read-Only | Yes |
| 132-133 | All | Temp High Warning | - | 46h 00h | Read-Only | Yes |
| 134-135 | All | Temp Low Warning | - | 00h 00h | Read-Only | Yes |
| 136-143 | All | Reserved | - |  | Read-Only |  |
| 144-145 | All | VCC High Alarm | - | 8Dh CCh | Read-Only | Yes |
| 146-147 | All | Vcc Low Alarm | - | 74h 04h | Read-Only | Yes |
| 148-149 | All | VCC High Warning | - | 87h 5Ah | Read-Only | Yes |
| 150-151 | All | Vcc Low Warning | - | 7Ah 76h | Read-Only | Yes |
| 152-159 | All | Reserved | - |  | Read-Only |  |
| 160-175 | All | Vendor-Specific | - |  | Read-Only |  |
| 176-177 | All | Rx Power High Alarm | - | C3h C7h | Read-Only | Yes |
| 178-179 | All | Rx Power Low Alarm | - | 02h F7h | Read-Only | Yes |
| 180-181 | All | Rx Power High Warning | - | 62h 1Fh | Read-Only | Yes |
| 182-183 | All | Rx Power Low Warning | - | 05h EAh | Read-Only | Yes |
| 184-185 | All | Tx Bias High Alarm | - | 17h 70h | Read-Only | Yes |
| 186-187 | All | Tx Bias Low Alarm | - | 00h 00h | Read-Only | Yes |
| 188-189 | All | Tx Bias High Warning | - | 13h 88h | Read-Only | Yes |
| 190-191 | All | Tx Bias Low Warning | - | 00h 00h | Read-Only | Yes |
| 192-193 | All | Tx Power High Alarm | - | C3h C7h | Read-Only | Yes |
| 194-195 | All | Tx Power Low Alarm | - | 07h 1Ch | Read-Only | Yes |
| 196-197 | All | Tx Power High Warning | - | 62h 1Fh | Read-Only | Yes |
| 198-199 | All | Tx Power Low Warning | - | 0Eh 2Fh | Read-Only | Yes |
| 200-207 | All | Reserved | - |  | Read-Only |  |
| 208-215 | All | Reserved | - |  | Read-Only |  |
| 216-223 | All | Vendor-Specific | - |  | Read-Only |  |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 224 | 7-4 | Max Tx Input equalization | 0000b | 0Fh | Read-Only | Yes |
|  | 3-0 | Max Rx output emphasis | 1111b |  | Read-Only | Yes |
| 225 | 7-6 | Reserved | 0b | 0Fh | Read-Only | Yes |
|  | 5-4 | Rx output emphasis | 0b |  | Read-Only | Yes |
|  | 3 | Rx output amplitude support | 1b |  | Read-Only | Yes |
|  | 2 |  | 1b |  | Read-Only | Yes |
|  | 1 |  | 1b |  | Read-Only | Yes |
|  | 0 |  | 1b |  | Read-Only | Yes |
| 226 | All | Reserved | - |  | Read-Only |  |
| 227 | 7 | Controllable Host-Side FEC support | 0b | 08h | Read-Only | Yes |
|  | 6 | Controllable Media-Side FEC support | 0b |  | Read-Only | Yes |
|  | 5-4 | Reserved | 0b |  | Read-Only | Yes |
|  | 3 | Tx Force Squelch Implemented | 1b |  | Read-Only | Yes |
|  | 2 | RXLOSL Fast Mode | 0b |  | Read-Only | Yes |
|  | 1 | TxDis Fast Mode Supported | 0b |  | Read-Only | Yes |
|  | 0 | Reserved | 0b |  | Read-Only | Yes |
| 228 | All | Maximum TC stabilization time | - | 00h | Read-Only | Yes |
| 229 | All | Maximum CTLE settling time | - | 00h | Read-Only | Yes |
| 230 | 7 | Host-Side FEC enables | 0b | 00h | Read/Write | No |
|  | 6 | Media-Side FEC enables | 0b |  | Read/Write | No |
|  | 5-0 | Reserved |  |  |  | No |
| 231 | 7-4 | Reserved |  | 00h |  |  |
|  | 3 | Tx4 Force Squelch | 0b |  | Read/Write | Yes |
|  | 2 | Tx3 Force Squelch | 0b |  | Read/Write | Yes |
|  | 1 | Tx2 Force Squelch | 0b |  | Read/Write | Yes |
|  | 0 | Tx1 Force Squelch | 0b |  | Read/Write | Yes |
| 232 | 7-0 | Reserved |  |  |  |  |
| 233 | 7-4 | Reserved |  | 00h |  | No |
|  | 3 | Tx1AEFreeze | 0b |  | Read/Write | No |
|  | 2 | Tx2AEFreeze | 0b |  | Read/Write | No |
|  | 1 | Tx3AEFreeze | 0b |  | Read/Write | No |
|  | 0 | Tx4AEFreeze | 0b |  | Read/Write | No |
| 234 | 7-4 | Tx1 input equalizer control |  | 00h | Read/Write | No |
|  | 3-0 | Tx2 input equalizer control |  |  | Read/Write | No |
| 235 | 7-6 | Tx3 input equalizer control |  | 00h | Read/Write | No |
|  | 5-4 | Tx4 input equalizer control |  |  | Read/Write | No |
| 236 | 7-4 | Rx1 output emphasis control |  | 00h | Read/Write | Yes |
|  | 3-0 | Rx2 output emphasis control |  |  | Read/Write | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 237 | 7-6 | Rx3 output emphasis control |  | 00h | Read/Write | Yes |
|  | 5-4 | Rx4 output emphasis control |  |  | Read/Write | Yes |
| 238 | 7-4 | Rx1 output amplitude control |  | 00h | Read/Write | Yes |
|  | 3-0 | Rx2 output amplitude control |  |  | Read/Write | Yes |
| 239 | 7-6 | Rx3 output amplitude control |  | 00h | Read/Write | Yes |
|  | 5-4 | Rx4 output amplitude control |  |  | Read/Write | Yes |
| 240 | 7 | Rx4 SQ Disable (squelch disable) |  | 00h | Read/Write | Yes |
|  | 6 | Rx3 SQ Disable (squelch disable) |  |  | Read/Write | Yes |
|  | 5 | Rx2 SQ Disable (squelch disable) |  |  | Read/Write | Yes |
|  | 4 | Rx1 SQ Disable (squelch disable) |  |  | Read/Write | Yes |
|  | 3 | Tx4 SQ Disable (squelch disable) |  |  | Read/Write | No |
|  | 2 | Tx3 SQ Disable (squelch disable) |  |  | Read/Write | No |
|  | 1 | Tx2 SQ Disable (squelch disable) |  |  | Read/Write | No |
|  | 0 | Tx1 SQ Disable (squelch disable) |  |  | Read/Write | No |
| 241 | 7 | Rx4 Output Disable |  | 0Fh | Read/Write | Yes |
|  | 6 | Rx3 Output Disable |  |  | Read/Write | Yes |
|  | 5 | Rx2 Output Disable |  |  | Read/Write | Yes |
|  | 4 | Rx1 Output Disable |  |  | Read/Write | Yes |
|  | 3 | Tx4 adaptive equalization control | 1b |  | Read/Write | No |
|  | 2 | Tx3 adaptive equalization control | 1b |  | Read/Write | No |
|  | 1 | Tx2 adaptive equalization control | 1b |  | Read/Write | No |
|  | 0 | Tx1 adaptive equalization control | 1b |  | Read/Write | No |
| 242 | 7 | M-Rx1 Power High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Rx1 Power Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Rx1 Power High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Rx1 Power Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Rx2 Power High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Rx2 Power Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Rx2 Power High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Rx2 Power Low Warning |  |  | Read/Write | Yes |
| 243 | 7 | M-Rx3 Power High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Rx3 Power Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Rx3 Power High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Rx3 Power Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Rx4 Power High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Rx4 Power Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Rx4 Power High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Rx4 Power Low Warning |  |  | Read/Write | Yes |

100G QSFP28 BIDI SR Transceiver

| Address |  | Name | Bits Value | Byte Value | Read-Only /Read/Write | Support |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit |  |  |  |  |  |
| 244 | 7 | M-Tx1 Bias High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx1 Bias Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Tx1 Bias High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Tx1 Bias Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Tx2 Bias High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Tx2 Bias Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Tx2 Bias High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Tx2 Bias Low Warning |  |  | Read/Write | Yes |
| 245 | 7 | M-Tx3 Bias High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx3 Bias Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Tx3 Bias High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Tx3 Bias Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Tx4 Bias High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Tx4 Bias Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Tx4 Bias High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Tx4 Bias Low Warning |  |  | Read/Write | Yes |
| 246 | 7 | M-Tx1 Power High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx1 Power Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Tx1 Power High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Tx1 Power Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Tx2 Power High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Tx2 Power Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Tx2 Power High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Tx2 Power Low Warning |  |  | Read/Write | Yes |
| 247 | 7 | M-Tx3 Power High Alarm |  | 00h | Read/Write | Yes |
|  | 6 | M-Tx3 Power Low Alarm |  |  | Read/Write | Yes |
|  | 5 | M-Tx3 Power High Warning |  |  | Read/Write | Yes |
|  | 4 | M-Tx3 Power Low Warning |  |  | Read/Write | Yes |
|  | 3 | M-Tx4 Power High Alarm |  |  | Read/Write | Yes |
|  | 2 | M-Tx4 Power Low Alarm |  |  | Read/Write | Yes |
|  | 1 | M-Tx4 Power High Warning |  |  | Read/Write | Yes |
|  | 0 | M-Tx4 Power Low Warning |  |  | Read/Write | Yes |
| 248-249 | All | Reserved |  |  |  |  |
| 250-251 | All | Reserved |  |  |  |  |
| 252-255 | All | Reserved |  |  |  |  |

100G QSFP28 BIDI SR Transceiver

Pin A

Block Diagram

Optical interface instruction

100G QSFP28 BIDI SR Transceiver

Pin Description

| Pin | Symbol | Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground |  |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 | GND | Ground |  |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 | GND | Ground |  |
| 8 | Model | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | V Rx | Receiver +3.3V DC Power Supply |  |
| 11 | SCL | I2C Serial Clock |  |
| 12 | SDA | I2C Serial Data |  |
| 13 | GND | Ground |  |
| 14 | Rx3p | Receiver Non-Inverted Differential Output |  |
| 15 | Rx3n | Receiver Inverted Differential Output |  |
| 16 | GND | Ground |  |
| 17 | Rx1p | Receiver Non-Inverted Differential Output |  |
| 18 | Rx1n | Receiver Inverted Differential Output |  |
| 19 | GND | Ground |  |
| 20 | GND | Ground |  |
| 21 | Rx2n | Receiver Inverted Differential Output |  |
| 22 | Rx2p | Receiver Non-Inverted Differential Output |  |
| 23 | GND | Ground |  |
| 24 | Rx4n | Receiver Inverted Differential Output |  |
| 25 | Rx4p | Receiver Non-Inverted Differential Output |  |
| 26 | GND | Ground |  |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | VCC Tx | Transmitter +3.3V DC Power Supply |  |
| 30 | VCC1 | +3.3V DC Power Supply |  |
| 31 | LPMod | Low Power Mode |  |
| 32 | GND | Ground |  |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground |  |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground |  |

100G QSFP28 BIDI SR Transceiver

Outline Dimensions

Digital Diagnostic Monitor Accuracy

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |

Appendix A Document Revision

| Version No. | Date | Description |
| --- | --- | --- |
| V1.0 | 2024-01-25 | First released |
| V1.1 | 2024-02-21 | Added: The mode Select description (Page 3-16) |
| V1.2 | 2025-03-13 | Update Part Number |