Product Specification

200G QSFP56 SR4 DSP Transceivers

The  200G QSFP56 SR4 is a 4x 53.125Gbps multi mode fiber, hot pluggable optical transceiver. The module integrates four parallel lanes with baud rate at 26.5625GBd each lane. It can transmit up to 70m on fiber OM3 fiber or 100m on OM4 fiber with FEC.

FEATURES

- Compliant with 200G-SR4 optical specifications

- 4x53.125Gb/s electrical interface (200GAUI-4)

- Reach up to 70m on MMF(OM3)

- Reach up to 100m on MMF(OM4)

- Single +3.3V power supply

- Case temperature range: 0 ~ +70℃

- Maximum power consumption 4.5W

- Option：Single MPO12 connector APC / UPC

- RoHS complaint

- Hot pluggable

- DSP

APPLICATIONS

- 200G BASE-SR4 Ethernet links

- Data centers

Standards

- IEEE 802.3cd

- SFF 8679

- Option：SFF 8636 / CMIS 5.0

ABSOLUTE MAXIMUM RATINGS

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | Vcc | -0.3 | 3.6 | V |
| Input Voltage | Vin | -0.3 | Vcc+0.3 | V |
| Storage Temperature | Tst | -40 | 85 | ºC |
| Case Operating Temperature | Top | 0 | 70 | ºC |
| Humidity(non-condensing) | RH | 10 | 85 | % |
| Receiver Damage Threshold, each lane | Pin | 5 |  | dBm |
| Maximum Power Consumption | Pmax |  | 4.5 | W |

RECOMMENDED OPERATING CONDITIONS

| Parameter | Symbol | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Case Operating Temperature | Tcase | 0 | - | 70 | ºC |
| Power Supply Voltage | VCC | 3.135 | 3.3 | 3.465 | V |

Transmitter characteristics

| Parameter | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- |
| Signaling Rate, each lane (range) | 26.5625±100ppm |  |  | GBd |
| Center Wavelength Range | 840 |  | 860 | nm |
| Modulation Format | PAM4 |  |  |  |
| RMS spectral width |  |  | 0.6 | nm |
| Average launch power, each lane | -6.5 |  | 4 | dBm |
| Outer Optical Modulation Amplitude (OMAouter), each lane | -4.5 |  | 3 | dBm |
| Launch power in OMAouter minus TDECQ | -5.9 |  |  | dBm |
| Transmitter and dispersion eye closure for PAM4 (TDECQ), each lane |  |  | 4.5 | dB |
| Extinction ratio, each lane | 3 |  |  | dB |
| Optical return loss tolerance |  |  | 12 | dB |

Receiver characteristics

| Parameter | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- |
| Signaling Rate, each lane (range) | 26.5625±100ppm |  |  | GBd |
| Center Wavelength Range | 840 |  | 860 | nm |
| Modulation Format | PAM4 |  |  |  |
| Average receive power, each lane | -8.4 |  | 4 | dBm |
| Receive power, each lane (OMAouter) |  |  | 3 | dBm |
| Receiver reflectance |  |  | -12 | dB |
| Stressed receiver sensitivity (OMAouter), each lane |  |  | -3.4 | dBm |
| Receiver sensitivity (OMAouter), each lane | Max(-6.5, SECQ-7.9) |  |  | dBm |
| Stressed eye closure for PAM4 (SECQ), lane under test |  | 4.5 |  | dB |
| SECQ – 10log10(Ceq) (max), lane under test |  |  | 4.5 | dB |

PIN ASSIGNMENT

PIN DESCRIPTION

| PIN | Logic | Symbol | Name/Description | Note |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground |  |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 |  | GND | Ground |  |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 |  | GND | Ground |  |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver |  |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data |  |
| 13 |  | GND | Ground |  |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground |  |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground |  |
| 20 |  | GND | Ground |  |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground |  |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground |  |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter |  |
| 30 |  | Vcc1 | +3.3 V Power Supply |  |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground |  |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Output |  |
| 35 |  | GND | Ground |  |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output |  |
| 38 |  | GND | Ground |  |

# Electrical Interface

OUTLINE DIMENSIONS

# Unit mm

Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |