Product Specification

200G QSFP56 500m DR4 Transceivers

Features

l QSFP56 MSA compliant

l 4x53.125Gb/s PAM4 electrical interface (200GAUI-4)

l Maximum power consumption 9W

l MPO-12-APC connector

l Up to 500m transmission on single mode fiber

l Operating case temperature: 0℃~70℃

l Single 3.3V power supply

l RoHS-6 compliant

Applications

l 200G Ethernet

l Data Center Applications

Compliance

l QSFP56 MSA Rev2.1

l IEEE 802.3cn

l IEEE 802.3ck

l CMIS Rev5.2

l GR-468-CORE

l RoHS compliance

Description

The transceiver is a high performance, cost effective module for optical data communication applications supporting 200G Ethernet.   The       transceiver is   designed   to operate    in    switch    and    router    applications supporting QSFP56 MSA / 200G DR4 compliant traffic for up to 500m links.

The    transceiver    can    convert    4-channel

53.125Gb/s     electrical     data     to      4-channel

53.125Gb/s  optical  signals.  Similarly,  it  optically converts 4-channel 53.125Gb/s optical signals to 4-channel  electrical  data  output  on  the  receiver side.   It   has   been  designed  to  withstand  the maximum range of external operating conditions including  temperature,  humidity  and  EMI.  The module offers very high functionality and feature integration,   accessible    via   a    two-wire   serial interface.

Absolute Maximum Ratings

Table1-Absolute Maximum Ratings

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature | TSTG | -40 |  | 85 | ºC |  |
| Operating Relative Humidity | RH | 5 |  | 85 | % | Note1 |
| Supply Voltage | VCC | -0.5 | 3.3 | 3.6 | V |  |
| Optical Input Power | PIN |  |  | 5 | dBm |  |

Note:

[ 1] Non-condensing.

Recommended Operating Conditions

Table2-Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Case temperature | Tcase | 0 |  | 70 | ℃ |  |
| Supply Voltage | VCC | 3.135 | 3.3 | 3.465 | V |  |
| Supply Current | ICC |  |  | 2871 | mA |  |
| Module Power Dissipation | P |  |  | 9 | W |  |

Optical ，Electrical Characteristic

# JH-200G-QSFP56-DR4

Tested under recommended operating conditions, unless otherwise noted

Table3-Transmitter Operating Characteristic-Optical ，Electrical

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 26.5625±100ppm |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  |  |  |
| Line wavelengths | λ | 1304.5 | 1311 | 1317.5 | nm |  |
| Average Launch Power, each lane | PAVG | -2.9 |  | 4 | dBm |  |
| Optical Modulation Amplitude (OMA), each lane | OMA | -0.8 |  | 4.2 | dBm |  |
| Extinction Ratio | ER | 3.5 |  |  | dB |  |
| Side-Mode Suppression Ratio | SMSR | 30 |  |  | dB |  |
| Launch power in OMA minus TDECQ, each lane |  | -2.2 |  |  | dBm |  |
| Transmitter and Dispersion Eye Closure for PAM4, each Lane | TDECQ |  |  | 3.4 | dB |  |
| Optical Return Loss Tolerance |  |  |  | 21.4 | dB |  |
| Transmitter Reflectance |  |  |  | -26 | dB |  |
| Average Launch Power of OFF Transmitter, each Lane |  |  |  | -15 | dBm |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  | GBd |  |
| Differential pk-pk input Voltage tolerance | Vpp | 800 |  |  | mV |  |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 | mV | Note1 |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 | % |  |
| Effective return loss |  |  |  | 8.5 | dB |  |
| Differential to Common Mode Input Return Loss |  | IEEE 802.3-ck Equation (120G-2) |  |  | dB |  |
| Module Stressed Input Test |  | IEEE 802.3ck 120G.3.4.2 |  |  |  | Note2 |

Notes:

[ 1] DC common mode voltage generated by the host. Specification includes effects of ground offset voltage.

[2] BER specified in IEEE 802.3ck 120G.1.1.

Table4-Receiver Operating Characteristic-Optical ，Electrical

| Parameter | Symbol | Min | Typical | Max |  | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 26.5625±100ppm |  |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  |  |  |  |
| Damage Threshold, each lane |  | 5 |  |  |  | dBm |  |
| Line wavelengths | λ | 1304.5 | 1311 | 1317.5 |  | nm |  |
| Average receiver power, each lane |  | -5.9 |  | 4 |  | dBm |  |
| Receiver power, each lane (OMA) |  |  |  | 4.2 |  | dBm |  |
| Receiver Sensitivity (OMAouter) , each lane |  |  |  | -4.4 |  | dBm |  |
| Stressed receiver sensitivity (OMAouter), each laned (max) |  |  |  | -1.9 |  | dBm |  |
| Receiver reflectance |  |  |  | -26 |  | dB |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  |  | GBd |  |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 |  | % |  |
| Differential output Voltage pk-pk | Vpp |  |  | 845 |  | mV |  |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 |  | mV | Note 1 |
| Effective return loss |  | 8.5 |  |  |  | dB |  |
| Common to differential mode conversion return loss |  | IEEE 802.3-CK Equation (120G-1) |  |  |  | dB |  |

Note:

[ 1] DC common mode voltage generated by the host. Specification includes effects of ground offset voltage.

Digital Diagnostic Functions and Control and Status I/O Timing Characteristics

# Table5- Digital Diagnostic Functions

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature monitor absolute error | DMI_Temp | -3 |  | 3 | ℃ | Note1 |
| Supply voltage monitor absolute error | DMI_Vcc | -3% |  | 3% | V | Note2 |
| Bias current monitor absolute error | DMI_Ibias | -10% |  | 10% | mA |  |
| Laser power monitor absolute error | DMI_Tx | -3 |  | 3 | dB |  |
| RX power monitor absolute error | DMI_Rx | -3 |  | 3 | dB |  |

Notes:

[ 1] Temperature here is depending on module case around Max power dissipation. Temperature monitor is done over operating temperature.

[2] Supply voltage monitor is done over operating voltage.

# Table6- Control and Status I/O Timing Characteristics

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MgmtInitDuration | Max MgmtInit Duration |  |  | 2000 | ms | Note1 |
| ResetL Assert Time | t_reset_init | 10 |  |  | µs | Note2 |
| IntL Assert Time | ton_IntL |  |  | 200 | ms | Note3 |
| IntL Deassert Time | toff_IntL |  |  | 500 | µs | Note4 |
| Rx LOS Assert Time | ton_los |  |  | 100 | ms | Note5 |
| Flag Assert Time | ton_flag |  |  | 200 | ms | Note6 |
| Mask Assert Time | ton_mask |  |  | 100 | ms | Note7 |
| Mask Deassert Time | toff_mask |  |  | 100 | ms | Note8 |

Notes:

[ 1] Time from power on, hot plug or rising edge of reset until completion of the MgmtInit State.

[2] Minimum pulse time on the ResetL signal to initiate a module reset.

[3] Time from occurrence of condition triggering IntL until Vout:IntL=Vol.

[4] Time from clear on read operation of associated flag until Vout:IntL=Voh.This includes deassert times for Rx LOS, Tx Fault and other flag bits.

[5] Time from Rx LOS state to Rx LOS bit set (value = 1b) and IntL asserted.

[6] Time from occurrence of condition triggering flag to associated flag bit set (value= 1b) and IntL asserted.

[7] Time from mask bit set (value= 1b) until associated IntL assertion is inhibited.

[8] Time from mask bit cleared (value=0b) until associated IntL operation resumes.

# Table7- Surge Current Requirements

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| VCC |  |  | 3.3 |  | V |  |
| Vcc set point accuracy |  | -5 |  | 5 | % |  |
| Power Supply Noise including ripple |  |  |  | 50 | mV |  |
| Module Maximum Current Inrush with LPMode Pin asserted |  |  |  | 0.55 | A |  |
| Module Maximum Current Inrush with LPMode Pin deasserted |  |  |  | 1.3 | A |  |
| Module Current Ramp Rate |  |  |  | 100 | mA/us |  |
| High Power Mode Power Class 8 module |  |  |  |  |  |  |
| Power Consumption | P_8 |  |  | 9 | W |  |
| Instantaneous peak current | Icc_ip_8 | - | - | 3600 | mA |  |
| Sustained peak current | Icc_sp_8 | - | - | 2970 | mA |  |
| Steady state current | Icc_8 |  |  | 2871 | mA | Note1 |

Notes:

[ 1] The module must stay within its declared power class.

Figure1

Pin-out Definitions

# Figure2

# Table8-Pin Function Definitions

| Pin | Logic | Symbol | Description | Plug Sequence | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1B | Note1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 3B |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input | 3B |  |
| 4 |  | GND | Ground | 1B | Note1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input | 3B |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input | 3B |  |
| 7 |  | GND | Ground | 1B | Note1 |
| 8 | LVTTL-I | ModSelL | Module Select | 3B |  |
| 9 | LVTTL-I | ResetL | Module Reset | 3B |  |
| 10 |  | Vcc Rx | +3.3V Power Supply Receiver | 2B | Note2 |
| 11 | LVCMOS-I/O | SCL | 2-wire serial interface clock | 3B |  |
| 12 | LVCMOS-I/O | SDA | 2-wire serial interface data | 3B |  |
| 13 |  | GND | Ground | 1B | Note1 |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output | 3B |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output | 3B |  |
| 16 |  | GND | Ground | 1B | Note1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output | 3B |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 3B |  |
| 19 |  | GND | Ground | 1B | Note1 |
| 20 |  | GND | Ground | 1B | Note1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 3B |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output | 3B |  |
| 23 |  | GND | Ground | 1B | Note1 |
| 24 | CML-O | Rx4n | Receiver Non-Inverted Data Output | 3B |  |
| 25 | CML-O | Rx4p | Receiver Inverted Data Output | 3B |  |
| 26 |  | GND | Ground | 1B | Note1 |
| 27 | LVTTL-I | ModPrsL | Module Present | 3B |  |
| 28 | LVTTL-I | IntL/RxL OS | Interrupt/optional RxLOS | 3B |  |
| 29 |  | VccTx | +3.3V Power supply transmitter | 2B | Note2 |
| 30 |  | Vcc1 | +3.3V Power supply | 2B | Note2 |
| 31 | LVTTL-I | LPMode/ TxDis | Low Power Mode/optional TX Disable | 3B |  |
| 32 |  | GND | Ground | 1B | Note1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input | 3B |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input | 3B |  |
| 35 |  | GND | Ground | 1B | Note1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input | 3B |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input | 3B |  |
| 38 |  | GND | Ground | 1B | Note1 |

Notes:

[ 1] GND is the symbol for signal and supply (power) common for the QSFP56 module. All are common within the QSFP56 module and all voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane

[2] Vcc Rx, Vcc1 and Vcc Tx are the receiver and transmitter power supplies and shall be applied concurrently.

Requirements, defined for the host side of the Host Edge Card Connector, are listed in Table 4. Vcc Rx, Vcc1and Vcc Tx may be internally connected within the QSFP56 module in any combination. The connector pins are each rated for a maximum current of 1.5A (max. current of 2.0 A is required for high module power of 15-20W)

Block Diagram of Transceiver

# Figure3

<Transmitter Section>: The 4-channel  53.125Gb/s electrical data to 4-channel 1310nm 53.125Gb/s optical signals for 200Gb/s optical transmission.

<Receiver Section>: Similarly, it optically converts 4-channel  1310nm  53.125Gb/s optical signals to 4-channel electrical data output on the receiver side.

Recommended Interface Circuit

Figure4

Dimensions

Figure5

Notes:

[ 1] Flatness: 0.05mm for the heat dissipation area and 0.075mm for the rest.

[2] Roughness: 0.8μm for the heat dissipation area and 1.6μm for the rest.

Figure7 Temperature measuring poin

Ordering Information

Table9- Ordering Information

| Part No. | Specification |  |  |  |  |  |  |  |  | Application |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pack | Soft | Tx | Pout | Rx | Sen | Top | Reach | Others |  |
| JH-200G-QSFP56-DR4 | QSFP56 | CMIS Res5.2 | 1311nm | -2.9 ~ 4dBm | PIN | <-4.4 dBm | 0~70 ℃ | 500m | MPO12-APC | 200G DR4 |