Product Specification

200G QSFP56 2km FR4 Transceivers

Features

- Up to 2km transmission on single mode fiber with FEC

- 4x53.125G PAM4 electrical interface (200GAUI-4)

- Supports 212.5Gb/s aggregate bit rate

- Maximum power consumption 6.5W

- 4 CWDM lanes MUX/DEMUX design

- Non-hermetic package design

- Operating case temperature: 0℃~70℃

- Single +3.3V power supply

- I2C digital management interface

- LC duplex connector

Applications

- Data Center Interconnect

Compliance

- IEEE 802.3-2018

- QSFP-DD-Hardware MSA

- SFF-8661

- SFF-8679

- CMIS Rev4.0

- RoHS compliance

- GR-468-CORE

Description

The JH-200G-QSFP56-FR4 transceiver is a high performance, cost effective module for optical data communication applications supporting 200G. And its transmission distance up to 2km on SM fiber. The transceiver consists of two sections. The transmitter section incorporates a quad channel EA-DFB Laser and a DSP. The receiver section consists of a quad channel PIN photodiode, a quad channel trans-impedance preamplifier (TIA) and a DSP shared with the transmitter.

The JH-200G-QSFP56-FR4 can convert 4-channel 53.125Gb/s electrical data to 4-channel optical signals, and multiplex them into a single channel for 212.5Gb/s optical transmission. Similarly, they optically de-multiplexes a 212.5Gb/s input into 4-channel signals, and convert them to 4-channel output electrical data on the receiver side. It has been designed to meet the harshest external operating conditions including temperature, humidity and EMI. The module offers very high functionality and feature integration, accessible via a two-wire serial interface.

Absolute Maximum Ratings

Table1-Absolute Maximum Ratings

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature | TSTG | -40 |  | 85 | ºC |  |
| Operating Relative Humidity | RH | 5 |  | 85 | % | Note1 |
| Supply Voltage | VCC | -0.3 | 3.3 | 3.6 | V |  |

Note:

[1] Non-condensing.

Recommended Operating Conditions

Table2-Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Case temperature | Tcase | 0 |  | 70 | ℃ |  |
| Supply Voltage | VCC | 3.135 | 3.3 | 3.465 | V |  |
| Supply Current | ICC |  |  | 2.073 | A |  |
| Module Power Dissipation | P |  |  | 6.5 | W |  |
| Link Distance with G.652 | L |  |  | 2 | km |  |

Optical，Electrical Characteristic

JH-200G-QSFP56-FR4

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 26.5625±100ppm |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  | - |  |
| Line wavelengths | λ1 | 1264.5 | 1271 | 1277.5 | nm |  |
|  | λ2 | 1284.5 | 1291 | 1297.5 | nm |  |
|  | λ3 | 1304.5 | 1311 | 1317.5 | nm |  |
|  | λ4 | 1324.5 | 1331 | 1337.5 | nm |  |
| Average Launch Power, each lane | PAVG | -4.2 |  | 4.7 | dBm |  |
| Total Average launch power | PTOTAL |  |  | 10.7 | dBm |  |
| Optical Modulation Amplitude (OMA), each lane | OMA | -1.2 |  | 4.5 | dBm |  |
| Difference in launch Power between any Two Lanes(OMAouter) |  |  |  | 4 | dB |  |

Tested under recommended operating conditions, unless otherwise noted Table3-Transmitter Operating Characteristic-Optical，Electrical

| Average Launch Power of OFF Transmitter, each Lane |  |  |  | -30 | dBm |  |
| --- | --- | --- | --- | --- | --- | --- |
| Extinction Ratio | ER | 3.5 |  |  | dB |  |
| Side-Mode Suppression Ratio | SMSR | 30 |  |  | dB |  |
| Launch power in OMA minus TDECQ, each lane ER≥4.5dB ER＜4.5dB |  | -2.6 -2.5 |  |  | dBm |  |
| Transmitter and Dispersion Eye Closure for PAM4, each Lane | TDECQ |  |  | 3.3 | dB |  |
| Optical Return Loss Tolerance |  |  |  | 16.5 | dB |  |
| Transmitter Reflectance |  |  |  | -26 | dB |  |
| Relative intensity Noise(OMA) |  |  |  | -132 | dB/Hz |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  | GBd |  |
| Differential pk-pk input Voltage tolerance | Vpp | 900 |  |  | mV | Note1 |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 | mV | Note2 |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 | % |  |
| Module Stressed Input Test |  | IEEE 802.3bs 120E.3.4.1 |  |  |  | Note3 |

Notes:

- With the exception to IEEE 802.3bs 120E.3.1.2 that the pattern is PRBS31Q or scrambled idle.

- DC common mode voltage generated by the host. Specification includes effects of ground offset voltage.

- BER specified in IEEE 802.3bs 120E.1.1.

Table4-Receiver Operating Characteristic-Optical，Electrical

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 53.125±100ppm |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  |  |  |
| Damage Threshold, each lane |  | 5.7 |  |  | dBm |  |
| Line wavelengths | λ1 | 1264.5 | 1271 | 1277.5 | nm |  |
|  | λ2 | 1284.5 | 1291 | 1297.5 | nm |  |
|  | λ3 | 1304.5 | 1311 | 1317.5 | nm |  |
|  | λ4 | 1324.5 | 1331 | 1337.5 | nm |  |
| Average receiver power, each lane |  | -8.2 |  | 4.7 | dBm |  |

| Receiver power, each lane (OMA) |  |  |  | 4.5 |  | dBm |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Difference in receive Power between any Two Lanes(OMAouter) |  |  |  | 4.1 |  | dBm |  |
| Receiver Sensitivity (OMAouter) , each lane |  |  |  | -6 |  | dBm |  |
| Stressed receiver sensitivity (OMAouter), each laned (max) |  |  |  | -3.6 |  | dBm |  |
| Receiver reflectance |  |  |  | -26 |  | dB |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  |  | GBd |  |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 |  | % |  |
| Differential output Voltage pk-pk | Vpp |  |  | 900 |  | mV |  |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 |  | mV |  |
| Near-end Eye Width at 10E-6 probability | EW6 | 0.265 |  |  |  | UI |  |
| Near-end Eye Height at 10E-6 probability | EH6 | 70 |  |  |  | mV |  |
| Far-end Eye Width at 10E-6 probability | EW6 | 0.2 |  |  |  | UI |  |
| Far-end Eye Height at 10E-6 probability | EH6 | 30 |  |  |  | mV |  |
| Transition time | Trise/Tfall | 9.5 |  |  |  | ps | Note1 |

Note:

[1] 20%~80%.

Digital Diagnostic Functions and Control and Status I/O Timing Characteristics

Table5- Digital Diagnostic Functions

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature monitor absolute error | DMI_Temp | -3 |  | 3 | ℃ | Note1 |
| Supply voltage monitor absolute error | DMI_Vcc | -3% |  | 3% | V | Note2 |
| Bias current monitor absolute error | DMI_Ibias | -10% |  | 10% | mA |  |
| Laser power monitor absolute error | DMI_Tx | -3 |  | 3 | dB |  |
| RX power monitor absolute error | DMI_Rx | -3 |  | 3 | dB |  |

Notes:

- Temperature here is depending on module case around max power dissipation. Temperature monitor is done over operating temperature.

- Supply voltage monitor is done over operating voltage.

Table6- Control and Status I/O Timing Characteristics

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MgmtInitDuration | Max MgmtInit Duration |  |  | 2000 | ms | Note1 |
| ResetL Assert Time | t_reset_init | 10 |  |  | µs | Note2 |
| IntL Assert Time | ton_IntL |  |  | 200 | ms | Note3 |
| IntL Deassert Time | toff_IntL |  |  | 500 | µs | Note4 |
| Rx LOS Assert Time | ton_los |  |  | 100 | ms | Note5 |
| Tx Fault Assert Time | ton_Txfault |  |  | 200 | ms | Note6 |
| Flag Assert Time | ton_flag |  |  | 200 | ms | Note7 |
| Mask Assert Time | ton_mask |  |  | 100 | ms | Note8 |
| Mask Deassert Time | toff_mask |  |  | 100 | ms | Note9 |

Notes:

- Time from power on, hot plug or rising edge of reset until completion of the MgmtInit State.

- Minimum pulse time on the ResetL signal to initiate a module reset.

- Time from occurrence of condition triggering IntL until Vout:IntL=Vol.

- Time from clear on read operation of associated flag until Vout:IntL=Voh.This includes deassert times for Rx LOS, Tx Fault and other flag bits.

- Time from Rx LOS state to Rx LOS bit set (value = 1b) and IntL asserted.

- Time from Tx Fault state to Tx Fault bit set (value=1b) and IntL asserted.

- Time from occurrence of condition triggering flag to associated flag bit set (value=1b) and IntL asserted.

- Time from mask bit set (value=1b) until associated IntL assertion is inhibited.

- Time from mask bit cleared (value=0b) until associated IntL operation resumes.

Table7- Surge Current Requirements

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Power supply voltages VccTx, Vcc1 & VccRx including ripple, droop and noise below 100kHz |  | 3.135 | 3.3 | 3.465 | V | Note1 |
| Module RMS noise output 10 Hz - 10 MHz |  |  |  | 30 | mV |  |
| Module power supply noise tolerance 10 Hz - 10 MHz (peak-to-peak) | PSNRmod |  |  | 66 | mV |  |
| Module inrush - instantaneous peak duration | T_ip |  |  | 50 | µs | Note2 |
| Module inrush - initialization time | T_init |  |  | 500 | ms | Note2 |
| High Power Mode Power Class 3 module |  |  |  |  |  |  |
| Power Consumption | P_3 |  |  | 7 | W | Note3 |
| Instantaneous peak current | Icc_ip_3 | - | - | 2800 | mA |  |
| Sustained peak current | Icc_sp_3 | - | - | 2310 | mA |  |
| Steady state current | Icc_3 |  |  | 2232.8 | mA | Note4 |

Notes:

- Measured at VccTx, Vcc1, VccRx.

- T_ip and T_init are test conditions for measuring inrush current and not characteristics of the module.

- User must read management register for maximum power consumption.

- The module must stay within its declared power class.

Pin-out Definitions

Figure1

Table8-Pin Function Definitions

| Pin | Name | Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground | Note1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 | GND | Ground | Note1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 | GND | Ground |  |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | Vcc Rx | +3.3V Power Supply Receiver | Note2 |
| 11 | SCL | 2-wire serial interface clock |  |
| 12 | SDA | 2-wire serial interface data |  |
| 13 | GND | Ground |  |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |
| 16 | GND | Ground | Note1 |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output | Note1 |
| 19 | GND | Ground | Note1 |
| 20 | GND | Ground |  |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |

| 23 | GND | Ground |  |
| --- | --- | --- | --- |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Ground | Note1 |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL/RxLOSL | Interrupt. Optionally configurable as RxLOSL via the management interface (SFF-8636). |  |
| 29 | VccTx | +3.3V Power supply transmitter | Note2 |
| 30 | Vcc1 | +3.3V Power supply | Note2 |
| 31 | LPMode/TxDis | Low Power Mode. Optionally configurable as TxDis via the management interface (SFF-8636). |  |
| 32 | GND | Ground | Note1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground | Note1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground | Note1 |

Notes：

- GND is the symbol for signal and supply (power) common for the module. All are common within the module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane.

- VccRx, Vcc1 and VccTx are applied concurrently and may be internally connected within the module in any combination.

Block Diagram of Transceiver

Figure2

<Transmitter Section>: The JH-200G-QSFP56-FR4 converts 4-channel 53.125Gb/s electrical data to 4-channel 53.125Gb/s optical signals multiplexed for 212Gb/s optical transmission.

<Receiver Section>: Similarly, it optically de-multiplexes a 212.5 Gb/s input into 4-channel signals, and converts them to 4-channel electrical output data on the receiver side.

Recommended Interface Circuit

Figure3

Dimensions

Unit: mm

Figure4

Digital Diagnostic Memory Map

Ordering Information

Table10- Ordering Information

| Part No | Specification |  |  |  |  |  |  |  |  | Application |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pack | Rate | Tx | Pout（per lane） | Rx | Sen（OMA） | Top | Reach | Others |  |
| JH-200G-QSFP56-FR4 | QSFP56 | 200G | CWDM EA-DFB | -4.2 ~ 4.7dBm | PIN | < -6dBm | 0~70℃ | 2Km | DDM/R oHS | 200G FR4 |