400G DR4 OSFP

Features

l OSFP Hot-pluggable form factor

l Maximum link length of 500m on SMF with FEC

l Single 3.3V power supply

l Power dissipation 8W

l Support DDMI

l Operating case temp: 0°C to +70 °C

l Single APC MPO-12 connector

l Compliant with CMIS 5.2

l Anti-sulphuration Compliant

l RoHS 6 Compliant

Applications

l 400GBASE-DR4

l 200GBASE-DR2

l 100GBASE-DR1

Order Information

Table 1-Order Information

| Part No. | Bit Rate (Gbps) | Laser (nm) | Distance note1 | Fiber Type | DDMI | Connector | Tempnote2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JH-400G-OSFP-DR4 | 425 | 1311 | 500m | SMF | YES | APC MPO12 | 0℃~+70℃ |

Note:

1.      SMF with FEC.

2.      Case Temperature.

Absolute Maximum Ratings

Table2- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | -0.5 | - | +3.6 | V |  |
| Storage Temperature | Ts | -40 | - | +85 | °C |  |
| Operating Case Temperature | TC | -5 | - | +75 | °C |  |
| Operating Humidity | RH | 5 | - | 85 | % | 1 |
| Control Input Voltage | Vcontrol | -0.3 | - | VCC+0.3 | V | 1 |
| Optical Receiver Damage Threshold, each lane | Rdamage | 5 |  |  | dBm |  |

Note:1 No condensation.

Recommended Operating Conditions

Table 3- Recommended operating Conditions

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |
| Power Supply Voltage | Vcc | 3.135 | 3.3 | 3.465 | V |  |
| Power Dissipation | Pd | - | - | 8 | W |  |
| Operating Humidity | RH | 5 | - | 85 | % | 1 |

Note:1 No condensation.

Electrical Characteristics

Table 4- Electrical Characteristics

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Host to Module Side |  |  |  |  |  |  |
| Data Rate, each lane | DR | 53.125 |  |  | GBd |  |
| Data Rate Variation | - | -100 | - | +100 | ppm |  |
| Modulation Format | - | PAM4 |  |  | - |  |
| Differential Input Peak-peak Voltage Tolerance | Vin-diff | 750 | - | - | mVpp |  |
| Differential Input Termination Mismatch | - | - | - | 10 | % |  |
| Single-ended Voltage Tolerance | - | -0.4 | - | 3.3 | V |  |
| DC Common-mode Voltage Tolerance | - | -0.35 | - | 2.85 | V |  |
| Module to Host Side |  |  |  |  |  |  |
| Data Rate, each lane | DR | 53.125 |  |  | GBd |  |
| Data Rate Variation | - | -100 | - | +100 | ppm |  |
| Modulation Format | - | PAM4 |  |  | - |  |
| Differential Output Peak-to-peak Voltage (Short mode) | Vout-diff,short | - | - | 600 | mVpp |  |
| Differential Output Peak-to-peak Voltage (Long mode) | Vout-diff,Long | - | - | 845 | mVpp |  |
| Transition Time (20% to 80%) | Tr/tf | 8.5 | - | - | ps |  |
| Eye Height | EH | - | - | 15 | mV |  |
| Vertical Eye Closure | VEC | - | - | 12 | dB |  |
| Differential Output Termination Mismatch | - | - | - | 10 | % |  |
| DC Common-mode Voltage Tolerance | - | -0.35 | - | 2.85 | V |  |

Optical Characteristics

Table 5-Optical Characteristics

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Transmitter |  |  |  |  |  |  |

| Data Rate, each lane | DR | 53.125±100 ppm |  |  | GBd |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data Rate Variation | - | -100 | - | +100 | ppm |  |
| Modulation Format | - | PAM4 |  |  | - |  |
| Center Wavelength Lane0 | L0 | 1304.5 | 1311 | 1317.5 | nm |  |
| Center Wavelength Lane1 | L1 | 1304.5 | 1311 | 1317.5 | nm |  |
| Center Wavelength Lane2 | L2 | 1304.5 | 1311 | 1317.5 | nm |  |
| Center Wavelength Lane3 | L3 | 1304.5 | 1311 | 1317.5 | nm |  |
| Single-mode Suppression Ration , each lane | SMSR | 30 |  |  | dB |  |
| Average Launch Power, each lane | PTxAOP | -2.9 | - | 4 | dBm | 1 |
| Outer Optical Modulation Amplitude, each lane | TxOMA | -0.8 | - | 4.2 | dBm | 2 |
| Extinction Ration, each lane | ER | 3.5 | 4.5 | - | dB |  |
| Launch Power in OMA minus TDECQ, each lane | - | -2.2 | - | - | dBm |  |
| Transmitter and Dispersion Eye Closure for PAM4, each lane | TDECQ | - | - | 3.4 | dB |  |
| RIN OMA | RIN | - | - | -136 | dB/ Hz |  |
| Optical Return Loss Tolerance | ORL | - | - | 21.4 | dB |  |
| Transmitter Reflectance | - | - | - | -26 | dB |  |
| Average Launch Power of OFF Transmitter, each lane | - | - | - | -15 | dBm |  |
| Optical Receiver |  |  |  |  |  |  |
| Data Rate, each lane | DR | 53.125±100 ppm |  |  | GBd |  |
| Data Rate Variation | - | -100 | - | +100 | ppm |  |
| Modulation Format | - | PAM4 |  |  | - |  |
| Receiver Wavelength Range Lane0 | L0 | 1304.5 | 1311 | 1317.5 | nm |  |
| Receiver Wavelength Range Lane1 | L1 | 1304.5 | 1311 | 1317.5 | nm |  |
| Receiver Wavelength Range Lane2 | L2 | 1304.5 | 1311 | 1317.5 | nm |  |
| Receiver Wavelength Range Lane3 | L3 | 1304.5 | 1311 | 1317.5 | nm |  |
| Average Receiver Power, each lane | PRxAOP | -5.9 | - | 4 | dBm |  |
| Receive Power(OMAouter), each lane | RxOMA | - | - | 4.2 | dBm |  |
| Receiver Reflectance | - | - | - | -26 | dB |  |
| Receiver Sensitivity(OMAouter), each lane | OMASen | - | - | -4.5 | dBm | 3 |
| Stressed Receiver Sensitivity(OMAouter), each lane | - | - | - | -1.9 | dBm |  |
| Stressed Eye Closure fo PAM4(SECQ),lane under test | - | - | - | 3.4 | dB |  |
| OMAouter of each Aggressor Lane | - | - | - | 4.2 | dBm |  |
| Los Assert | LOSAssert | -14 | - | -11.5 | dBm |  |

| Los De-assert | LOSDe-assert | -11 | - | -8 | dBm |  |
| --- | --- | --- | --- | --- | --- | --- |
| LOS Hysteresis | LOSHys | 0.5 | - | 4 | dB |  |

Note:

1.    Minimum value is informative only and not the principal indicator of signal strength.

2.    Transmitter reflectance is defined looking into the transmitter.

3.    @BER 1E-6.

Recommended Interface

Figure 1    Recommended Interface Circuit

Pin Arrangement

Figure 2    Pin Arrangement

Table 6-Pin Function Definitions

| Pin | Symbol | Name/Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground | 1 |
| 2 | TX2p | Transmitter Data Non-Inverted |  |
| 3 | TX2n | Transmitter Data Inverted |  |
| 4 | GND | Ground | 1 |
| 5 | TX4p | Transmitter Data Non-Inverted |  |
| 6 | TX4n | Transmitter Data Inverted |  |
| 7 | GND | Ground | 1 |
| 8 | NC | NC,unsued | 2 |
| 9 | NC | NC,unsued | 2 |
| 10 | GND | Ground | 1 |
| 11 | NC | NC,unsued | 2 |
| 12 | NC | NC,unsued | 2 |
| 13 | GND | Ground | 1 |
| 14 | SCL | 2-wire Serial interface clock | 3 |
| 15 | VCC | +3 .3V Power |  |
| 16 | VCC | +3 .3V Power |  |
| 17 | LPWn/PRSn | Low-Power Mode/Module Present | 4 |

| 18 | GND | Ground | 1 |
| --- | --- | --- | --- |
| 19 | NC | NC,unsued | 2 |
| 20 | NC | NC,unsued | 2 |
| 21 | GND | Ground | 1 |
| 22 | NC | NC,unsued | 2 |
| 23 | NC | NC,unsued | 2 |
| 24 | GND | Ground | 1 |
| 25 | RX3n | Receiver Data Inverted |  |
| 26 | RX3p | Receiver Data Non-Inverted |  |
| 27 | GND | Ground | 1 |
| 28 | RX1n | Receiver Data Inverted |  |
| 29 | RX1p | Receiver Data Non-Inverted |  |
| 30 | GND | Ground | 1 |
| 31 | GND | Ground | 1 |
| 32 | RX2p | Receiver Data Non-Inverted |  |
| 33 | RX2n | Receiver Data Inverted |  |
| 34 | GND | Ground | 1 |
| 35 | RX4p | Receiver Data Non-Inverted |  |
| 36 | RX4n | Receiver Data Inverted |  |
| 37 | GND | Ground | 1 |
| 38 | NC | NC,unsued | 2 |
| 39 | NC | NC,unsued | 2 |
| 40 | GND | Ground | 1 |
| 41 | NC | NC,unsued | 2 |
| 42 | NC | NC,unsued | 2 |
| 43 | GND | Ground | 1 |
| 44 | INT/ RSTn | Module Interrupt / Module Reset | 5 |
| 45 | VCC | +3 .3V Power |  |
| 46 | VCC | +3 .3V Power |  |
| 47 | SDA | 2-wire Serial interface data | 3 |
| 48 | GND | Ground | 1 |
| 49 | NC | NC,unsued | 2 |
| 50 | NC | NC,unsued | 2 |
| 51 | GND | Ground | 1 |
| 52 | NC | NC,unsued | 2 |
| 53 | NC | NC,unsued | 2 |
| 54 | GND | Ground | 1 |
| 55 | TX3n | Transmitter Data Inverted |  |
| 56 | TX3p | Transmitter Data Non-Inverted |  |
| 57 | GND | Ground | 1 |
| 58 | TX1n | Transmitter Data Inverted |  |

| 59 | TX1p | Transmitter Data Non-Inverted |  |
| --- | --- | --- | --- |
| 60 | GND | Ground | 1 |

Note 1. Circuit ground is internally isolated from chassis ground. Note 2. 400G DR4 unused PINs, NC.

Note 3. Open-Drain with pull up resistor on Host.

Note 4. See pin description for required circuit as shown in Figure 3.

Note 5. See pin description for required circuit as shown in Figure 4.

Figure 3    LPWn/PRSn circuit and voltage zones

Figure 4    INT/RSTn circuit and voltage zones

Memory Map

Compatible with CMIS rev 5.2.

400G DR4 OSFP

Optical Interface Arrangement

The optical port is a male MPO connector receptacle, with fiber lane assignments as shown in Figure 5.

Figure 5 Optical interface arrangement

Mechanical

400G OSFP DR4 transceivers are compatible with OSFP module specification. (Unit: mm)

Figure 6 Mechanical Diagram

# Warnings

Handling Precautions: This device is susceptible to damage as a result of electrostatic discharge (ESD).

A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety: Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radiation.

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City