Product Specification

400GBASE SR4 QSFP-DD 100m Transceiver

Features

- Hot-pluggable QSFP-DD form factor

- Maximum link length of 100m on OM4 fiber with FEC

	+3.3V single power supply

- Power dissipation < 8W

- Operating case temp Commercial: 0°C to +70 °C

- Single MPO-12 APC connector

- RoHS compliant

- Compliant with IEEE 802.3bs standard: 400GAUI-8 electrical interface

Applications

- 400GBASE-SR4 Ethernet

- Data Center

- Application case 1, 400G SR4, 1 of 400G per port point to point connections

- Application case 2, 2x200G SR2, 2 of 200G per channel breakout connections

- Application case 3, 4x100G SR, 4 of 100G per channel breakout connections

Order Information

Table 1-Order Information

| Part No. | Bit Rate (Gbps) | Laser (nm) | Distance note1 | Fiber Type | DDMI | Connector | Temp note2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JH-400G-QDD-SR4 | 425.0 | 850 | 60m@OM3 100m@OM4 | MMF | YES | MPO 1x12 APC | 0℃~+70℃ |

Note:

- 400 Gigabit Ethernet links over multimode fiber up to 60m on OM3 MMF or 100m on OM4 MMF.

- Case Temperature

Absolute Maximum Ratings

Table2- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc3 | -0.5 | - | +3.6 | V |  |
| Storage Temperature | Ts | -40 | - | +85 | °C |  |
| Operating Humidity | RH | 0 | - | +85 | % | 1 |
| Control Input Voltage | VI | -0.3 | - | VCC+0.5 | V | 1 |

Note:1 No condensation

Recommended Operating Conditions

Table 3- Recommended operating Conditions

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |
| Power Supply Voltage | Vcc | 3.135 | 3.3 | 3.465 | V |  |
| Power Dissipation | Pd | - | - | 8 | W |  |
| Supply Current | Icc | - | - | 2550 | mA |  |
| Pre-FEC Bit Error Ratio | - | - | - | 2.4x10-4 | - |  |
| Post-FEC Bit Error Ratio | - | - | - | 1x10-12 | - | 1 |
| Link Distance (OM4) | - | 2 | - | 100 | m | 2 |
| Link Distance (OM3) | - | 2 | - | 60 | m |  |

Note:

- FEC provided by host system

- FEC required on host system to support maximum distance

Electrical Characteristics

Table 4- Electrical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Differential pk-pk input Voltage tolerance | - | mV | 900 | - | - |  |
| Differential termination mismatch | - | % | - | - | 10 |  |
| Single-ended voltage tolerance range | - | V | -0.4 | - | 3.3 |  |
| DC common mode Voltage | - | mV | -350 | - | 2850 |  |
| Receiver |  |  |  |  |  |  |
| AC common-mode output Voltage (RMS) | - | mV | - | - | 17.5 |  |
| Differential output Voltage | - | mV | - | - | 900 |  |
| Near-end Eye height, differential | - | mV | 70 | - | - |  |
| Far-end Eye height, differential | - | mV | 30 | - | - |  |
| Far end pre-cursor ISI ratio | - | % | -4.5 | - | 2.5 |  |
| Differential Termination Mismatch | - | % | - | - | 10 |  |
| Transition Time (min, 20% to 80%) | - | ps | 9.5 | - | - |  |
| DC common mode Voltage | - | mV | -350 | - | 2850 |  |

Table 5-Optical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling Rate per Lane | SR | Gbd | 53.125 ± 100 ppm |  |  |  |
| Modulation format | - | - | PAM4 |  |  |  |
| Center wavelength | CW | nm | 842 | - | 948 |  |
| RMS Spectral Width | SW | dBm | - | - | 0.65 | 1 |

| Average Launch Power per Lane | AOP | dBm | -4.6 | - | 4.0 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Outer Optical Modulation Amplitude (OMAouter), each lane (min) For max(TECQ,TDECQ)≤1.8dB For 1.8<max(TECQ,TDECQ)≤4.4dB | TxOMA | dBm | -2.6 -4.4+max( TECQ,TDE CQ) | - | 3.5 |  |
| Transmitter and Dispersion Eye Closure for PAM4 (TDECQ), each lane | TDECQ | dB | - | - | 4.4 |  |
| Overshoot/undershoot | - | % | - | - | 29 |  |
| Transimitter power excursion,each lane | - | dBm | - | - | 2.3 |  |
| Transition Time | Tt | ps | - | - | 17 |  |
| Average Launch Power of OFF Transmitter, each lane | TOFF | dBm | - | - | -30 |  |
| Extinction Ratio, each lane | ER | dB | 2.5 | - | - |  |
| Encircled flux | - | dBm | ≥86% at 19μm ≤30% at 4.5μm |  |  | 2 |
| Receiver |  |  |  |  |  |  |
| Signaling Rate per Lane | SR | Gbd | 53.125 ± 100 ppm |  |  |  |
| Modulation format | - | - | PAM4 |  |  |  |
| Wavelength | W | nm | 842 | - | 948 |  |
| Damage Threshold, average optical power, each lane | DT | dBm | 5 | - | - | 3 |
| Average Receive Power, each lane | RXPx | dBm | -6.3 | - | 4 | 4 |
| Receive Power (OMA) per Lane | RxOMA | dBm | - | - | 3.5 |  |
| Receiver sensitivity (OMA outer) (max) for TECQ≤1.8dB for 1.8＜TECQ≤4.4dB | Rx Sen | dBm | - | - | -4.4 -6.2+TE CQ | 5 |
| LOS Assert | LOSA | dBm | -15 | - | - |  |
| LOS De-assert | LOSD | dBm | - | - | -9.2 |  |
| LOS Hysteresis | LOSH | dB | 0.5 | - | - |  |
| OMA outer of each aggressor lane | - | dBm | - | 3.5 | - |  |

Note:

- RMS spectral width is the standard deviation of the spectrum

- Receiver sensitivity (OMAouter) is informative and is defined for a transmitter with a value of TECQ up to

4.4 dB

Recommended Interface

Figure 1, Recommended Interface Circuit

Pin arrangement

Table 7-Pin Function Definitions

Figure 2, Pin View

| Pin # | Logic | Symbol | Definition | Pin # | Logic | Symbol | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 39 |  | GND | Ground |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 40 | CML-I | Tx6n | Transmitter Inverted Data Input |
| 3 | CML-I | Tx2p | Transmitter Non-inverted Data Input | 41 | CML-I | Tx6p | Transmitter Non-inverted Data Input |
| 4 |  | GND | Ground | 42 |  | GND | Ground |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input | 43 | CML-I | Tx8n | Transmitter Inverted Data Input |
| 6 | CML-I | Tx4p | Transmitter Non-inverted Data Input | 44 | CML-I | Tx8p | Transmitter Non-inverted Data Input |
| 7 |  | GND | Ground | 45 |  | GND | Ground |
| 8 | LVTTL-I | ModSelL | Module Select | 46 |  | Reserved |  |
| 9 | LVTTL-I | ResetL | Module Reset | 47 |  | VS1 | Module Vendor Specific 1 |
| 10 |  | VccRx | +3.3V Power Supply Receiver | 48 |  | VccRx1 | 3.3V Power Supply |
| 11 | LVCMOS-I/O | SCL | 2-wire serial interface clock | 49 |  | VS2 | Module Vendor Specific 2 |
| 12 | LVCMOS-I/O | SDA | 2-wire serial interface data | 50 |  | VS3 | Module Vendor Specific 3 |
| 13 |  | GND | Ground | 51 |  | GND | Ground |
| 14 | CML-O | Rx3p | Receiver Non-inverted Data Output | 52 | CML-O | Rx7p | Receiver Non-inverted Data Output |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output | 53 | CML-O | Rx7n | Receiver Inverted Data Output |
| 16 |  | GND | Ground | 54 |  | GND | Ground |
| 17 | CML-O | Rx1p | Receiver Non-inverted Data Output | 55 | CML-O | Rx5p | Receiver Non-inverted Data Output |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 56 | CML-O | Rx5n | Receiver Inverted Data Output |
| 19 |  | GND | Ground | 57 |  | GND | Ground |

| 20 |  | GND | Ground | 58 |  | GND | Ground |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 59 | CML-O | Rx6n | Receiver Inverted Data Output |
| 22 | CML-O | Rx2p | Receiver Non-inverted Data Output | 60 | CML-O | Rx6p | Receiver Non-inverted Data Output |
| 23 |  | GND | Ground | 61 |  | GND | Ground |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output | 62 | CML-O | Rx8n | Receiver Inverted Data Output |
| 25 | CML-O | Rx4p | Receiver Non-inverted Data Output | 63 | CML-O | Rx8p | Receiver Non-inverted Data Output |
| 26 |  | GND | Ground | 64 |  | GND | Ground |
| 27 | LVTTL-O | ModPrsL | Module Present | 65 |  | NC | Not connected |
| 28 | LVTTL-O | IntL | Interrupt | 66 |  | Reserved |  |
| 29 |  | VccTx | +3.3V Power Supply Transmitter | 67 |  | VccTx1 | 3.3V Power Supply |
| 30 |  | Vcc1 | +3.3V Power Supply | 68 |  | Vcc2 | 3.3V Power Supply |
| 31 | LVTTL-I | InitMode | Initialization mode | 69 |  | Reserved |  |
| 32 |  | GND | Ground | 70 |  | GND | Ground |
| 33 | CML-I | Tx3p | Transmitter Non-inverted Data Input | 71 | CML-I | Tx7p | Transmitter Non-inverted Data Input |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input | 72 | CML-I | Tx7n | Transmitter Inverted Data Input |
| 35 |  | GND | Ground | 73 |  | GND | Ground |
| 36 | CML-I | Tx1p | Transmitter Non-inverted Data Input | 74 | CML-I | Tx5p | Transmitter Non-inverted Data Input |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input | 75 | CML-I | Tx5n | Transmitter Inverted Data Input |
| 38 |  | GND | Ground | 76 |  | GND | Ground |

Memory Map

Compatible with CMIS rev 5.2.

Optical interface arrangement

The optical port is a male MPO connector receptacle, with fiber lane assignments as shown in Figure 3

Mechanical

Figure 3, Optical interface arrangement.

400G SR4 QSFP-DD transceivers are compatible with the QSFP-DD Type 2 Specification for pluggable form factor modules.

Unit mm

Warnings

Figure 4, Mechanical Diagram

Handling Precautions: This device is susceptible to damage as a result of electrostatic discharge (ESD).

A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety: Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radiation.