# 400G QSFP-DD SR8 Optical Transceiver

## Features

● 8 independent parallel optical channels

● Each channel data rate up to 26.56GBaud with  CDR

● Hot Pluggable

● Up to 100m link on OM4 Multi-mode

● 850nm VCSEL/PD Array Technology

● CML Compatible electrical I/O

● QSFP-DD MSA  Compliance

● Optical  connectivity  via   industry  standard

MPO/ MTP  terminated fiber ribbon

● MPO-16 APC Optical Receptacle Type

● CMIS 4.0 Compliance

● Monitors for VCSEL  bias,  transmitted,  received power ,  module

● temperature, and module supply

● Case Operating Temperature: Commercial: 0 to 70ºC

● RoHS II Compliance

## Applications

● High performance computing interconnect

● Data center

## Description

JH-400G-QDD-SR8 is a QSFP-DD Optical transceiver for 8 x 26.56GBaud  optical links. It is compliant with the QSFP-DD MSA specifications. It operates at 26.56GBaud  up to 100m over  OM4  Multi-mode fiber.

# 400G QSFP-DD SR8 Optical Transceiver

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- |
| Storage Ambient Temperature | Tstg | -40 | +85 | ºC | Exceeding the Absolute Maximum Ratings may cause irreversible damage to the device. The device is not intended to be operated under the condition of sim- ultaneous Absolute Maximum Ratings, a condition which may cause irreversible damage to the device. RH is Non- condensing condition. |
| Relative Humidity - Storage | RHS | 0 | 95 | % |  |
| Relative Humidity - Operating | RHO | 0 | 85 | % |  |
| Module Supply Voltage | VCC | -0.5 | 3.6 | V |  |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Case Operating Tempera- ture | Tcase | 0 | +25 | +70 | ºC | Temperature Range = C |
| Module Supply Voltage | VCC | 3.14 | 3.3 | 3.46 | V |  |
| Power Consumption | P | - | - | 10 | W |  |
| Signaling Speed Per Channel | S | - | 26.56 | - | GB |  |

Transmitter Electrical Interfaces

| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Tx_Data Differential Input Voltage | VIN | - | - | 900 | mV |  |
| Tx_Data Differential Input Impedance | ZIN | - | 100 | - | Q |  |

Receiver Electrical Interfaces

| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Rx_Data Differential Output Voltage | VOUT | - | - | 900 | mV |  |
| Rx_Data Differential Output Impedance | ZOUT | - | 100 | - | Q |  |

# 400G QSFP-DD SR8 Optical Transceiver

Transmitter Optical Characteristics

| Parameter | Symbol |  |  |  | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max |  |  |
| Signaling rate, each lane | SR | - | 26.56 | - | GBd |  |
| Signaling Speed Accuracy | SSA | -100 |  | 100 | ppm |  |
| Modulation format |  | PAM4 |  |  |  |  |
| Average Launch Power ,each lane | POUT | -6.5 | - | 4 | dBm | Average Optical Out- put |
| Outer Optical Modulation Amplitude each lane | OMAouter | -4.5 | - | 3 | dBm |  |
| Optical Output with Tx OFF | POFF | - | - | -30 | dBm |  |
| Extinction ratio | ER | 3 |  |  | dB |  |
| Center Wavelength | λ | 840 | 850 | 868 | nm |  |
| RMS Spectral Width | Δλ | - | - | 0.6 | nm |  |
| Transmitter and dispersion eye closure (TDECQ) each lane | TDECQ | - | - | 4.5 | dB |  |
| Launch power in OMAouter minus TDECQ, each lane (min) |  | -5.9 | - | - | dB |  |
| TDECQ – 10log10(Ceq), each lane (max) |  | - | - | 4.5 | dB |  |
| RIN12 OMA | RIN12 OMA | - | - | -128 | dB/Hz |  |
| Optical return loss tolerance (Max) | ORLT |  |  | 12 | dB |  |

# 400G QSFP-DD SR8 Optical Transceiver

Receiver Optical Characteristics

| Parameter | Symbol |  |  |  | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max |  |  |
| Signaling rate, each lane | SR | - | 26.56 | - | GBd |  |
| Signaling Speed Accuracy |  | -100 |  | 100 | ppm |  |
| Modulation format |  | PAM4 |  |  |  |  |
| Average power at receive input, each lane | PIN | -8.4 | - | 4 | dBm | Note1 |
| Receive power (OMAouter), each lane (max) |  | - | - | 3 | dBm |  |
| Receiver sensitivity (OMAouter) each lane | Sen | - | - | max(– 6.6, SECQ – 8) | dBm | Note2 |
| Center Wavelength | λ | 840 | 850 | 868 | nm |  |
| Rx_LOS of Signal - Assert | PA | -24.6 | - | - | dBm |  |
| Rx_LOS of Signal - Deassert | PD | - | - | -7 | dBm |  |
| Rx_LOS of Signal - Hysteresis | PHy | 0.5 | - | - | dB |  |
| Note 1: Average received power where the BER = 2.4E-4 measured with a PRBS 31Q test pattern@26.56GBaud Note 2: Sensitivity where the BER = 2.4E-4 measured with a PRBS 31Q test pattern@26.56GBaud |  |  |  |  |  |  |

# 400G QSFP-DD SR8 Optical Transceiver

Block Diagram

Pin Assignment

# 400G QSFP-DD SR8 Optical Transceiver

Pin Description

| Pin | Symbol | Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground |  |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non Inverted Data Input |  |
| 4 | GND | Ground |  |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non Inverted Data Input |  |
| 7 | GND | Ground |  |
| 8 | ModselL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | VCC Rx | Receiver +3.3V DC Power Supply |  |
| 11 | SCL | I2C Serial Clock |  |
| 12 | SDA | I2C Serial Data |  |
| 13 | GND | Ground |  |
| 14 | Rx3p | Receiver Non Inverted Differential Output |  |
| 15 | Rx3n | Receiver Inverted Differential Output |  |
| 16 | GND | Ground |  |
| 17 | Rx1p | Receiver Non Inverted Differential Output |  |
| 18 | Rx1n | Receiver Inverted Differential Output |  |
| 19 | GND | Ground |  |
| 20 | GND | Ground |  |
| 21 | Rx2n | Receiver Inverted Differential Output |  |
| 22 | Rx2p | Receiver Non Inverted Differential Output |  |
| 23 | GND | Ground |  |
| 24 | Rx4n | Receiver Inverted Differential Output |  |
| 25 | Rx4p | Receiver Non Inverted Differential Output |  |
| 26 | GND | Ground |  |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | VCC Tx | Transmitter +3.3V DC Power Supply |  |
| 30 | VCC 1 | +3.3V DC Power Supply |  |
| 31 | Init Mode | Initialization Mode |  |

# 400G QSFP-DD SR8 Optical Transceiver

| 32 | GND | Ground |  |
| --- | --- | --- | --- |
| 33 | Tx3p | Transmitter Non Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground |  |
| 36 | Tx1p | Transmitter Non Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground |  |
| 39 | GND | Ground |  |
| 40 | Tx6n | Transmitter Inverted Data Input |  |
| 41 | Tx6p | Transmitter Non Inverted Data Input |  |
| 42 | GND | Ground |  |
| 43 | Tx8n | Transmitter Inverted Data Input |  |
| 44 | Tx8p | Transmitter Non Inverted Data Input |  |
| 45 | GND | Ground |  |
| 46 | Reserved | No connect |  |
| 47 | NC | No connect |  |
| 48 | VCC Rx1 | +3.3V DC Power Supply |  |
| 49 | NC | No connect |  |
| 50 | NC | No connect |  |
| 51 | GND | Ground |  |
| 52 | Rx7p | Receiver Non Inverted Differential Output |  |
| 53 | Rx7n | Receiver Inverted Differential Output |  |
| 54 | GND | Ground |  |
| 55 | Rx5p | Receiver Non Inverted Differential Output |  |
| 56 | Rx5n | Receiver Inverted Differential Output |  |
| 57 | GND | Ground |  |
| 58 | GND | Ground |  |
| 59 | Rx6n | Receiver Inverted Differential Output |  |
| 60 | Rx6p | Receiver Non Inverted Differential Output |  |
| 61 | GND | Ground |  |

# 400G QSFP-DD SR8 Optical Transceiver

| 62 | Rx8n | Receiver Inverted Differential Output |  |
| --- | --- | --- | --- |
| 63 | Rx8p | Receiver Non Inverted Differential Output |  |
| 64 | GND | Ground |  |
| 65 | NC | No Connect |  |
| 66 | Reserved | No Connect |  |
| 67 | VCCTx1 | +3.3V DC Power Supply |  |
| 68 | VCC2 | +3.3V DC Power Supply |  |
| 69 | Reserved | No Connect |  |
| 70 | GND | Ground |  |
| 71 | Tx7p | Transmitter Non Inverted Data Input |  |
| 72 | Tx7n | Transmitter Inverted Data Input |  |
| 73 | GND | Ground |  |
| 74 | Tx5p | Transmitter Non Inverted Data Input |  |
| 75 | Tx5n | Transmitter Inverted Data Input |  |
| 76 | GND | Ground |  |

# 400G QSFP-DD SR8 Optical Transceiver

Electrical Interface

# 400G QSFP-DD SR8 Optical Transceiver

Mechanical Dimensions

## Optical  Lane Assignment

Connector type: MPO-16

# 400G QSFP-DD SR8 Optical Transceiver

## Warnings

Handling Precautions:

This device is susceptible to damage as a result of electrostatic discharge (ESD). A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety:

Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radia- tion.

Notice:

The information provided on this page contains the product target specifications which are subject to change with- out notice.

Check with your  Sales Office for product updates, changes in specifications, sample availability and production release dates.

EYE SAFETY

Complies with 21 CFR 1040.10 and 1040.11 except for conformance with IEC 60825-1 Ed.3., as described in La- ser Notice No. 56, dated May 8, 2019.

Complies with IEC/EN 60825-1 and IEC/EN 60825-2. Class 1 laser safety compliant.

INVISIBLE LASER RADIATION.

CAUTION:

Use of controls or adjustments or performance of procedures other than those specified herein may result in haz- ardous radiation exposure.

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City