Product Specification

QSFP-DD 400G-DR4 500m transceivers

Description

The Transceiver is a high-performance, cost-effective module for optical data communication applications supporting 400G Ethernet. The Transceiver is designed to operate in switch and router applications supporting QSFP-DD MSA / 400G DR4 compliant traffic for up to 500m links.

The Transceiver can convert 8-channel 53.125Gb/s electrical data to 4-channel 106.25Gb/s optical signals. Similarly, it optically converts 4-channel 106.25Gb/s optical signals to 8-channel electrical data output on the receiver side. It has been designed to withstand the maximum range of external operating conditions including temperature, humidity and EMI. The module offers very high functionality and feature integration, accessible via a two-wire serial interface.

Features

- QSFP-DD MSA compliant

- 8x53.125Gb/s PAM4 electrical interface (400GAUI-8)

- Maximum power consumption 8W

- MPO-12 APC connector

- Up to 500m transmission on single mode fiber

- Operating case temperature: 0℃~70℃

- Single 3.3V power supply

- RoHS-6 compliant

Applications

- 400G Ethernet

- Data Center Applications

Compliance

- QSFP-DD MSA Rev5.1

- IEEE 802.3bs

- CMIS Rev4.0

- RoHS compliance

- GR-468-CORE

Absolute Maximum Ratings

Table1-Absolute Maximum Ratings

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature | TSTG | -40 |  | 85 | ºC |  |
| Operating Relative Humidity | RH | 5 |  | 85 | % | Note1 |
| Supply Voltage | VCC | -0.5 | 3.3 | 3.6 | V |  |
| Damage Threshold, each lane |  | 5 |  |  | dBm |  |

Note:

[1] Non-condensing.

Recommended Operating Conditions

Table2-Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Case temperature | Tcase | 0 |  | 70 | ℃ |  |
| Supply Voltage | VCC | 3.135 | 3.3 | 3.465 | V |  |
| Supply Current | ICC |  |  | 2552 | mA |  |
| Module Power Dissipation | P |  |  | 8 | W |  |

Optical，Electrical Characteristic

Tested under recommended operating conditions, unless otherwise noted

Table3-Transmitter Operating Characteristic-Optical，Electrical

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 53.125±100ppm |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  |  |  |
| Line wavelengths | λ | 1304.5 | 1311 | 1317.5 | nm |  |
| Average Launch Power, each lane | PAVG | -2.9 |  | 4 | dBm |  |
| Optical Modulation Amplitude (OMA), each lane | OMA | -0.8 |  | 4.2 | dBm |  |
| Extinction Ratio | ER | 3.5 |  |  | dB |  |
| Side-Mode Suppression Ratio | SMSR | 30 |  |  | dB |  |
| Launch power in OMA minus TDECQ, each lane |  | -2.2 |  |  | dBm |  |
| Transmitter and Dispersion Eye Closure for PAM4, each Lane | TDECQ |  |  | 3.4 | dB |  |
| Optical Return Loss Tolerance |  |  |  | 21.4 | dB |  |
| Transmitter Reflectance |  |  |  | -26 | dB |  |
| Average Launch Power of OFF Transmitter, each Lane |  |  |  | -15 | dBm |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  | GBd |  |
| Differential pk-pk input Voltage tolerance | Vpp |  | 900 |  | mV | Note1 |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 | mV | Note2 |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 | % |  |
| Differential Input Return Loss |  | IEEE 802.3-2018 Equation (83E-5) |  |  | dB |  |
| Differential to Common Mode Input Return Loss |  | IEEE 802.3-2018 Equation (83E-6) |  |  | dB |  |
| Module Stressed Input Test |  | IEEE 802.3bs 120E.3.4.1 |  |  |  | Note3 |

Notes:

[1] With the exception to IEEE 802.3bs 120E.3.1.2 that the pattern is PRBS31Q or scrambled idle.

[2] DC common mode voltage generated by the host. Specification includes effects of ground offset voltage.

[3] BER specified in IEEE 802.3bs 120E.1.1.

Table4-Receiver Operating Characteristic-Optical，Electrical

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Data Rate, each Lane |  | 53.125±100ppm |  |  | GBd |  |
| Modulation Format |  | PAM4 |  |  |  |  |
| Optical Input Power | PIN |  |  | 5 | dBm |  |
| Line wavelengths | λ | 1304.5 | 1311 | 1317.5 | nm |  |
| Average receiver power, each lane |  | -5.9 |  | 4 | dBm |  |
| Receiver power, each lane (OMA) |  |  |  | 4.2 | dBm |  |
| Receiver Sensitivity (OMAouter) , each lane |  |  |  | -4.4 | dBm |  |
| Stressed receiver sensitivity (OMAouter), each laned (max) |  |  |  | -1.9 | dBm |  |
| Receiver reflectance |  |  |  | -26 | dB |  |
| Losa |  | -15 |  |  | dBm |  |
| Losd |  |  |  | -8.4 | dBm |  |
| Losh |  | 0.5 |  |  | dB |  |
| Electrical Data Rate, each lane |  | 26.5625±100ppm |  |  | GBd |  |
| Differential Termination Resistance Mismatch |  | -10 |  | 10 | % |  |
| Differential output Voltage pk-pk | Vpp |  |  | 900 | mV |  |
| DC Common Mode Voltage | Vcm | -350 |  | 2850 | mV | Note 1 |
| Differential output return loss |  | IEEE 802.3-2018 Equation (83E-2) |  |  | dB |  |
| Common to differential mode conversion return loss |  | IEEE 802.3-2018 Equation (83E-3) |  |  | dB |  |

Note:

[1] DC common mode voltage generated by the host. Specification includes effects of ground offset voltage.

Digital Diagnostic Functions and Control and Status I/O Timing Characteristics

# Table5- Digital Diagnostic Functions

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature monitor absolute error | DMI_Temp | -3 |  | 3 | ℃ | Note1 |
| Supply voltage monitor absolute error | DMI_Vcc | -3% |  | 3% | V | Note2 |
| Bias current monitor absolute error | DMI_Ibias | -10% |  | 10% | mA |  |
| Laser power monitor absolute error | DMI_Tx | -3 |  | 3 | dB |  |
| RX power monitor absolute error | DMI_Rx | -3 |  | 3 | dB |  |

Notes:

[1] Temperature here is depending on module case around Max power dissipation. Temperature monitor is done over operating temperature.

[2] Supply voltage monitor is done over operating voltage.

# Table6- Control and Status I/O Timing Characteristics

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MgmtInitDuration | Max MgmtInit Duration |  |  | 2000 | ms | Note1 |
| ResetL Assert Time | t_reset_init | 10 |  |  | µs | Note2 |
| IntL Assert Time | ton_IntL |  |  | 200 | ms | Note3 |
| IntL Deassert Time | toff_IntL |  |  | 500 | µs | Note4 |
| Rx LOS Assert Time | ton_los |  |  | 100 | ms | Note5 |
| Tx Fault Assert Time | ton_Txfault |  |  | 200 | ms | Note6 |
| Flag Assert Time | ton_flag |  |  | 200 | ms | Note7 |
| Mask Assert Time | ton_mask |  |  | 100 | ms | Note8 |
| Mask Deassert Time | toff_mask |  |  | 100 | ms | Note9 |

Notes:

[1] Time from power on, hot plug or rising edge of reset until completion of the MgmtInit State.

[2] Minimum pulse time on the ResetL signal to initiate a module reset.

[3] Time from occurrence of condition triggering IntL until Vout:IntL=Vol.

[4] Time from clear on read operation of associated flag until Vout:IntL=Voh.This includes deassert times for Rx LOS, Tx Fault and other flag bits.

[5] Time from Rx LOS state to Rx LOS bit set (value = 1b) and IntL asserted.

[6] Time from Tx Fault state to Tx Fault bit set (value=1b) and IntL asserted.

[7] Time from occurrence of condition triggering flag to associated flag bit set (value=1b) and IntL asserted.

[8] Time from mask bit set (value=1b) until associated IntL assertion is inhibited.

[9] Time from mask bit cleared (value=0b) until associated IntL operation resumes.

# Table7- Surge Current Requirements

| Parameter | Symbol | Min | Typical | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Power supply voltages VccTx, VccTx1, VccRx, VccRx1, Vcc1 & Vcc2 including ripple, droop and noise below 100kHz |  | 3.135 | 3.3 | 3.465 | V | Note1 |
| Module RMS noise output 10 Hz - 10 MHz |  |  |  | 30 | mV |  |
| Module power supply noise tolerance 10 Hz - 10 MHz (peak-to-peak) | PSNRmod |  |  | 66 | mV |  |
| Module inrush - instantaneous peak duration | T_ip |  |  | 50 | µs | Note2 |
| Module inrush - initialization time | T_init |  |  | 500 | ms | Note2 |
| High Power Mode Power Class 4 module |  |  |  |  |  |  |
| Power Consumption | P_4 |  |  | 8 | W | Note3 |
| Instantaneous peak current | Icc_ip_4 | - | - | 3200 | mA |  |
| Sustained peak current | Icc_sp_4 | - | - | 2640 | mA |  |
| Steady state current | Icc_4 |  |  | 2551 | mA | Note4 |

Notes:

[1] Measured at VccTx, VccTx1, VccRx, VccRx1, Vcc1 and Vcc2.

[2] T_ip and T_init are test conditions for measuring inrush current and not characteristics of the module.

[3] User must read management register for maximum power consumption.

[4] The module must stay within its declared power class.

Figure1

Pin-out Definitions

# Figure2

# Table8-Pin Function Definitions

| Pin | Logic | Symbol | Description | Plug Sequence | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1B | Note1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input | 3B |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input | 3B |  |
| 4 |  | GND | Ground | 1B | Note1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input | 3B |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input | 3B |  |
| 7 |  | GND | Ground | 1B | Note1 |
| 8 | LVTTL-I | ModSelL | Module Select | 3B |  |
| 9 | LVTTL-I | ResetL | Module Reset | 3B |  |
| 10 |  | Vcc Rx | +3.3V Power Supply Receiver | 2B | Note2 |
| 11 | LVCMOS-I/O | SCL | 2-wire serial interface clock | 3B |  |
| 12 | LVCMOS-I/O | SDA | 2-wire serial interface data | 3B |  |
| 13 |  | GND | Ground | 1B | Note1 |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output | 3B |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output | 3B |  |
| 16 |  | GND | Ground | 1B | Note1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output | 3B |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output | 3B |  |
| 19 |  | GND | Ground | 1B | Note1 |
| 20 |  | GND | Ground | 1B | Note1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output | 3B |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output | 3B |  |
| 23 |  | GND | Ground | 1B | Note1 |
| 24 | CML-O | Rx4n | Receiver Non-Inverted Data Output | 3B |  |
| 25 | CML-O | Rx4p | Receiver Inverted Data Output | 3B |  |
| 26 |  | GND | Ground | 1B | Note1 |
| 27 | LVTTL-I | ModPrsL | Module Present | 3B |  |
| 28 | LVTTL-I | IntL | Interrupt | 3B |  |
| 29 |  | VccTx | +3.3V Power supply transmitter | 2B | Note2 |
| 30 |  | Vcc1 | +3.3V Power supply | 2B | Note2 |
| 31 | LVTTL-I | LPMode | Low Power Mode | 3B |  |
| 32 |  | GND | Ground | 1B | Note1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input | 3B |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input | 3B |  |
| 35 |  | GND | Ground | 1B | Note1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input | 3B |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input | 3B |  |
| 38 |  | GND | Ground | 1B | Note1 |
| 39 |  | GND | Ground | 1A | Note1 |
| 40 | CML-I | Tx6n | Transmitter Inverted Data Input | 3A |  |
| 41 | CML-I | Tx6p | Transmitter Non-Inverted Data Input | 3A |  |
| 42 |  | GND | Ground | 1A | Note1 |
| 43 | CML-I | Tx8n | Transmitter Inverted Data Input | 3A |  |
| 44 | CML-I | Tx8p | Transmitter Non-Inverted Data Input | 3A |  |
| 45 |  | GND | Ground | 1A | Note1 |
| 46 |  | Reserved | For future use | 3A | Note3 |
| 47 |  | VS1 | Module Vendor Specific 1 | 3A | Note3 |
| 48 |  | VccRx1 | +3.3V Power supply | 2A | Note2 |
| 49 |  | VS2 | Module Vendor Specific 2 | 3A | Note3 |
| 50 |  | VS3 | Module Vendor Specific 3 | 3A | Note3 |
| 51 |  | GND | Ground | 1A | Note1 |
| 52 | CML-O | Rx7p | Receiver Non-Inverted Data Output | 3A |  |
| 53 | CML-O | Rx7n | Receiver Inverted Data Output | 3A |  |
| 54 |  | GND | Ground | 1A | Note1 |
| 55 | CML-O | Rx5p | Receiver Non-Inverted Data Output | 3A |  |
| 56 | CML-O | Rx5n | Receiver Inverted Data Output | 3A |  |
| 57 |  | GND | Ground | 1A | Note1 |
| 58 |  | GND | Ground | 1A | Note1 |
| 59 | CML-O | Rx6n | Receiver Non-Inverted Data Output | 3A |  |
| 60 | CML-O | Rx6p | Receiver Inverted Data Output | 3A |  |
| 61 |  | GND | Ground | 1A | Note1 |
| 62 | CML-O | Rx8n | Receiver Non-Inverted Data Output | 3A |  |
| 63 | CML-O | Rx8p | Receiver Inverted Data Output | 3A |  |
| 64 |  | GND | Ground | 1A | Note1 |
| 65 |  | NC | No Connect | 3A | Note3 |
| 66 |  | Reserved | For Future Use | 3A | Note3 |
| 67 |  | VccTx1 | +3.3V Power supply | 2A | Note2 |
| 68 |  | Vcc2 | +3.3V Power supply | 2A | Note2 |
| 69 | LVTTL-I | ePPS | Precision Time Protocol (PTP) reference clock input | 3A | Note3 |
| 70 |  | GND | Ground | 1A | Note1 |
| 71 | CML-I | Tx7p | Transmitter Non-Inverted Data Input | 3A |  |
| 72 | CML-I | Tx7n | Transmitter Inverted Data Input | 3A |  |
| 73 |  | GND | Ground | 1A | Note1 |
| 74 | CML-I | Tx5p | Transmitter Non-Inverted Data Input | 3A |  |
| 75 | CML-I | Tx5n | Transmitter Inverted Data Input | 3A |  |
| 76 |  | GND | Ground | 1A | Note1 |

Notes:

[1] QSFP-DD uses common ground (GND) for all signals and supply (power). All are common within the QSFP-DD module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane.

[2] VccRx, VccRx1, Vcc1, Vcc2, VccTx and VccTx1 shall be applied concurrently. Requirements defined for the host side of the Host Card Edge Connector are listed in Table 6 from QSFP-DD MSA. VccRx, VccRx1, Vcc1, Vcc2, VccTx and VccTx1 may be internally connected within the module in any combination. The connector Vcc pins are each rated for a maximum current of 1000 mA.

[3] All Vendor Specific, Reserved and No Connect and ePPS(if not used) pins may be terminated with 50 ohms to ground on the host. Pad 65 (No Connect) shall be left unconnected within the module. Vendor specific and Reserved pads shall have an impedance to GND that is greater than 10 kOhms and less than 100 pF.

[4] Plug Sequence specifies the mating sequence of the host connector and module. The sequence is 1A,2A,3A,1B,2B,3B. (see Figure 2 for pad locations) Contact sequence A will make, then break contact with additional QSFP-DD pads. Sequence 1A,1B will then occur simultaneously, followed by 2A,2B, followed by 3A,3B.

Block Diagram of Transceiver

# Figure3

<Transmitter Section>: The MQD-35F2C converts 8-channel 53.125Gb/s electrical data to 4-channel 1310nm 106.25Gb/s optical signals for 425Gb/s optical transmission.

<Receiver Section>: Similarly, it optically converts 4-channel 1310nm 106.25Gb/s optical signals to 8-channel electrical data output on the receiver side.

Recommended Interface Circuit

Figure4

Dimensions

Figure5

Notes:

[1] Flatness: 0.05mm for the heat dissipation area and 0.075mm for the rest.

[2] Roughness: 0.8μm for the heat dissipation area and 1.6μm for the rest.

Figure6 Temperature measuring piont