# JH-400G-QDD-LR4

# 400G QSFP-DD LR4 Optical Transceiver

Product Features

 Compliant with QSFP-DD MSA

 Compliant with 100G Lambda MSA 400G-LR4 Specification compliant

 Compliant with CMIS5.0

 Compliant with IEEE Std 802.3bs

 8x53. 125Gb/s electrical interface (400GAUI-8)

 Cooled EML laser with CWDM wavelength

 Up to 10km transmission on single mode fiber (SMF) with FEC

 Single +3.3V power supply

 Case temperature range: 0 ~ +70℃

 Maximum power consumption 12W

 Duplex LC connector

 RoHS complaint

## Applications

400G

BASE-LR4 Ethernet

Data Center Interconnect •Enterprise Networking

## Description

This product is a 400Gb/s QSFP-DD optical module designed for 10km optical communication applications. The module converts 8 channels of 50Gb/s (PAM4) electrical input data to 4 channels of CWDM optical        signals and multiplexes them into a single channel for 400Gb/s optical transmission.

On the receiver side, the module optically de-multiplexes a 400Gb/s optical input into 4 channels of CWDM optical signals and converts them to 8 channels of 50Gb/s (PAM4) electrical output data.

The central wavelengths ofthe 4 CWDM channels are 1271, 1291, 1311 and 1331 nm as members of the CWDM wavelength grid defined in ITU-T G.694.2.

Host FEC is required to support up to 10km fiber transmission.

## Module Block Diagram

Ordering Information

| Part Number | Description |
| --- | --- |
| JH-400G-QDD-LR4 | 400G QSFP-DD LR4 Optical Transceiver |

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Storage Temperature Range | TSTG | -40 | +85 | ⁰C |
| Supply Voltage | VCC | 0 | 4 | V |
| Relative Humidity | RH | 10% to 90% non-condensing |  |  |

Max

Operating Conditions

| Parameter | Symbol | Min |  | Unit |
| --- | --- | --- | --- | --- |
| Case Temperature- Operating | TCASE | 0 | 70 | ⁰C |
| Supply Voltage | Vcc | 3.14 | 3.46 | V |
| Power Consumption | PDISS |  | 12 | W |
| Pre-FEC Bit Error Ratio |  |  | 2.4x10-4 |  |
| Link Distance | 2 |  | 10, | KM |

Electrical Characteristics

| Parameter | Min | Typical | Max | Unit | Notes |  |
| --- | --- | --- | --- | --- | --- | --- |
| Receiver electrical output characteristics at TP4 |  |  |  |  |  |  |
| Signaling rate per lane |  | 26.5625 |  | GBd |  |  |
| AC common-mode output voltage(RMS) |  | - | 17.5 | mV |  |  |
| Differential peak-to-peak output voltage |  |  | 900 | mV |  |  |
| Near-end ESMW (Eye symmetry mask width) |  | 0.265 |  | UI |  |  |
| Near-end Eye height, differential | 70 |  |  | mV |  |  |
| Near-end vertical eye closure |  |  | 7.5 | dB |  |  |
| Far-end ESMW (Eye symmetry mask width) |  | 0.20 |  | UI |  |  |
| Far-end Eye height, differential | 30 |  |  | mV |  |  |
| Far-end vertical eye closure |  |  | 7.5 | dB |  |  |
| Far-end pre-cursor ISI ratio | -4.5 |  | 2.5 | % |  |  |
| Common mode to differential conversion return loss | 802.3 Equation(83E-3) |  |  | dB |  |  |
| Differential output return loss | 802.3 Equation(83E-2) |  |  | dB |  |  |
| Differential termination mismatch |  |  | 10 | % |  |  |

| Transition time (min, 20% to 80%) |  |  | 9.5 |  |  |  | ps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC common mode voltage |  | -350 |  |  | 2850 |  | mV |  |  |  |
| Transmitter electrical input characteristics at TP1 |  |  |  |  |  |  |  |  |  |  |
| Signaling rate, per lane |  |  | 26.5625 |  |  |  | GBd |  |  |  |
| Differential peak-to-peak input voltage tolerance |  | 900 |  |  |  |  | mV |  |  |  |
| Differential input return loss |  | 802.3 Equation(83E-5) |  |  |  |  |  |  |  |  |
| Differential to common mode input return loss |  | 802.3 Equation(83E-6) |  |  |  |  | mV |  |  |  |
| AC common-mode output voltage(RMS) |  |  |  |  | 17.5 |  | mV |  |  |  |
| Single-ended voltage tolerance range |  | -0.4 |  |  | 3.3 |  | V |  |  |  |
| Module stressed input |  | 802.3 120E.3.4.1 |  |  |  |  | UI |  |  |  |
| Differential termination mismatch |  |  |  |  | 10 |  | % |  |  |  |
| DC common mode voltage | -350 |  |  |  |  | mV |  |  |  |  |

Optical Characteristics

| Transmitter Parameter | Lane | Min | Typical | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Lane Wavelength Range | Lane 0 Lane 1 Lane 2 Lane 3 | 1264.5 1284.5 1304.5 1324.5 | 1271 1291 1311 1331 | 1277.5 1297.5 1317.5 1337.5 | nm nm nm nm |
| Signal rate per lane |  |  | 53.125 |  | GBd |
| Average launch Power per lane |  | -2.7 |  | 5.1 | dBm |
| Total Average launch power |  |  |  | 11.1 | dBm |
| Outer Optical Modulation Amplitude (OMAouter), each lane for TDECQ<1 .4 dB for 1.4dB<=TDECQ<=3 .9dB |  | 0.3 -1. 1+TDECQ |  | 4.4 | dBm |
| Difference in launch power between any two lanes (OMAouter) |  |  |  | 4 | dB |
| Average Launch Power per Lane @ TX Off State |  |  |  | -16 | dBm |
| Transmitter Eye Closure for PAM4(TECQ),each Lane |  |  |  | 3.9 | dB |
| Transmitter and dispersion Eye Closure for PAM4(TDECQ), each Lane |  |  |  | 3.9 | dB |
| ¦TDECQ - TECQ¦ |  |  |  | 2.5 | dB |
| Extinction Ratio |  | 3.5 |  |  | dB |
| Relative Intensity Noise (OMA) |  |  |  | -136 | dB/H |

|  |  |  |  |  | z |
| --- | --- | --- | --- | --- | --- |
| Side-Mode Suppression Ration (SMSR) |  | 30 |  |  | dB |
| Optical Return Loss Tolerance |  |  |  | 15.6 | dB |
| Transmitter Reflectance |  |  |  | -26 | dB |
| Transmitter over/under-shoot |  |  |  | 25 | % |
| Transmitter peak-to-peak power |  |  |  | 5.2 | dBm |
| Transmitter transition time |  |  |  | 17 | ps |

Min             Typica

l

Receiver Optical Specifications

| Receiver Parameter | Lane |  |  | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Lane Wavelength Range | Lane 0 Lane 1 Lane 2 Lane 3 | 1264.5 1284.5 1304.5 1324.5 | 1271 1291 1311 1331 | 1277.5 1297.5 1317.5 1337.5 | nm nm nm nm |
| Signal rate per lane |  |  | 53.125 |  | GBd |
| Damage Threshold |  | 6.1 |  |  | dBm |
| Average Receive Power, each lane |  | -9 |  | 5.1 | dBm |
| Receiver Power, each lane (OMA) |  |  |  | 4.4 | dBm |
| Receiver Reflectance |  |  |  | -26 | dB |
| Difference in receive Power between any Two Lanes(OMAouter) |  |  |  | 4.3 | dBm |
| Receiver Sensitivity each lane (OMAouter) for TECQ<1 .4dB for 1.4dB<=TECQ<=3 .9dB |  |  |  | -6.8 - 8.2+TECQ | dBm |
| Stressed Receiver Sensitivity (OMAouter), each |  |  |  | -4.3 | dBm |
| Stressed Conditions for Stress Receiver Sensitivity |  |  |  |  |  |
| Stressed Eye Closure for PAM4 (SECQ),Lane under Test |  |  | 3.9 |  | dB |
| OMAouter of each Aggressor Lane |  |  | -0.4 |  | dBm |

Receiver Output Power Thresholds for Loss of Signal(LOS)

| Parameter | Min | Typical Max |  | Unit |
| --- | --- | --- | --- | --- |
| RX_LOS_Assert Min/Max | -20.0 |  |  | dBm |

| RX_LOS_De-Assert Min/Max |  |  | -12.2 | dBm |
| --- | --- | --- | --- | --- |
| RX_LOS_Hysteresis | 0.5 |  |  | dB |

Digital Diagnostic Functions

| Parameters | Unit | Specification |
| --- | --- | --- |
| Temperature Monitor absolute error | 。C | ±3 |
| Supply Voltage Monitor absolute error | % | ±5 |
| I_bias Monitor absolute error | % | ±10 |
| Received Power (Rx) Monitor absolute error | dB | ±3.0 |
| Transmit Power (Tx) Monitor absolute error | dB | ±3.0 |

Pin Definition

PIN Description

| Pin No. | Symbol | Description | Note |
| --- | --- | --- | --- |
| 1 | GND | Ground | 1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 | GND | Ground | 1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 | GND | Ground | 1 |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | VccRx | 3.3V Power Supply Receiver | 2 |
| 11 | SCL | 2-Wire serial Interface Clock |  |
| 12 | SDA | 2-Wire serial Interface Data |  |
| 13 | GND | Ground | 1 |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |
| 16 | GND | Ground | 1 |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output |  |

| 19 | GND | Ground | 1 |
| --- | --- | --- | --- |
| 20 | GND | Ground | 1 |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 | GND | Ground | 1 |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Ground | 1 |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | VccTx | 3.3V power supply transmitter | 2 |
| 30 | Vcc1 | 3.3V power supply | 2 |
| 31 | Init Mode | Initialization mode |  |
| 32 | GND | Ground | 1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Output |  |
| 35 | GND | Ground | 1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Output |  |
| 38 | GND | Ground | 1 |
| 39 | GND | Ground | 1 |
| 40 | Tx6n | Transmitter Inverted Data Input |  |
| 41 | Tx6p | Transmitter Non-Inverted Data output |  |
| 42 | GND | Ground | 1 |
| 43 | Tx8n | Transmitter Inverted Data Input |  |
| 44 | Tx8p | Transmitter Non-Inverted Data output |  |
| 45 | GND | Ground | 1 |
| 46 | Reserved | For Future Use | 3 |
| 47 | VS1 | Module Vendor Specific 1 | 3 |
| 48 | VccRx1 | 3.3V Power Supply | 2 |
| 49 | VS2 | Module Vendor Specific 2 | 3 |
| 50 | VS3 | Module Vendor Specific 3 | 3 |
| 51 | GND | Ground | 1 |
| 52 | Rx7p | Receiver Non-Inverted Data Output |  |

| 53 | Rx7n | Receiver Inverted Data Output |  |
| --- | --- | --- | --- |
| 54 | GND | Ground | 1 |
| 55 | Rx5p | Receiver Non-Inverted Data Output |  |
| 56 | Rx5n | Receiver Inverted Data Output |  |
| 57 | GND | Ground | 1 |
| 58 | GND | Ground | 1 |
| 59 | Rx6n | Receiver Inverted Data Output |  |
| 60 | Rx6p | Receiver Non-Inverted Data Output |  |
| 61 | GND | Ground | 1 |
| 62 | Rx8n | Receiver Inverted Data Output |  |
| 63 | Rx8p | Receiver Non-Inverted Data Output |  |
| 64 | GND | Ground | 1 |
| 65 | NC | No Connect | 3 |
| 66 | Reserved | For Future Use | 3 |
| 67 | VccTx1 | 3.3Vpower supply | 2 |
| 68 | Vcc2 | 3.3Vpower supply | 2 |
| 69 | Reserved | For Future Use | 3 |
| 70 | GND | Ground | 1 |
| 71 | Tx7p | Transmitter Non-Inverted Data Input |  |
| 72 | Tx7n | Transmitter Inverted Data Output |  |
| 73 | GND | Ground | 1 |
| 74 | Tx5p | Transmitter Non-Inverted Data Input |  |
| 75 | Tx5n | Transmitter Inverted Data Output |  |
| 76 | GND | Ground | 1 |

Note：

1．QSFP-DD uses common ground (GND)for all signals and supply (power). All are common within the

QSFP-DD module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane.

2．VccRx, VccRx1, Vcc1, Vcc2, VccTx and VccTx1 shall be applied concurrently. Requirements defined for

the host side of the Host Card Edge Connector are listed in Table 8.    VccRx, VccRx1, Vcc1, Vcc2, VccTx and VccTx1 may be internally connected within the module in any combination.    For power classes 4 and above the module differential loading of input voltage pins must not result in exceeding pin current limits.   The connector Vcc pins are each rated for a maximum current of 1000 mA.

3． All Vendor Specific, Reserved, No Connect and ePPS (if not used) pins may be terminated with 50  to

ground on the host.    Pad 65 (No Connect) shall be left unconnected within the module.    Vendor specific and Reserved pads shall have an impedance to GND that is greater than 10 k      and less than 100 pF.

Recommended application circuit

Mechanical Dimensions

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City