800G OSFP 2DR4

# Features

●    Hot-pluggable 800G OSFP 2DR4 form factor

●    Compliant with OSFP MSA and CMIS Rev 5.2

●    Built-in DDMI function

●    Silicon photonics MZM solution

●    8x53.125GBd PAM4 electrical and optical interface

●    Maximum link length of 500m G.652 SMF fiber with KP4-FEC

●    Single 3.3V power supply

●    Operating case temperature 0°C to +70 °C

●    Power dissipation < 15W

●    Dual MPO-12 APC receptacles

●    Compliant with RoHS 2.0

# Applications

●    Data center and cloud networks

●    800G Ethernet and InfiniBand NDR

●    Support applications: 1x800G,2x400G,4x200G, 8x100G

# Order Information

# Table 1-Order Information

| Part No. | Bit Rate (Gbps) | Wavelength (nm) | Distancenote1 | Fiber Type | DDMI |  | Connector | Tempnote2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JH-800G-OSFP-2DR4 | 850.0 | 1310 | 500m | SMF |  | YES | Dual MPO-12 APC | 0~70℃ |

Note:

# 1.     500m G.652 SMF fiber with KP4-FEC

# 2.     Case Temperature

# Absolute Maximum Ratings

# Table2- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typical |  | Max. | Unit | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | -0.3 | - |  | 3.6 | V |  |  |
| Storage Temperature | Ts | -40 | - |  | +85 | °C |  |  |
| Operating Humidity | RH | 5 | - |  | +85 | % | 1 |  |
| Control Input Voltage | VI | -0.3 | - |  | VCC+0.3 | V |  |  |
| RX damage threshold, each Lane | - | 5 |  | - | - | dBm |  |  |

Note:1 No condensation

1  /  7

800G OSFP 2DR4

# Recommended Operating Conditions

# Table 3- Recommended operating Conditions

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | 70 | °C |  |
| Power Supply Voltage | Vcc | 3.135 | 3.3 | 3.465 | V |  |
| Power Dissipation | Pdiss | - | - | 15 | W |  |
| Supply Current | Icc | - | - | 4.79 | A |  |
| Pre-FEC Bit Error Ratio | - | - | - | 2.4x10-4 | - |  |
| Post-FEC Bit Error Ratio | - | - | - | 1x10-12 | - | 1 |
| Link Distance (G.652 SMF) | - | 2 | - | 500 | m | 2 |

Note:

# 1.   FEC provided by host system

# 2.   FEC required on host system to support maximum distance

# Electrical Characteristics

# Table 4- Electrical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling Rate,each Lane | - | Gbaud | 53.125±100 ppm |  |  |  |
| Modulation Format | - | - | PAM4 |  |  |  |
| Differential Input Voltage Amplitude | Vin,ppd | mV | 400 | - | 900 |  |
| Differential Termination Mismatch | - | % | - | - | 10 |  |
| DC Common-mode Voltage Tolerance | - | mV | 350 | - | 2850 |  |
| Receiver |  |  |  |  |  |  |
| Signaling Rate,each Lane | - | Gbaud | 53.125±100 ppm |  |  |  |
| Modulation Format | - | - | PAM4 |  |  |  |
| Differential Output Voltage (Long mode) | Vout,ppd | mV | - | - | 845 |  |
| Differential Output Voltage (Short mode) | Vout,ppd | mV | - | - | 600 |  |
| Eye Height | - | mV | 15 | - | - |  |
| Vertical Eye Closure | VEC | mV | - | - | 12 |  |
| Differential Termination Mismatch | - | % | - | - | 10 |  |
| DC Common-mode Voltage Tolerance | - | mV | -350 | - | 2850 |  |

# Table 5-Optical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Not es |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling Rate,each Lane | - | Gbaud | 53.125±100 ppm |  |  |  |

800G OSFP 2DR4

| Modulation Format | - | - | PAM4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength | λc | nm | 1304.5 | 1310 |  | 1317.5 |  |
| Side-mode Suppression Ratio | SMSR | dB | 30 | - |  | - |  |
| Average Launch Power, each Lane | TxAOP | dBm | -2.9 | - |  | 4 | 1 |
| Outer Optical Modulation Amplitude, (OMAouter), each Lane | OMA | dBm | -0.8 | - |  | 4.2 |  |
| Transmitter and Dispersion Eye Closure for PAM4, each Lane | TDECQ | dB | - | - |  | 3.4 |  |
| Extinction Ratio, each lane | ER | dB | 3.5 | 4 |  | - |  |
| Average Launch Power of OFF Transmitter, each lane | TOFF | dBm | - | - |  | -15 |  |
| RIN21.4OMA | RIN | dB/Hz | - | - |  | -136 |  |
| Optical Return Loss Tolerance | ORL | dB | - | - |  | 21.4 |  |
| Transmitter reflectance | - | dB | - | - |  | -26 | 2 |
| Receiver |  |  |  |  |  |  |  |
| Signaling Rate,each Lane | - | Gbaud | 53.125±100 ppm |  |  |  |  |
| Modulation format | - | - | PAM4 |  |  |  |  |
| Receive Wavelength Range | λc | nm | 1304.5 |  | 1310 | 1317.5 |  |
| Average Receive Power, each lane | RxAOP | dBm | -5.9 |  | - | 4 |  |
| Receive Power (OMAouter), each Lane | RxOMA | dBm | - |  | - | 4.2 |  |
| Receiver Reflectance | - | dB | - |  | - | -26 |  |
| LOS Assert | LOSassert | dBm | -18 |  | - | -11 |  |
| LOS De-assert | LOSde-assert | dBm | -17 |  | - | -10 |  |
| LOS Hysteresis | LOSHys | dB | 0.5 |  | - | - |  |

Note:

1. Average launch power, each lane (min) is informative and not the principal indicator of signal strength. A transmitter with launch power below this value cannot be compliant; however, a value above this does not ensure compliance.

2. Transmitter reflectance is defined looking into the transmitter.

# Digital Diagnostic Specifications

# Table 6-Digital Diagnostic Characteristics

| Parameter | Symbol | Unit | Min | Max | Notes |
| --- | --- | --- | --- | --- | --- |
| Temperature monitor absolute error | DMI_Temp | degC | -3 | 3 | Over operating temperature range |
| Supply voltage monitor absolute error | DMI_VCC | V | -0.1 | 0.1 | Over full operating range |
| Channel RX power monitor absolute error | DMI_RX_Ch | dB | -3 | 3 |  |
| Channel Bias current monitor | DMI_Ibias_Ch | mA | -10% | 10% |  |
| Channel TX power monitor absolute error | DMI_TX_Ch | dB | -3 | 3 |  |

800G OSFP 2DR4

# Recommended Interface

## Figure 1 Recommended Interface Circuit

# Pin arrangement

## Figure 2 Pin View

800G OSFP 2DR4

Table 7-Pin Function Definitions

| Pin# | Symbol | Description | Logic | Direction | Plug Sequnce |
| --- | --- | --- | --- | --- | --- |
| 1 | GND |  | Ground |  | 1 |
| 2 | TX2p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 3 | TX2n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 4 | GND |  | Ground |  | 1 |
| 5 | TX4p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 6 | TX4n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 7 | GND |  | Ground |  | 1 |
| 8 | TX6p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 9 | TX6n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 10 | GND |  | Ground |  | 1 |
| 11 | TX8p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 12 | TX8n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 13 | GND |  | Ground |  | 1 |
| 14 | SCL | 2-wire Serial interface clock | LVCMOS-I/O | Bi-directional | 3 |
| 15 | VCC | +3.3V Power |  | Power from Host | 2 |
| 16 | VCC | +3.3V Power |  | Power from Host | 2 |
| 17 | LPWn/PRSn | Low-Power Mode / Module Present | Multi-Level | Bi-directional | 3 |
| 18 | GND |  | Ground |  | 1 |
| 19 | RX7n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 20 | RX7p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 21 | GND |  | Ground |  | 1 |
| 22 | RX5n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 23 | RX5p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 24 | GND |  | Ground |  | 1 |
| 25 | RX3n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 26 | RX3p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 27 | GND |  | Ground |  | 1 |
| 28 | RX1n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 29 | RX1p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 30 | GND |  | Ground |  | 1 |
| 31 | GND |  | Ground |  | 1 |
| 32 | RX2p | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 33 | RX2n | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 34 | GND |  | Ground |  | 1 |
| 35 | RX4p | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 36 | RX4n | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 37 | GND |  | Ground |  | 1 |
| 38 | RX6p | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 39 | RX6n | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 40 | GND |  | Ground |  | 1 |
| 41 | RX8p | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 42 | RX8n | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 43 | GND |  | Ground |  | 1 |
| 44 | INT/RSTn | Module Interrupt / Module Reset | Multi-Level | Bi-directional | 3 |
| 45 | VCC | +3.3V Power |  | Power from Host | 2 |
| 46 | VCC | +3.3V Power |  | Power from Host | 2 |
| 47 | SDA | 2-wire Serial interface data | LVCMOS-I/O | Bi-directional | 3 |
| 48 | GND |  | Ground |  | 1 |
| 49 | TX7n | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 50 | TX7p | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 51 | GND |  | Ground |  | 1 |
| 52 | TX5n | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 53 | TX5p | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 54 | GND |  | Ground |  | 1 |
| 55 | TX3n | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 56 | TX3p | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 57 | GND |  | Ground |  | 1 |
| 58 | TX1n | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 59 | TX1p | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 60 | GND |  | Ground |  | 1 |

800G OSFP 2DR4

## Figure 3, INT/RS Tn circuit

## Figure 4 LPWn/PRSn circuit

# Memory Map

Compatible with CMIS rev 5.2.

# Optical interface arrangement

The optical port is dual MPO-12 APC connector, with fiber lane assignments as shown in Figure 5

## Figure 5 Optical interface arrangement.

# Mechanical

800G 2DR4 OSFP transceivers are compatible with OSFP Module Specification Rev5.0 for pluggable form factor module.

# Unit mm

800G OSFP 2DR4

## Figure 6 Mechanical Diagram

# Warnings

Handling Precautions: This device is susceptible to damage as a result of electrostatic discharge (ESD).

A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety: Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radiation.

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City