800G QSFP-DD800 DR8 V1.1

The 800G QSFP-DD800 DR8 Transceiver is

designed to transmit and receive serial optical data links up to 106.25Gps data rate (per channel) by  PAM4 modulation format over single-mode fiber. It is a small-form-factor hot pluggable transceiver

module integrated with high performance EML

laser. It is compliant with 800G Ethernet specs and QSFP-DD MSA.

Features                                                   Applications

l Up to 106.25Gb ps data rate per channel by PAM4 modulation

l 8 duplex channels transmitters and receivers

l 8 × 100G PAM4 EML lasers

l Single MPO-16 connector receptacle optical interface compliant

l Single +3.3V power supply

l DDM function implemented

l Hot-pluggable QSFP-DD800 form factor

l Maximum link length of 500m on SMF fiber

l Power consumption: <14W

l International class 1 laser safety certified

l Operating temperature range: 0℃ ~ +70℃

# l Compliant with RoHS6.0

l 800GBASE-DR8 Ethernet

l Switch & Router Connections

l Data Centers

l Other 800G Interconnect Requirements

Standards

l IEEE Std 802.3cu™-2021

l IEEE Std 802.3ck™-2022

l IEEE Std 802.3df ™-2024

l QSFP-DD MSA 6.3

l CMIS Rev 5.1 or later version

1

# Document History

| VER.&STATUS | DATE | DESCRIPTION OF CHANGE | Modifier |
| --- | --- | --- | --- |
| 1.0 | 2023-04-06 | Initial Release | YJQ |
| 1.1 | 2024-07-01 | Optical parameters refer to IEEE Std 802.3df™-2024 | WAQ |

Specifications

(Tested under recommended operating conditions, unless otherwise noted)

| Parameter | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| Transmit characteristics |  |  |  |  |  |
| Signaling rate | 53.125 -50ppm | 53.125 | 53.125 +50ppm | GBd |  |
| Modulation format | PAM4 |  |  |  |  |
| Wavelength | 1304.5 |  | 1317.5 | nm |  |
| Side-mode suppression ratio(SMSR) | 30 |  |  | dB |  |
| Average launch power, each lane | -2.9 |  | 4 | dBm | a,b |
| Outer Optical Modulation Amplitude (OMAouter), each lane for TDECQ < 1.4 dB | -0.8 |  | 4.2 | dBm | c |
| for 1.4 dB < TDECQ < 3.4 dB | –2.2 +TDECQ |  | 4.2 | dBm |  |
| Transmitter and dispersion eye closure for PAM4(TDECQ) |  |  | 3.4 | dB |  |
| Transmitter eye closure for PAM4 (TECQ) |  |  | 3.4 | dB |  |
| ¦ TDECQ – TECQ ¦ |  |  | 2.5 | dB |  |
| Transmitter overshoot and undershoot |  |  | 22 | % |  |
| Transmitter power excursion |  |  | 5 | dBm |  |
| Extinction ratio | 3.5 |  |  | dB |  |
| Transmitter transition time |  |  | 17 | ps |  |
| Average launch power of OFF transmitter |  |  | -15 | dBm |  |
| RIN21.4OMA |  |  | -136 | dB/H z |  |
| Optical return loss tolerance |  |  | 21.4 | dB |  |
| Transmitter reflectance |  |  | -26 | dB | d |
| Receive characteristics |  |  |  |  |  |
| Signaling rate | 53.125 -50ppm | 53.125 | 53.125 +50ppm | GBd |  |
| Modulation format | PAM4 |  |  |  |  |
| Wavelength | 1304.5 |  | 1317.5 | nm |  |
| Damage threshold | 5 |  |  | dBm | e |
| Average receive power | -5.9 |  | 4 | dBm | f |
| Receive power (OMAouter) |  |  | 4.2 | dBm |  |
| Receiver reflectance |  |  | -26 | dB |  |

| Receiver sensitivity (OMAouter) for TECQ < 1.4 dB |  |  | -3.9 | dBm |  |
| --- | --- | --- | --- | --- | --- |
| for 1.4 dB ≤ TECQ ≤ 3.4 dB |  |  | TECQ-5.3 | dBm |  |
| Stressed receiver sensitivity (OMAouter) |  |  | -1.9 | dBm | g |
| Conditions of stressed receiver sensitivity test: |  |  |  |  | h |
| Stressed eye closure for PAM4 (SECQ), lane under test | 3.4 |  |  | dB |  |
| OMAouter of each aggressor lane | 4.2 |  |  | dBm |  |

Note a. Average launch power, each lane (min) is not the principal indicator of signal strength. A transmitter with  launch  power  below  this  value  cannot  be  compliant;  however,  a  value  above  this  does  not  ensure compliance.

Note b. Average launch power of –2.9 dBm corresponds to an OMA of –0.8 dBm with an extinction ratio of approximately 10 dB or an OMA of –0.1 dBm with an extinction ratio of approximately 16 dB.

Note c. For 400GBASE-DR4, the requirement on the OMAouter (min) applies even in the cases where TDECQ < 1.4 dB.

Note d. Transmitter reflectance is defined looking into the transmitter.

Note e. The receiver shall be able to tolerate, without damage, continuous exposure to an optical input signal having this average power level. The receiver does not have to operate correctly at this input power.

Note f. Average receive power, each lane (min) is not the principal indicator of signal strength. A received power below this value cannot be compliant; however, a value above this does not ensure compliance.

Note g. Measured with conformance test signal at TP3 (see IEEE Std 802.3df ™-2024 124.8.10) for the BER specified in IEEE Std 802.3df ™-2024 124.1.1.

Note h. These test conditions are for measuring stressed receiver sensitivity. They are not characteristics of

# the receiver.

Ordering Information

| Part No. | Specifications |  |  |  |  |  |  |  |  | Application |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Package | Data rate | Laser | Optical Power | Detector | Sensitivity | Temp | Reach | Others |  |
| JH-800G-QDD-DR8 | QSFP-DD 800 | 850G | EML | -2.9~4dBm | PD | < -3.9dBm @2E-4 | 0~70 oC | 500m | RoHS | Ethernet |

# Absolute Maximum Ratings

| Parameter | Symbol | Unit | Min | Max |
| --- | --- | --- | --- | --- |
| Storage Temperature | Ts | oC | -40 | +85 |
| Relative Humidity | RH | % | 5 | 85 |
| Power Supply Voltage | Vcc | V | -0.3 | +3.6 |

# Recommended Operating Conditions

| Parameter | Symbol | Unit | Min | Typ | Max |
| --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | Tc | oC | 0 |  | 70 |
| Power Supply Voltage | Vcc | V | 3.135 | 3.3 | 3.465 |
| Power Consumption | Pc | W |  |  | 14 |

# Optical Interface

Figure 1 Optical Lane Sequence

Note: Optical interface is 8° APC MPO-16. Lane sequence is shown in Figure 1.

Principle diagram

Figure 2 Module Principle Diagram

# Electric Ports Definition

| Parameter | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| Supply voltage | 3.135 |  | 3.465 | V |  |
| Signaling rate, each lane | 53.125 -100ppm | 53.125 | 53.125 +100ppm | GBd |  |
| Module input characteristics |  |  |  |  |  |
| Differential peak-to-peak input voltage tolerance | 750 |  |  | mV | TP1a |
| Peak-to-peak AC common-mode voltage tolerance Low-frequency, VCMLF | 32 |  |  | mV | TP1a |
| Full-band, VCMFB | 80 |  |  | mV |  |
| Differential-mode to common-mode return loss, RLcd | See IEEE Std 802.3ck™-2022 Equation(120G-2) |  |  | dB | TP1 |

| Parameter | Min | Typ | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| Effective return loss, ERL | 8.5 |  |  | dB | TP1 |
| Differential termination mismatch |  |  | 10 | % | TP1 |
| Module stressed input tolerance | See IEEE Std 802.3ck™-2022 120G.3.4.3 |  |  |  | TP1a |
| Single-ended voltage tolerance | -0.4 |  | 3.3 | V | TP1a |
| DC common-mode voltage tolerance | -0.35 |  | 2.85 | V | TP1 |
| Module output characteristics |  |  |  |  |  |
| Peak-to-peak AC common-mode voltage Low-frequency, VCMLF |  |  | 32 | mV | TP4 |
| Full-band, VCMFB |  |  | 80 | mV |  |
| Differential peak-to-peak output voltage Short mode |  |  | 600 | mV | TP4 |
| Long mode |  |  | 845 | mV |  |
| Eye height | 15 |  |  | mV | TP4 |
| Vertical eye closure, VEC |  |  | 12 | dB | TP4 |
| Common-mode to differential-mode return loss, RLdc | See IEEE Std 802.3ck™-2022 Equation(120G-1) |  |  | dB | TP4 |
| Effective return loss, ERL | 8.5 |  |  | dB | TP4 |
| Differential termination mismatch |  |  | 10 | % | TP4 |
| Transition time | 8.5 |  |  | ps | TP4 |
| DC common-mode voltage tolerance | -0.35 |  | 2.85 | V | TP4 |
| IIC communication |  |  |  |  |  |
| IIC Clock frequency | 100 |  | 1000 | kHz |  |
| Clock stretching |  |  | 500 | μs |  |
| # Data In Hold Time | 0 |  |  | μs |  |
| Data In Setup Time | 0.1 |  |  | μs |  |

Pin Description

Figure 3 Module pad assignment and layout

| Pad | Logic | Symbol | Description | Notes |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Intput |  |
| 4 |  | GND | Ground | 1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 |  | GND | Ground | 1 |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | +3.3V Power Supply Receiver | 2 |
| 11 | LVCOMS-I/O | SCL | TWI serial interface clock |  |
| 12 | LVCOMS-I/O | SDA | TWI serial interface data |  |
| 13 |  | GND | Ground | 1 |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground | 1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground | 1 |
| 20 |  | GND | Ground | 1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground | 1 |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground | 1 |
| 27 | LVTTL-O | Mod PrsL | Module Present |  |
| 28 | LVTTL-O | IntL/RxLOS | Interrupt/optional RxLOS |  |
| 29 |  | VccTx | +3.3V Power supply transmitter | 2 |
| 30 |  | Vcc1 | +3.3V Power supply | 2 |
| 31 | LVTTL-I | LPMode/TxDis | Low Power mode/optional TX Disable |  |

| Pad | Logic | Symbol | Description | Notes |
| --- | --- | --- | --- | --- |
| 32 |  | GND | Ground | 1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input |  |
| 35 |  | GND | Ground | 1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input |  |
| 38 |  | GND | Ground | 1 |
| 39 |  | GND | Ground | 1 |
| 40 | CML-I | Tx6n | Transmitter Inverted Data Input |  |
| 41 | CML-I | Tx6p | Transmitter Non-Inverted Data Input |  |
| 42 |  | GND | Ground | 1 |
| 43 | CML-I | Tx8n | Transmitter Inverted Data Input |  |
| 44 | CML-I | Tx8p | Transmitter Non-Inverted Data Input |  |
| 45 |  | GND | Ground | 1 |
| 46 | LVCMOS/CML-I | P/VS4 | Programmable/Module Vendor Specific 4 | 5 |
| 47 | LVCMOS/CML-I | P/VS1 | Programmable/Module Vendor Specific 1 | 5 |
| 48 |  | VccRx1 | +3.3V Power Supply | 2 |
| 49 | LVCMOS/CML-O | P/VS2 | Programmable/Module Vendor Specific 2 | 5 |
| 50 | LVCMOS/CML-O | P/VS3 | Programmable/Module Vendor Specific 3 | 5 |
| 51 |  | GND | Ground | 1 |
| 52 | CML-O | Rx7p | Receiver Non-Inverted Data Output |  |
| 53 | CML-O | Rx7n | Receiver Inverted Data Output |  |
| 54 |  | GND | Ground | 1 |
| 55 | CML-O | Rx5p | Receiver Non-Inverted Data Output |  |
| 56 | CML-O | Rx5n | Receiver Inverted Data Output |  |
| 57 |  | GND | Ground | 1 |
| 58 |  | GND | Ground | 1 |
| 59 | CML-O | Rx6n | Receiver Inverted Data Output |  |
| 60 | CML-O | Rx6p | Receiver Non-Inverted Data Output |  |
| 61 |  | GND | Ground | 1 |
| 62 | CML-O | Rx8n | Receiver Inverted Data Output |  |

| Pad | Logic | Symbol | Description | Notes |
| --- | --- | --- | --- | --- |
| 63 | CML-O | Rx8p | Receiver Non-Inverted Data Output |  |
| 64 |  | GND | Ground | 1 |
| 65 |  | NC | Not Connect | 3 |
| 66 |  | Reserved | For future use | 3 |
| 67 |  | VccTx1 | +3.3 V Power Supply | 2 |
| 68 |  | Vcc2 | +3.3 V Power Supply | 2 |
| 69 | LVCMOS-I | ePPS/Clock | 1PPS PTP clock or reference clock input | 6 |
| 70 |  | GND | Ground | 1 |
| 71 | CML-I | Tx7p | Transmitter Non-Inverted Data Input |  |
| 72 | CML-I | Tx7n | Transmitter Inverted Data Input |  |
| 73 |  | GND | Ground | 1 |
| 74 | CML-I | Tx5p | Transmitter Non-Inverted Data Input |  |
| 75 | CML-I | Tx5n | Transmitter Inverted Data Input |  |
| 76 |  | GND | Ground | 1 |

Note 1: QSFP-DD uses common ground (GND) for all signals and supply (power). All are common within the QSFP-DD module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane. Each connector Gnd contact is rated for a steady state current of 500mA.

Note 2: VccRx, VccRx1, Vcc1, Vcc2, VccTx and VccTx1 shall be applied concurrently. Supply requirements defined for the host side of the Host Card Edge Connector are listed in QSFP-DD MSA Specification Table 13. For  power  classes 4  and  above  the  module  differential  loading  of  input  voltage  pads  must  not  result  in exceeding contact current limits. Each connector Vcc contact is rated for a steady state current of 1500mA.

Note 3: Reserved pad recommended to be terminated with 10kΩ to ground on the host. Pad 65 (No Connect) Shall be left unconnected within the module, optionally pad 65 may get terminated with 10kΩ to ground on the host.

Note 4: Plug Sequence specifies the mating sequence of the host connector and module. The sequence is 1A, 2A, 3A, 1B, 2B, 3B. (see Figure 3 for pad locations) Contact sequence A will make, then break contact with additional QSFP-DD  pads. Sequence  1A  and  1B  will then occur simultaneously, followed  by 2A and 2B, followed by 3A and 3B.

Note 5: Full definitions of the P/VSx signals currently under development. For module designs using

programmable/vendor specific inputs P/VS1 and P/VS4 signals it is recommended each to be terminated in the module with 10kΩ . For host designs using programmable/vendor specific outputs P/VS2 and P/VS3 signals it is recommended each to be terminated on the host with 10kΩ .

Note 6: For host not implementing ePPS/Clock, it is not necessary to parallel terminate the ePPS/Clock signal to  ground  on  the  host.  ePPS/Clock  already  has  parallel  termination  in  the  module  see  QSFP-DD  MSA Specification clause 4.2.6.

# Module Memory Map

Figure 4 Digital Diagnostic Memory Map

Host Board Power Supply Filtering

Any voltage drop across a filter network on the host is counted against the host DC set point accuracy

specification. System designers should choose components with appropriate DCR and ESR, to minimize the voltage drop and the amount of noise coupled to the module. Hosts supporting higher power classes modules may require additional design considerations, in order to minimize the voltage drop and the amount of noise  coupled to the module.

Figure 5 Reference Power Supply Filter for Module Testing

# Package Outline

Figure 6 Package Outline