10GBASE-T Copper SFP+ Transceiver

SFP+-10G-T-RM-Y

Rate Matching mode

1. PRODUCT FEATURES

- Support 10Gbase-T / 5Gbase-T / 2.5Gbase-T / 1000base-T on line port

- Support 10Gbase-R on the host port

- Hot-pluggable SFP footprint

- Compact RJ-45 connector assembly

- RoHS compliant and lead-free

- Single +3.3V power supply

- 10 Gigabit Ethernet over Cat 6a cable

- Ambient Operating temperature:0°C to +70°C

2. PRODUCT DESCRIPTION

SFP+-10G-T-RM Copper Small Form Pluggable (SFP) transceivers are based on the SFP Multi Source Agreement (MSA) . They are compatible with the 10Gbase-T / 5Gbase-T / 2.5Gbase-T / 1000base-T standards as specified in IEEE Std 802.3 . SFP+-10GBASE-T uses the SFP's RX_LOS (must be pulled up on the host) pin for link indication. If you pull up or open the SFP's TX_DISABLE pin, the PHY IC is reset.

3. Cable Length

| Line Port | Cable | Reach | Host Port |
| --- | --- | --- | --- |
| 10Gbase-T | Cat 6a F/FTP | 30m | 10GBase-R |
| 5Gbase-T | CAT5E | 50m | 10GBase-R |
| 2.5Gbase-T | CAT5E | 50m | 10GBase-R |
| 1000base-T | CAT5E | 100m | 10GBase-R |
| 100BASE-Tx | CAT5E | 100m | 10GBase-R |

4. SFP to Host Connector Pin Out

| Pin | Symbol | Name/Description | Ref. |
| --- | --- | --- | --- |
| 1 | VEET | Transmitter Ground (Common with Receiver Ground) |  |
| 2 | TFAULT | Transmitter Fault. Not supported. |  |
| 3 | TDIS | Transmitter Disable. Laser output is disabled on high or open. | 1 |
| 4 | MOD_DEF(2) | Module Definition 2. Data line for Serial ID. | 2 |
| 5 | MOD_DEF(1) | Module Definition 1. Clock line for Serial ID. | 2 |
| 6 | MOD_DEF(0) | Module Definition 0. Grounded within the module. | 2 |
| 7 | RS0 | No connection required |  |
| 8 | LOS | High indicates no link. Low indicates linked. |  |
| 9 | RS1 | No connection required |  |
| 10 | VEER | Receiver Ground (Common with Transmitter Ground) |  |
| 11 | VEER | Receiver Ground (Common with Transmitter Ground) |  |
| 12 | RD- | Receiver Inverted DATA out. AC Coupled |  |
| 13 | RD+ | Receiver Non-inverted DATA out. AC Coupled |  |
| 14 | VEER | Receiver Ground (Common with Transmitter Ground) |  |
| 15 | VCCR | Receiver Power Supply |  |
| 16 | VCCT | Transmitter Power Supply |  |
| 17 | VEET | Transmitter Ground (Common with Receiver Ground) |  |
| 18 | TD+ | Transmitter Non-Inverted DATA in. AC Coupled. |  |
| 19 | TD- | Transmitter Inverted DATA in. AC Coupled. |  |
| 20 | VEET | Transmitter Ground (Common with Receiver Ground) |  |

Notes:

1. PHY disabled on TDIS  > 2.0V or open, enabled on TDIS < 0.8V

2. Should be pulled up with 4.7 k–10 k Ohms on the host board to a voltage between 2.0 V and 3.6 V. MOD_DEF(0) pulls the line low to indicate the module is plugged in.

Figure 1. Diagram of host board connector block pin numbers and names

5.  +3.3V Volt Electrical Power Interface

The SFP+-10G-T has an input voltage range of 3.3 V +/- 5%. The 4V maximum voltage is not allowed for continuous operation.

| +3.3 Volt Electrical Power Interface |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Supply Current | Is |  | 570 | 600 | mA | 10G speed 30m transmission |
| Input Voltage | Vcc | 3.13 | 3.3 | 3.47 | V | Referenced to GND |
| Maximum Voltage | Vmax |  |  | 4 | V |  |

6. Low-Speed Signals

MOD_DEF(1)  (SCL)  and  MOD_DEF(2)  (SDA) are open-drain  CMOS  signals  (see section VII, "Serial Communication Protocol").   Both MOD_DEF(1) and MOD_DEF(2) must be pulled up to host_Vcc

| Low-Speed Signals, Electronic Characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Max | unit | Notes/Conditions |
| SFP Output LOW | VOL | 0 | 0.5 | V | 4.7k to 10k pull-up to host_Vcc, measured at the host side of the connector |
| SFP Output HIGH | VOH | host_Vcc -0.5 | host_Vcc + 0.3 | V | 4.7k to 10k pull-up to host_Vcc, measured at the host side of the connector |
| SFP Input LOW | VIL | 0 | 0.8 | V | 4.7k to 10k pull-up to Vcc, measured at the SFP side of the connector |
| SFP Input HIGH | VIH | 2 | Vcc + 0.3 | V | 4.7k to 10k pull-up to Vcc, measured at the SFP side of the connector |

7. High-Speed Electrical Interface

All high-speed signals are AC-coupled internally.

| High-Speed Electrical Interface, Transmission Line-SFP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Line Frequency | fL |  | 125 |  | MHz | 5-level encoding, per IEEE 802.3 |
| Tx Output Impedance | Zout, TX |  | 100 |  | Ohm | Differential, for all frequencies between 1MHz and 125 MHz |
| Rx Input Impedance | Zin, RX |  | 100 |  | Ohm | Differential, for all frequencies between 1MHz and 125 MHz |

| High-Speed Electrical Interface, Host-SFP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Single-ended data input swing | Vinsing | 250 |  | 1200 | mV | Single-ended |
| Single-ended data output swing | Voutsing | 350 |  | 800 | mV | Single ended |
| Rise/Fall Time | Tr, Tf |  | 175 |  | psec | 20%-80% |
| Tx Input Impedance | Zin |  | 50 |  | Ohm | Single ended |
| Rx Output Impedance | Zout |  | 50 |  | Ohm | Single ended |

8. General Specifications

| General |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Data Rate | BR | 1 |  | 10 | Gb/sec | IEEE 802.3 compatible. See Notes 1,2 below |

Notes:

- Clock tolerance is +/- 50 ppm

| Addr | Field Size (Bytes) | Name of Field | HEX | Description |
| --- | --- | --- | --- | --- |
| 0 | 1 | Identifier | 03 | SFP |
| 1 | 1 | Ext. Identifier | 04 | MOD4 |
| 2 | 1 | Connector | 07 | LC |
| 3-10 | 8 | Transceiver | 10 00 00 00 40 40 0C 00 | Transmitter Code |
| 11 | 1 | Encoding | 06 | 64B66B |
| 12 | 1 | BR, nominal | 67 | 10300 Mbps |
| 13 | 1 | Reserved | 00 |  |
| 14 | 1 | Length (9 um)-km | 00 |  |
| 15 | 1 | Length (9 μm) | 00 |  |
| 16 | 1 | Length (50 μm) | 08 | 80 |
| 17 | 1 | Length (62.5 μm) | 03 | 30 |
| 18 | 1 | Length (copper) | 00 |  |
| 19 | 1 | Reserved | 1E | 30 |
| 20-35 | 16 | Vendor name | 57 49 4E 54 4F 50 20 20 20 20 20 20 20 20 20 20 | JH |
| 36 | 1 | Reserved | 00 |  |
| 37-39 | 3 | Vendor OUI | 00 00 00 |  |
| 40-55 | 16 | Vendor PN | xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx | ASC II |
| 56-59 | 4 | Vendor Rev | 31 2E 30 20 | V1.0 |
| 60-61 | 2 | Wavelength | 03 52 | 850nm |
| 62 | 1 | Reserved | 00 |  |
| 63 | 1 | CC BASE | XX | Check sum of bytes 0~62 |
| 64-65 | 2 | Options | 00 1A | LOS, TX_DISABLE, TX_FAULT |
| 66 | 1 | BR, max | 00 |  |
| 67 | 1 | BR, min | 00 |  |
| 68-83 | 16 | Vendor SN | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | Unspecified |
| 84-91 | 8 | Vendor date code | XX XX XX 20 | Year, Month, Day |
| 92-94 | 3 | Reserved | XX XX XX |  |
| 95 | 1 | CC_EXT | XX | Check sum of bytes 64~94 |
| 96-255 | 160 | Vendor-specific |  |  |

9. EEPROM INFORMATION (A0)

10. Environmental Specifications

Automatic crossover detection is enabled. An external crossover cable is not required

| Environmental Specifications |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Operating Temperature | Top | 0 |  | 70 | °C | Case temperature |
| Storage temperature | Tsto | -40 |  | 85 | °C | Ambient temperature |

11. Serial Communication Protocol

| Serial Bus Timing, Requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| I 2C Clock Rate |  | 0 |  | 200,000 | Hz |  |

All JH SFPs support the 2-wire serial communication protocol outlined in the SFP MSA. These SFPs use an MCU and can be accessed with an address of A0h.

12. Recommended Application Circuit

13. Outline Dimensions (mm)