400G OSFP Direct Attach Cable (DAC) Passive

Product Specification

Features

- Compliant to IEEE802.3ck&802.3cd

- Data rates : 112-PAM4（400G CR4） or 56G-PAM4(400G CR8）

- Maximum aggregate data rate: 400Gb/s (8 x 50Gb/s or 4X100Gb/s)

- I/O Connector designed for high speed differential signal applications

- Compliant with OSFP Octal Small Form Factor Pluggable- OSFP MSA

- Low Power Consumption < 0.5W

- Wire AWG : 30 / 28 / 26

- Length = 1- 3 meters

- EEPROM signature can be customized

- RoHS compliant

Applications

- Storage, Switch, & RAID Systems

- High-Performance Computing (HPC)

- Network Interface Cards (NIC's)

- Infniband NDR&HDR data rate

- Telecommunication equipment

- Data center and enterprise storage systems

Part Number Ordering Information

| Part No. | Bit Rate (Gbps) | Distance | AWG | Tempnote1 |
| --- | --- | --- | --- | --- |
| JH-400G-OSFP-D1-DAC | 400Gbps | 1m | 30，28，26AWG | 0℃~+70℃ |

Note: 1 Case Temperature

General Specifications

| Parameter | Value |
| --- | --- |
| Type | Passive / Copper |
| End | End A =OSFP 8X (OSFP MSA) |
|  | End B =OSFP 8X (OSFP MSA) |
| Bend Radius | 5X OD (Compliant to SFF 8417) |
| Differential signal rate | Tx and Rx each 56G-PAM4 X8 pair |
| Impedance | 100 Ohms differential, AC-coupled I/O |
| Voltage | 30 Volts AC (RMS)/DC Max. |
| Current | 0.5 Amps Max |
| Temperature | Operating: 0°C to 70°C |
|  | Storage:-20°C to +80°C |
| Durability | Contact Pad Plating:0.76μm Au Module Cycles -50 cycles Connector/Cage:Cycles:100 cycles |

Performance Test

Electrical Connector Performance Test

| Test | Test Procedure | Condition | Requirement |
| --- | --- | --- | --- |
| EIA-364-23 Low Level Contact Resistance | Apply a maximum voltage of 20 mV and a current of 100mA | Mated | <10 mΩ, △ max from initial |
| EIA-364-09 Durability (precondition) | Perform plug & unplug cycles: 50 times |  | No evidence of physical damage |
| EIA-364-17 Temperature Life | method A, Test Condition 3 a105°±2°C :( 120 hours) | Mate | Conditioning Exposure |
| Reseating | Manually unplug & plug the connector, 3 cycles |  | No evidence of physical damage |
| EIA 364-32; Thermal Shock | Method A, test condition I Cycle connectors 10 times between 15°±3°C at 80% RH and 85°±3°C at 50% RH. Ramp should be a minimum of 2°C per minute and dwell times should insure that the contacts reach the extreme temperature for a minimum of 5 minutes | Mated | None |
| EIA-364-31 Cyclic Temperature & Humidity | Cycle connector between 25°±3°C at 80% RH and 65°±3°C at 50% RH 24 cycles. Ramp times should be 0.5 hour and dwell should be 1.0 hour | Mated | None |
| EIA-364-28 Mechanical Vibration | Test Condition VII Test Condition letter D 15 minutes in each of 3 mutually perpendicular directions. Both mating halves rigidly fixed to not contribute to relative motion of one contact against another. | Mated | Discontinuity <1μsec No evidence of physical damage |
| EIA-364-27 Mechanical Shock | Test Condition letter A 3 shocks in each axis(18) Both mating halves rigidly fixed to not contribute to relative motion of one contact against another. | Mated | Discontinuity <1μsec No evidence of physical damage |
| EIA-364-65 Mixed Flowing Gas (Mated) | Class IIA :14 days | See Note | None |
| Thermal Disturbance | Cycle connectors 10 times between 15°±3°C at 80% RH and 85°±3°C at 50% RH. Ramp should be a minimum of 2°C per minute and dwell times should insure that the contacts reach the temperature extremes for a minimum of 5 minutes | Mated | None |
| EIA-364-20 Dielectric Withstanding voltage | Apply a voltage of 500 VDC for 1 minute between adjacent terminals and between adjacent terminals and ground | Mated | No disruptive discharge No leakage current in excess of 5mA |
| Note: - For PL1, expose 1/2 of the specimens unmated for 224 hours of the test duration. Mate the specimen to the same one used during preconditioning temperature life. Expose for the duration of the test (112 hours). - For PL2, expose1/2of the specimens unmated for 5 days, mate & un-mate all Specimens 1 time, measure LLCR on all specimens. Expose same1/2of specimens unmated for 5 days, mate & un-mate all specimens 1 time, measure LLCR on all Specimens. Expose all specimens mated for 5 days, mate & un-mate all specimens 1 time, measure LLCR on all specimens. Expose all specimens mated for 5 days, mate & un-mate all specimens 1 time. Characterize porosity & plating thickness before test sequence. |  |  |  |

General requirements

| Test | Parameter | Requirement |
| --- | --- | --- |
| Connection testing (Capacitance measurement) | CTXy-RXy (300µs @ 3V) | 100nF ± 10% |
| Short circuit testing | RXXy-XXy (500µs @ 25V DC) | ≥ 1MΩ |

Mechanical Test 1

| Test | Test Procedure | Condition | Requirement |
| --- | --- | --- | --- |
| Temperature Rise (via current cycling) | Measure the temperature rise at the rated current after 96 hours. (45 minutes ON and 15 minutes OFF). Fixture as required. | Mated | Temperature Rise: +30°C maximum |
| Module Mate Forces (Module only) | Mate connector at a rate of 12.7 mm per minute | Mated | 40N MAX Mating force |
| Module Un-mate Forces (Module only) | Un-Mate connector at a rate of 12.7 mm per minute | Un-Mated | 30N MAX Unmating force |
| Module Retention | Apply force of 125N to the cable module Axial Load with latch enabled | Mated | 125N MIN No damage |
| Cage Retention (Latch Strength) | Measure force necessary to remove OSFP Module from cage assembly with latch enabled | Mated | 125N MIN No damage |
| Cage Press-fit Insertion Force | Axial load on top of Cage |  | 550N MAXIMUN Insertion force |
| Cage Press-fit Retention Force | Axial load on all exposed Press-fit tails simultaneously |  | 114N MINIMUM Retention force |
| Cable Latitudinal Force | Apply force of 75N to the cable module parallel to the test board and perpendicular to the cage in either direction for 10 minutes | Mated | No discontinuities of 1μsec or longer duration |
| Cable Longitudinal Force | Apply force of 75N to the cable module perpendicular to the test board and downward for 10 minutes | Mated | No discontinuities of 1μsec or longer duration |

Mechanical Test 2

| Test | Test Procedure | Condition | Requirement |
| --- | --- | --- | --- |
| Differential Impedance | EIA-364-108 Rise time of 70 ps (20 % - 80 %) | Mated | 100Ω+/-10Ω |
| Cable Minimum bend Radius | EIA-364-41 1Cycle in each of 4 Perpendicular directions |  | No physical damage |
| Wire Flex | EIA-368-41E Flex cable 180° for 15 cycles 2.5 inch. from back of shell to top of roller | Mated | Discontinuity <1μsec No physical damage |
| Cable Strain Relief | 25 mm/min ,90N min. | Mated | No physical damage |
| Solderability | Per SMES-152 | Mated | Solder coverage 95% minimum |
| SMT Process compatibility |  | - | Dimensional: Conformance to sales Drawing Requirements Visual: No Damage |

Signal Integrity Requirements Test (Connector ONLY)

| Test | Test Procedure | Requirement |
| --- | --- | --- |
| Differential Impedance (Connector Area) | EIA 364-108 Rise time: 25ps (20-80%) Includes Host Board Interface, Connector and Edge Card Interface | For Reference Only 90-110 Ohms (Distribution) |
| Near End Isolation | EIA 364-90 60 MHz to 16 GHz | -40 dB Min. at 6GHz -30 dB Min. at 6~16GHz (Frequencies up to 16 GHz) |
| Differential Insertion Loss | EIA 364-101 50 MHz to 16 GHz | -1.0 dB Max. at 6GHz -2.5 dB Max. at 6~8GHz -6 dB Max. at 8~16GHz (Frequencies up to 16 GHz) |
| Differential Return Loss | EIA 364-108 10 MHz to 16 GHz | -12 dB Min. at 4GHz -5 dB Min. at 4~16GHz (Frequencies up to 16 GHz) |

Signal Integrity Requirements (Connector& Cable systems)

| Test | Condition | Requirement |
| --- | --- | --- |
| Return Loss (SDD22) | Measured from 10 MHz <f < 19 GHz | Following SPEC Equation |
| Insertion Loss (SDD21) | Measured from 10 MHz <f < 19 GHz | Max: 17.16 dB @13.28GHz |
| Differential to common-mode return loss (SCD22) | Measured from 10 MHz <f < 19 GHz | Following SPEC Equation |
| Differential to common-mode conversion loss (SCD21) | Measured from 10 MHz <f < 19 GHz | Following SPEC Equation |
| Common-mode to common-mode return loss (SCC22) | Measured from 10 MHz <f < 19 GHz | Following SPEC Equation |
| SCD21-SDDD21 | Measured from 10 MHz <f < 19 GHz | Following SPEC Equation |
| Channel Operating Margin | COM = THRU+4NEXT+3FEXT | Min: 3 dB |

Temperature - (Connector only )

| Parameter | Value |
| --- | --- |
| Average Ramp Rate | 3°C/sec Max |
| Preheat Temperature | 150°C Min to 200°C Max |
| Preheat Time | 60 to 180 sec |
| Ramp to Peak | 3°C/sec Max |
| Time over Liquid as(217℃) | 60 to 150 sec |
| Peak Temperature | 250+0/-5°C |
| Time within 5℃ of Peak | 20 to 40 sec |
| Ramp-Cool Down | 6°C/sec Max |
| Time 25℃ to Peak | 7min Max |

Pin Descriptions

| Pin # | Symbol | Description | Logic | Direction | Plug Sequence |
| --- | --- | --- | --- | --- | --- |
| 1 | GND |  | Ground |  | 1 |
| 2 | TX2p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 3 | TX2n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 4 | GND |  | Ground |  | 1 |
| 5 | TX4p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 6 | TX4n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 7 | GND |  | Ground |  | 1 |
| 8 | TX6p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 9 | TX6n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 10 | GND |  | Ground |  | 1 |
| 11 | TX8p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 12 | TX8n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 13 | GND |  | Ground |  | 1 |
| 14 | SCL | 2-wire Serial interface clock | LVCMOS-I/O | Bi-directional | 3 |
| 15 | VCC | +3.3V Power |  | Power from Host | 2 |
| 16 | VCC | +3.3V Power |  | Power from Host | 2 |
| 17 | LPWn/PRSn | Low-Power Mode / Module Present | Multi-Level | Bi-directional | 3 |
| 18 | GND |  | Ground |  | 1 |
| 19 | RX7n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 20 | RX7p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 21 | GND |  | Ground |  | 1 |
| 22 | RX5n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 23 | RX5p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 24 | GND |  | Ground |  | 1 |
| 25 | RX3n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 26 | RX3p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 27 | GND |  | Ground |  | 1 |
| 28 | RX1n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 29 | RX1p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 30 | GND |  | Ground |  | 1 |
| 31 | GND |  | Ground |  | 1 |
| 32 | RX2p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 33 | RX2n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 34 | GND |  | Ground |  | 1 |
| 35 | RX4p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 36 | RX4n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 37 | GND |  | Ground |  | 1 |
| 38 | RX6p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |

| Pin # | Symbol | Description | Logic | Direction | Plug Sequence |
| --- | --- | --- | --- | --- | --- |
| 39 | RX6n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 40 | GND |  | Ground |  | 1 |
| 41 | RX8p | Receiver Data Non-Inverted | CML-O | Output to Host | 3 |
| 42 | RX8n | Receiver Data Inverted | CML-O | Output to Host | 3 |
| 43 | GND |  | Ground |  | 1 |
| 44 | INT/RSTn | Module Interrupt / Module Reset | Multi-Level | Bi-directional | 3 |
| 45 | VCC | +3.3V Power |  | Power from Host | 2 |
| 46 | VCC | +3.3V Power |  | Power from Host | 2 |
| 47 | SDA | 2-wire Serial interface data | LVCMOS-I/O | Bi-directional | 3 |
| 48 | GND |  | Ground |  | 1 |
| 49 | TX7n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 50 | TX7p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 51 | GND |  | Ground |  | 1 |
| 52 | TX5n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 53 | TX5p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 54 | GND |  | Ground |  | 1 |
| 55 | TX3n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 56 | TX3p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 57 | GND |  | Ground |  | 1 |
| 58 | TX1n | Transmitter Data Inverted | CML-I | Input from Host | 3 |
| 59 | TX1p | Transmitter Data Non-Inverted | CML-I | Input from Host | 3 |
| 60 | GND |  | Ground |  | 1 |

Electrical Pin-out Assignment

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City