# 400G QSFP-DD Direct Attach Cable - PAM4 Datasheet

## General   Description

QSFP-DD passive copper cable assembly feature eight differential copper pairs,providing four data transmission channels at speeds up to 56Gbps(PAM4) per channel,and meets 400G Ethernet and InfiniBand Enhanced Data Rate(EDR) requirements.Available in a broad rang of wire gages-from 28AWG through 30AWG-this 400G copper cable assembly features low insertion loss and low cross talk.

QSFP-DD uses PAM4 signals for transmission, which doubles the rate. However, there are more stringent requirements for cable insertion loss. For detailed requirements, please see High Speed Characteristics.

## Features   and Benefits

l  Compatible with IEEE 802.3bj and IEEE 802.3cd

l  Supports aggregate data rates of 400Gbps(PAM4)

l  Optimized construction to minimize insertion loss and cross talk

l  Pull-to-release slide latch design

l  28AWG through 30AWG cable

l  Straight and break out assembly configurations available

l  Customized cable braid termination limits EMI radiation

l  Customizable EEPROM mapping for cable signature

l  RoHS omplia

## Product     Applications

l Switches,servers and routers

l Data Center networks

l Storage area networks

l High performance computing

l Telecommunication and wireless infrastructure

l Medical diagnostics and networking

l Test and measurement equipment

## Industry   Standards

l 400G Ethernet(IEEE 802.3cd)

l InfiniBand EDR

## High Speed Characteristics

| Parameter | Symbol | Min | Typical | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Differential Impedance | TDR | 90 | 100 | 110 | Ώ |  |
| Insertion loss | SDD21 | -16.06 |  |  | dB | At 13.28 GHz |
| Differential Return Loss | SDD11 SDD22 |  |  | See 1 | dB | At 0.05 to 4.1 GHz |
|  |  |  |  | See 2 | dB | At 4.1 to 19 GHz |
| Common-mode to common-mode output return loss | SCC11 SCC22 |  |  | -2 | dB | At 0.2 to 19 GHz |
| Differential to common-mode return loss | SCD11 SCD22 |  |  | See 3 | dB | At 0.01 to 12.89 GHz |
|  |  |  |  | See 4 |  | At 12.89 to 19 GHz |
| Differential to common Mode Conversion Loss | SCD21-IL |  |  | -10 | dB | At 0.01 to 12.89 GHz |
|  |  |  |  | See 5 |  | At 12.89 to 15.7 GHz |
|  |  |  |  | -6.3 |  | At 15.7 to 19 GHz |
| Notes: 1. Reflection Coefficient given by equation SDD11(dB) < -16.5 + 2 × SQRT(f ), with f in GHz 2. Reflection Coefficient given by equation SDD11(dB) < -10.66 + 14 × log10(f/5.5), with f in GHz 3. Reflection Coefficient given by equation SCD11(dB) < -22 + (20/25.78)*f, with f in GHz 4. Reflection Coefficient given by equation SCD11(dB) < -15 + (6/25.78)*f, with f in GHz 5. Reflection Coefficient given by equation SCD21(dB) < -27 + (29/22)*f, with f in GHz |  |  |  |  |  |  |

## Pin Descriptions

QSFP-DD Pin Function Definition

| Pin | Logic | Symbol | Description |
| --- | --- | --- | --- |
| 1 |  | GND | Ground |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input |
| 4 |  | GND | Ground |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input |
| 7 |  | GND | Ground |
| 8 | LVTTL-I | ModSelL | Module Select |
| 9 | LVTTL-I | ResetL | Module Reset |
| 10 |  | Vcc Rx | +3.3V Power Supply Receiver |
| 11 | LVCMOS- | SCL | 2-wire serial interface clock |
|  | I/O |  |  |

| 12 | LVCMOS- | SDA | 2-wire serial interface data |
| --- | --- | --- | --- |
|  | I/O |  |  |
| 13 |  | GND | Ground |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |
| 16 |  | GND | Ground |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |
| 19 |  | GND | Ground |
| 20 |  | GND | Ground |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |
| 23 |  | GND | Ground |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |
| 26 |  | GND | Ground |
| 27 | LVTTL-O | Mod PrsL | Module Present |
| 28 | LVTTL-O | IntL | Interrupt |
| 29 |  | Vcc Tx | +3.3V Power supply transmitter |
| 30 |  | Vcc1 | +3.3V Power supply |
| 31 | LVTTL-I | LPMode | Low Power Mode |
| 32 |  | GND | Ground |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input |
| 35 |  | GND | Ground |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input |
| 38 |  | GND | Ground |
| 39 |  | GND | Ground |
| 40 | CML-I | Tx6n | Transmitter Inverted Data Input |
| 41 | CML-I | Tx6p | Transmitter Non-Inverted Data Input |
| 42 |  | GND | Ground |
| 43 | CML-I | Tx8n | Transmitter Inverted Data Input |
| 44 | CML-I | Tx8p | Transmitter Non-Inverted Data Input |
| 45 |  | GND | Ground |
| 46 |  | Reserved |  |
| 47 |  | VS1 |  |
| 48 |  | VccRx1 | +3.3V Power supply |
| 49 |  | VS2 |  |
| 50 |  | VS3 |  |
| 51 |  | GND | Ground |
| 52 | CML-O | Rx7p | Receiver Non-Inverted Data Output |
| 53 | CML-O | Rx7n | Receiver Inverted Data Output |
| 54 |  | GND | Ground |
| 55 | CML-O | Rx5p | Receiver Non-Inverted Data Output |
| 56 | CML-O | Rx5n | Receiver Inverted Data Output |
| 57 |  | GND | Ground |
| 58 |  | GND | Ground |
| 59 | CML-O | Rx6n | Receiver Inverted Data Output |
| 60 | CML-O | Rx6p | Receiver Non-Inverted Data Output |
| 61 |  | GND | Ground |

| 62 | CML-O | Rx8n | Receiver Inverted Data Output |
| --- | --- | --- | --- |
| 63 | CML-O | Rx8p | Receiver Non-Inverted Data Output |
| 64 |  | GND | Ground |
| 65 |  | NC |  |
| 66 |  | Reserved |  |
| 67 |  | VccTx1 | +3.3V Power supply |
| 68 |  | VCC2 | +3.3V Power supply |
| 69 |  | Reserved |  |
| 70 |  | GND | Ground |
| 71 | CML-I | Tx7p | Transmitter Non-Inverted Data Input |
| 72 | CML-I | Tx7n | Transmitter Inverted Data Input |
| 73 |  | GND | Ground |
| 74 | CML-I | Tx5p | Transmitter Non-Inverted Data Input |
| 75 | CML-I | Tx5n | Transmitter Inverted Data Input |
| 76 |  | GND | Ground |

Mechanical Specifications

The connector is compatible with the QSFP-DD specification.

| Length (m) | Cable AWG |
| --- | --- |
| 1 | 30 |
| 2 | 28 |
| 3 | 28 |

## Regulatory Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1(>2000 Volts) |
| Electromagnetic Interference(EMI) | FCC Class B | Compliant with Standards |
|  | CENELEC EN55022 Class B |  |
|  | CISPR22 ITE Class B |  |
| RF Immunity(RFI) | IEC61000-4-3 | Typically Show no Measurable Effect from a 10V/m Field Swept from 80 to 1000MHz |
| RoHS Compliance | RoHS Directive 2011/65/EU and it's Amendment Directives （EU） 2015/863 | RoHS （EU）2015/863 compliant |
| REACH Compliance | REACH Regulation (EC) No 1907/2006 | REACH (EC) No 1907/2006 compliant |

## Contact Information:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City