400Gbps QSFP DD To 2x 200G QSFP56 Passive High Speed Cable

Specification

- Description ：

QSFP-DD (Double Density) Interconnect System and Cable Assemblies feature an eight-lane electrical interface that transmits up to 28Gbps NRZ or 56Gbps PAM-4, up to 200Gbps or 400Gbps aggregate. QSFP-DD offers the same footprint as QSFP interconnects, making them backward compatible. The double density feature is an extended paddle card with two rows of high-speed context. QSFP-DD meets IEEE 802.3bj, InfiniBand EDR, and SAS 3.0 specifications, allowing these connectors and cable assemblies to function across a variety of next-generation technologies and applications 。

200G QSFP56  passive cable assembly products, based on 4X50G or 4X56G structure, this product can well meet the next generation of 200G switches, servers, routers and other products application needs. The QSFP56 cable assembly is optimised to reduce crosstalk and insertion loss and has excellent signal integrity, fully compliant with the next generation 200G Ethernet and InfiniBand HDR standards 。

- Product Features

- MEET SFF-8636&QSFP-DD MSA

- MEET IEEE802.3bj&IEEE802.3 cd

- Support I2C two - line string interface, easy to control

- Support for hot plugging

- Low crosstalk

- Eight-lane electrical interface transmits up to 28Gbps NRZ or 56Gbps PAM-4

- Applications :

- Telecommunications equipment

- Servers

- Routers

- Switches

- Central office

- Cellular infrastructure

- Multi-platform service systems

- Data networking equipment

- Servers

- Storage

- 外形图(Outline drawing):

| M.P/N | C.P/N | L(mm) | L1 | AWG |
| --- | --- | --- | --- | --- |
| QDD2QS56P-005301-002 | TBD | 500±15 | 100±10 | 30 |
| QDD2QS56P-010301-002 | TBD | 1000±25 | 200±10 | 30 |
| QDD2QS56P-015301-002 | TBD | 1500±30 |  | 30 |
| QDD2QS56P-020281-002 | TBD | 2000±35 |  | 28 |
| QDD2QS56P-025281-002 | TBD | 2500±35 |  | 28 |
| QDD2QS56P-030281-002 | TBD | 3000±45 | 200±10 | 28 |

- Wiring Diagram：

- Electrical  Performance：

  - Signal Integrity

| ITEM |  | REQUIREMENT | TEST CONDITION |
| --- | --- | --- | --- |
| Differential Impedance | Cable Impedance | 100+10/-Ω | Rise time of 25ps (20 % - 80 %). |
|  | Paddle Card Impedance | 100±10Ω |  |
|  | Cable Termination Impedance | 100 +10/-15Ω |  |
| Differential (Input/Output)Return loss SDD11/SDD22 |  | 16.5-2√f 0.05≤f﹤4.1 10.66-14log10(f/ 5.5) 4.1≤f≤19 Return_loss(f)≥ Where f is the frequency in GHz Return loss(f) is the return loss at frequency f | 10MHz≤f ≤26.5GHz |
| Differential to common-mode (Input/Output)Return loss SCD11/SCD22 |  | 22-(20/25.78)f 0.01≤f﹤12.89 15-(6/25.78)f 12.89≤f≤19 Return_loss(f)≥ Where f is the frequency in GHz Return_loss(f) is the Differential to common-mode return loss at frequency f | 10MHz≤f ≤26.5GHz |
| Common-mode to Common-mode (Input/Output)Return loss SCC11/SCC22 |  | Return_loss(f)≥2dB 0.2≤f≤19 Where f is the frequency in GHz Return_loss(f) is the common-mode to common-mode return loss at frequency f | 10MHz≤f ≤26.5GHz |
| Differential Insertion Loss (SDD21 Max.) |  | (Differential InsertionLoss Max. For TPa to TPb Excluding Test fixture ) Passive Cable: -17.16dB Min@13.28GHz. | 10MHz≤f ≤13.28GHz |
| Differential to common-mode Conversion Loss-Differential Insertion Loss(SCD21-SDD21) |  | 10 0.01≤f﹤12.89 27-(29/22)f 12.89≤f﹤15.7 6.3 15.7≤f≤19 Conversion _loss(f) – IL(f)≥ Where f is the frequency in GHz Conversion_loss(f ) is the cable assembly differential to common-mode conversion loss IL(f) is the cable assembly insertion loss | 10MHz≤f ≤26.5GHz |
| MDNEXT(multiple disturber near-end crosstalk) |  | ≥35dB @19GHz | 10MHz≤f ≤19GHz |

6.2 Other Electrical Performance

| ITEM | REQUIREMENT | TEST CONDITON |
| --- | --- | --- |
| Low Level Contact Resistance | 70milliohms Max. From initial. | EIA-364-23:Apply a maximum voltage of 20mV And a current of 100 mA. |
| Insulation Resistance | 10Mohm(Min.) | EIA364-21:AC 300V 1minute |
| Dielectric Withstanding Voltage | NO disruptive discharge. | EIA-364-20:Apply a voltage of 300 VDC for 1minute between adjacent terminals And between adjacent terminals and ground. |

- Environment Performance

| ITEM | REQUIREMENT | TEST CONDITON |
| --- | --- | --- |
| Operating Temp. Range | 0°C to +70°C | Cable operating temperature range. |
| Storage Temp. Range (in packed condition) | -40°C to +85°C | Cable storage temperature range in packed condition. |
| Thermal Cycling Non-Powered | No evidence of physical damage | EIA-364-32D, Method A, -25 to 90C, 100 cycles, 15 min. dwells |
| Salt Spraying | 48 hours salt spraying after shell corrosive area less than 5％. | EIA-364-26 |
| Mixed Flowing Gas | Pass electrical tests per 3.1 after stressing. (For connector only) | EIA-364-35 Class II,14 days. |
| Temp. Life | No evidence of physical damage | EIA-364-17C w/ RH, Damp heat 90℃ at 85% RH for 500 hours then return to ambient |
| Cable Cold Bend | 4H,No evidence of physical damage | Condition: -20℃±2℃, mandrel diameter is 6 times the cable diameter. |

- Mechanical and Physical Characteristics

| ITEM | REQUIREMENT | TEST CONDITON |
| --- | --- | --- |
| Vibration | Pass electrical tests per 3.1 after stressing. | Clamp & vibrate per EIA-364-28E, TC-VII, test condition letter – D, 15 minutes in X, Y & Z axis. |
| Cable Flex | No evidence of physical damage | Flex cable 180° for 20 cycles (±90° from nominal position) at 12 cycles per minute with a 1.0kg load applied to the cable jacket. Flex in the boot area 90º in each direction from vertical. Per EIA-364-41C |
| Cable Plug Retention in Cage | 90N Min. No evidence of physical damage | Force to be applied axially with no damage to cage. Per SFF 8661 Rev 2.1 Pull on cable jacket approximately 1 ft behind cable plug. No functional damage to cable plug below 90N. Per SFF-8432 Rev 5.0 |
| Cable Retention in Plug | 90N Min. No evidence of physical damage | Cable plug is fixtured with the bulk cable hanging vertically. A 90N axial load is applied (gradually) to the cable jacket and held for 1 minute. Per EIA-364-38B |
| Mechanical Shock | Pass electrical tests Per 3.1 after stressing. | Clamp and shock per EIA-364-27B, TC-G,3 times in 6 directions, 100g, 6ms. |
| Cable Plug Insertion | 40N Max.(QSFP56) 90N Max.(QSFP DD) | Per SFF8661 Rev 2.1 Per QSFP-DD Hardware Rev 5.0 |
| Cable plug Extraction | 30N Max. (QSFP56) 50N Max.(QSFP DD) | Place axial load on de-latch to de-latch plug.Per SFF8661 Rev 2.1 Measure without the aid of any cage kick-out springs. Place axial load on de-latch to de-latch plug. Per SFF-8432 Rev 5.0 |
| Durability | 50 cycles,No evidence of physical damage | EIA-364-09, perform plug &unplug cycles:Plug and receptacle mate rate: 250times/hour. 50times for QSFP28/SFP28 module (CONNECTOR TO PCB) |