400G QSFP-DD-OSFP Direct Attach Cable (DAC) Datasheet

General Description

400G QSFP-DD_OSFP passive copper cable assembly feature sixteen differential copper pairs, providing eight data transmission channels at speeds up to 56Gbps(PAM4) per channel, and meets 400G Ethernet and InfiniBand High Data Rate(HDR) requirements. Available in 26AWG and 30AWG wire gauges, this 400G copper cable assembly features low insertion loss and low crosstalk.

400G QSFP-DD_OSFP passive copper cable uses PAM4 signals for transmission, which doubles the rate. However, there are more stringent requirements for cable insertion loss. For detailed requirements, please see High Speed Characteristics.

Features and Benefits

- Compatible with IEEE 802.3cd

- Supports aggregate data rates of 400Gbps(PAM4)

- Optimized construction to minimize insertion loss and crosstalk

- Pull-to-release slide latch design

- Straight and break out assembly configurations available

- Customized cable braid termination limits EMI radiation

- Customizable EEPROM mapping for cable signature

- 26AWG and 30AWG cable

- 3.3V Power supply

- Temperature Range: 0~ 70 °C

- RoHs Compliant

Product Applications

- Switches, servers and routers

- Data Center networks

- Storage area networks

- High performance computing

- Telecommunication and wireless infrastructure

- Medical diagnostics and networking

- Test and measurement equipment

Industry Standards

- 400G Ethernet(IEEE 802.3cd)

- InfiniBand HDR

Pin Descriptions

QSFP-DD Pin Function Definition

| Pin | Logic | Symbol | Description |
| --- | --- | --- | --- |
| 1 |  | GND | Ground |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input |
| 4 |  | GND | Ground |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input |
| 7 |  | GND | Ground |
| 8 | LVTTL-I | ModSelL | Module Select |
| 9 | LVTTL-I | ResetL | Module Reset |
| 10 |  | Vcc Rx | +3.3V Power Supply Receiver |
| 11 | LVCMOS- I/O | SCL | 2-wire serial interface clock |
| 12 | LVCMOS- I/O | SDA | 2-wire serial interface data |
| 13 |  | GND | Ground |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |
| 16 |  | GND | Ground |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |
| 19 |  | GND | Ground |
| 20 |  | GND | Ground |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |
| 23 |  | GND | Ground |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |
| 26 |  | GND | Ground |
| 27 | LVTTL-O | ModPrsL | Module Present |
| 28 | LVTTL-O | IntL | Interrupt |
| 29 |  | Vcc Tx | +3.3V Power supply transmitter |
| 30 |  | Vcc1 | +3.3V Power supply |
| 31 | LVTTL-I | LPMode | Low Power Mode |
| 32 |  | GND | Ground |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input |
| 35 |  | GND | Ground |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input |
| 38 |  | GND | Ground |
| 39 |  | GND | Ground |
| 40 | CML-I | Tx6n | Transmitter Inverted Data Input |
| 41 | CML-I | Tx6p | Transmitter Non-Inverted Data Input |
| 42 |  | GND | Ground |
| 43 | CML-I | Tx8n | Transmitter Inverted Data Input |
| 44 | CML-I | Tx8p | Transmitter Non-Inverted Data Input |
| 45 |  | GND | Ground |
| 46 |  | Reserved |  |
| 47 |  | VS1 |  |
| 48 |  | VccRx1 | +3.3V Power supply |
| 49 |  | VS2 |  |
| 50 |  | VS3 |  |
| 51 |  | GND | Ground |
| 52 | CML-O | Rx7p | Receiver Non-Inverted Data Output |
| 53 | CML-O | Rx7n | Receiver Inverted Data Output |
| 54 |  | GND | Ground |
| 55 | CML-O | Rx5p | Receiver Non-Inverted Data Output |
| 56 | CML-O | Rx5n | Receiver Inverted Data Output |
| 57 |  | GND | Ground |
| 58 |  | GND | Ground |
| 59 | CML-O | Rx6n | Receiver Inverted Data Output |
| 60 | CML-O | Rx6p | Receiver Non-Inverted Data Output |
| 61 |  | GND | Ground |
| 62 | CML-O | Rx8n | Receiver Inverted Data Output |
| 63 | CML-O | Rx8p | Receiver Non-Inverted Data Output |
| 64 |  | GND | Ground |
| 65 |  | NC |  |
| 66 |  | Reserved |  |
| 67 |  | VccTx1 | +3.3V Power supply |
| 68 |  | VCC2 | +3.3V Power supply |
| 69 |  | Reserved |  |
| 70 |  | GND | Ground |
| 71 | CML-I | Tx7p | Transmitter Non-Inverted Data Input |
| 72 | CML-I | Tx7n | Transmitter Inverted Data Input |
| 73 |  | GND | Ground |
| 74 | CML-I | Tx5p | Transmitter Non-Inverted Data Input |
| 75 | CML-I | Tx5n | Transmitter Inverted Data Input |
| 76 |  | GND | Ground |

OSFP Pin Function Definition

| Pin |  | Logic | Symbol | Description |
| --- | --- | --- | --- | --- |
| 1 |  |  | GND | Ground |
| 2 |  | CML-I | Tx2p | Transmitter Non-Inverted Data Input |
| 3 |  | CML-I | Tx2n | Transmitter Inverted Data Input |
| 4 |  |  | GND | Ground |
| 5 |  | CML-I | Tx4p | Transmitter Non-Inverted Data Input |
| 6 |  | CML-I | Tx4n | Transmitter Inverted Data Input |
| 7 |  |  | GND | Ground |
| 8 |  | CML-I | Tx6p | Transmitter Non-Inverted Data Input |
| 9 |  | CML-I | Tx6n | Transmitter Inverted Data Input |
| 10 |  |  | GND | Ground |
| 11 |  | CML-I | Tx6p | Transmitter Non-Inverted Data Input |
| 12 |  | CML-I | Tx6n | Transmitter Inverted Data Input |
| 13 |  |  | GND | Ground |
| 14 |  | LVCMOS- | SCL | 2-wire serial interface clock |
|  |  | I/O |  |  |
| 15 |  |  | VCC | +3.3V Power supply |
| 16 |  |  | VCC | +3.3V Power supply |
| 17 |  |  | LPWn/PRSn | Low-Power Mode / Module Present |
| 18 |  |  | GND | Ground |
| 19 |  | CML-O | Rx7n | Receiver Inverted Data Output |
| 20 |  | CML-O | Rx7p | Receiver Non-Inverted Data Output |
| 21 |  |  | GND | Ground |
| 22 |  | CML-O | Rx5n | Receiver Inverted Data Output |
| 23 |  | CML-O | Rx5p | Receiver Non-Inverted Data Output |
| 24 |  |  | GND | Ground |
| 25 |  | CML-O | Rx3n | Receiver Inverted Data Output |
| 26 |  | CML-O | Rx3p | Receiver Non-Inverted Data Output |
| 27 |  |  | GND | Ground |
| 28 |  | CML-O | Rx1n | Receiver Inverted Data Output |
| 29 |  | CML-O | Rx1p | Receiver Non-Inverted Data Output |
| 30 |  |  | GND | Ground |
| 31 |  |  | GND | Ground |
| 32 | CML-O |  | Rx2p | Receiver Non-Inverted Data Output |
| 33 | CML-O |  | Rx2n | Receiver Inverted Data Output |
| 34 |  |  | GND | Ground |
| 35 | CML-O |  | Rx4p | Receiver Non-Inverted Data Output |
| 36 | CML-O |  | Rx4n | Receiver Inverted Data Output |
| 37 |  |  | GND | Ground |
| 38 | CML-O |  | Rx6p | Receiver Non-Inverted Data Output |
| 39 | CML-O |  | Rx6n | Receiver Inverted Data Output |
| 40 |  |  | GND | Ground |
| 41 | CML-O |  | Rx8p | Receiver Non-Inverted Data Output |
| 42 | CML-O |  | Rx8n | Receiver Inverted Data Output |
| 43 |  |  | GND | Ground |
| 44 |  |  | INT/RSTn | Module Interrupt / Module Reset |
| 45 |  |  | VCC | +3.3V Power supply |
| 46 |  |  | VCC | +3.3V Power supply |
| 47 | LVCMOS- |  | SDA | 2-wire serial interface data |
|  | I/O |  |  |  |
| 48 |  |  | GND | Ground |
| 49 | CML-I |  | Tx7n | Transmitter Inverted Data |
| 50 | CML-I |  | Tx7p | Input Transmitter Non-Inverted Data Input |
| 51 |  |  | GND | Ground |
| 52 | CML-I |  | Tx5n | Transmitter Inverted Data |
| 53 | CML-I |  | Tx5p | Input Transmitter Non-Inverted Data Input |
| 54 |  |  | GND | Ground |
| 55 | CML-I |  | Tx3n | Transmitter Inverted Data |
| 56 | CML-I |  | Tx3p | Input Transmitter Non-Inverted Data Input |
| 57 |  |  | GND | Ground |
| 58 | CML-I |  | Tx1n | Transmitter Inverted Data |
| 59 | CML-I |  | Tx1p | Input Transmitter Non-Inverted Data Input |
| 60 |  |  | GND | Ground |

General Product Characteristics

| QSFP-DD-OSFP DAC Specifications |  |
| --- | --- |
| Number of Lanes | Tx8 & Rx8 |
| Channel Data Rate | 53.125Gbps |
| Operating Temperature | 0 to + 70°C |
| Storage Temperature | -40 to + 85°C |
| Supply Voltage | 3.3 V nominal |
| Electrical Interface | 76pins edge connector(QSFP-DD) 60pins edge connector(OSFP) |
| Management Interface | Serial, I2C |

High Speed Characteristics

| Parameter | Symbol | Min | Typical | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Differential Impedance | TDR | 90 | 100 | 110 | Ώ |  |
| Insertion loss | SDD21 | -17.16 |  |  | dB | At 13.28 GHz |
| Differential Return Loss | SDD11 SDD22 |  |  | See 1 | dB | At 0.05 to 4.1 GHz |
|  |  |  |  | See 2 | dB | At 4.1 to 19 GHz |
| Common-mode to common-mode output return loss | SCC11 SCC22 |  |  | -2 | dB | At 0.2 to 19 GHz |
| Differential to common-mode return loss | SCD11 SCD22 |  |  | See 3 | dB | At 0.01 to 12.89 GHz |
|  |  |  |  | See 4 |  | At 12.89 to 19 GHz |
| Differential to common Mode Conversion Loss | SCD21- SDD21 |  |  | -10 | dB | At 0.01 to 12.89 GHz |
|  |  |  |  | See 5 |  | At 12.89 to 15.7 GHz |
|  |  |  |  | -6.3 |  | At 15.7 to 19 GHz |
| Notes: - Reflection Coefficient given by equation SDD11(dB) < -16.5 + 2 × SQRT(f ), with f in GHz - Reflection Coefficient given by equation SDD11(dB) < -10.66 + 14 × log10(f/5.5), with f in GHz - Reflection Coefficient given by equation SCD11(dB) < -22 + (20/25.78)*f, with f in GHz - Reflection Coefficient given by equation SCD11(dB) < -15 + (6/25.78)*f, with f in GHz - Reflection Coefficient given by equation SCD21(dB) < -27 + (29/22)*f, with f in GHz |  |  |  |  |  |  |

Mechanical Specifications

The connector is compatible with the QSFP-DD and SFF-8024（OSFP） specification.

| Length (m) | Cable AWG |
| --- | --- |
| 1 | 30 |
| 1.5 | 30 |
| 2 | 26 |
| 2.5 | 26 |
| 3 | 26 |

Regulatory Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1(>2000 Volts) |
| Electromagnetic Interference(EMI) | FCC Class B | Compliant with Standards |
|  | CENELEC EN55022 Class B |  |
|  | CISPR22 ITE Class B |  |
| RF Immunity(RFI) | IEC61000-4-3 | Typically Show no Measurable Effect from a 10V/m Field Swept from 80 to 1000MHz |
| RoHS Compliance | RoHS Directive 2011/65/EU and it's Amendment Directives （EU） 2015/863 | RoHS （EU）2015/863 compliant |
| REACH Compliance | REACH Regulation (EC) No 1907/2006 | REACH (EC) No 1907/2006 compliant |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City