# 100G QSFP28 to 2*50G QSFP28 Direct Attach Cable

## Features and Benefits

- QSFP28 conforms to the Small Form Factor SFF8665

- 4-Channel Full-Duplex Passive Copper Cable Transceiver

- Support data rates : 25.78Gb/s (per channel)

- Maximum aggregate data rate: 100Gb/s (4 x 25.78Gb/s)

- Meets or exceeds strict 100-Gigabit Ethernet (100GBASE-CR4) standards

- Complies with latest QSFP28 MSA (Multi-Source Agreement) SFF-8665 architecture

- Complies with IEEE 802.3bj and RoHS

- Compatible with all QSFP ports

- Pull-to-release slide latch design

## Product Applications

- Switches,servers and routers

- Data Center networks

- Storage area networks

- High performance computing

- Telecommunication and wireless infrastructure

- 100G Ethernet(IEEE 802.3bj)

- InfiniBand EDR

## General  Description

- The 100G QSFP28 cable assemblies are high performance, cost effective I/O solutions for LAN, HPC and SAN. The high speed cable assemblies meet and exceed 100 Gigabit Ethernet, InfiniBand EDR and temperature requirements for performance and reliability. The cables are compliant with SFF-8436 specifications and provide connectivity between devices using QSFP ports.

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Storage Ambient Temperature |  | -40 |  | +85 | °C |
| Operating Case Temperature | Tc | 0 |  | +70 | °C |
| Power Supply Voltage | VCC3 | 3.14 | 3.3 | 3.47 | V |
| Data Rate Per Lane |  | 1 |  | 25.78 | Gb/s |

## Performance Information

| Parameter | Symbol | Min | Typical | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Differential Impedance | TDR | 90 | 100 | 110 | Ώ |  |
| Insertion loss | SDD21 | -22.48 |  |  | d B | At 12.89GHz |
| Differential Return Loss | SDD11 SDD22 |  |  | See 1 | d B | At 0 05 to 4 1 GHz |
|  |  |  |  | See 2 | d B | At 4 1 to 19 GHz |
| Common-mode to common-mode output return loss | SCC11 SCC22 |  |  | -2 | d B | At 0.2 to 19 GHz |
| Differential to common-mode return loss | SCD11 SCD22 |  |  | See 3 | d B | At 0.01 to 12.89 GHz |
|  |  |  |  | See 4 |  | At 12.89 to 19 GHz |
| Differential to common Mode Conversion Loss | SCD21-IL |  |  | - 10 | d B | At 0 01 to 12 89 GHz |
|  |  |  |  | See 5 |  | At 12.89 to 15.7 GHz |
|  |  |  |  | -6.3 |  | At 15.7 to 19 GHz |
| Notes: 1. Reflection Coefficient given by equation SDD11(dB) < - 16.5 + 2 × SQRT(f ), with f in GHz 2. Reflection Coefficient given by equation SDD11(dB) < - 10.66 + 14 × log10(f/5.5), with f in GHz 3. Reflection Coefficient given by equation SCD11(dB) < -22 + (20/25.78)*f, with f in GHz 4. Reflection Coefficient given by equation SCD11(dB) < - 15 + (6/25.78)*f, with f in GHz 5. Reflection Coefficient given by equation SCD21(dB) < -27 + (29/22)*f, with f in GHz |  |  |  |  |  |  |

## Pin Descriptions

QSFP28 Pin Function Definition

| Pin | Logic | Symbol | Name/Description | Notes |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground | 1 |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 |  | GND | Ground | 1 |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 |  | GND | Ground | 1 |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | Vcc Rx | +3.3V Power Supply Receiver | 2 |
| 11 | LVCMOSI/O | SCL | 2-wire serial interface clock |  |
| 12 | LVCMOSI/O | SDA | 2-wire serial interface data |  |
| 13 |  | GND | Ground | 1 |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground | 1 |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground | 1 |
| 20 |  | GND | Ground | 1 |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground | 1 |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground | 1 |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | Vcc Tx | +3.3V Power supply transmitter | 2 |
| 30 |  | Vcc1 | +3.3V Power supply | 2 |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground | 1 |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input |  |
| 35 |  | GND | Ground | 1 |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input |  |
| 38 |  | GND | Ground | 1 |

Note:

- GND is the symbol for signal and supply (power) common for the QSFP+ module. All are common within the QSFP+ module and all module voltages are referenced to this potential unless otherwise noted. Connect these directly to the host board signal-common ground plane.

- Vcc Rx, Vcc1 and Vcc Tx are the receiver and transmitter power supplies and shall be applied concurrently. Requirements defined for the host side of the Host Edge Card Connector are listed in Table 6. Recommended host board power supply filtering is shown in Figure 4. Vcc Rx Vcc1 and Vcc Tx may be internally connected within the QSFP+ Module module in any combination. The connector pins are each

rated for a maximum current of 500 mA.

Wire Diagram

Mechanical Specifications

The connector is compatible with the SFF-8436 specification.

| Length (m) | CableAWG |
| --- | --- |
| 1 | 30 |
| 2 | 30 |
| 3 | 30 |
| 4 | 26 |
| 5 | 26 |

## RegulatoryCompliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1(>2000 Volts) |
| Electromagnetic Interference(EMI) | FCC Class B | Compliant with Standards |
|  | CENELEC EN55022 Class B |  |
|  | CISPR22 ITE Class B |  |
| RF Immunity(RFI) | IEC61000-4-3 | Typically Show no Measurable Effect from a 10V/m Field Swept from 80 to 1000MHz |
| RoHS Compliance | RoHS Directive 2011/65/EU and it's Amendment Directives (EU) 2015/863 | RoHS (EU) 2015/863 compliant |
| REACH Compliance | REACH Regulation (EC) No 1907/2006 | REACH (EC) No 1907/2006 compliant |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City