40Gbps QSFP+ To 4x 10G SFP+

Passive High Speed Cable Specification

| Signature | (Drafted By) | （Approved by ） | （ Customer Approved by ） |
| --- | --- | --- | --- |
|  | DANIEL | GRACE |  |
|  | 2025-07-21 | 2025-07-21 |  |

- Description：

QSFP + to 4SFP + Passive Direct Copper is a cost-effective, high-speed interconnect solution that allows cables to connect QSFP + and SFP + switches and network devices without the need to upgrade an entire data center or storage array. Enables Customers to Interconnect Between 40G and 10G Devices (NIC / HBA / CNA, Switch Devices, and Servers).

- Features：

•  Protocol agnostic support of 40GbE, QDR InfiniBand, SAS & Fibre Channel

•  10Gbps transfer rate per SFP+ channel (40 Gb/s aggregate)

•  Compliant with SFF-8436 / SFF-8431

•  Compliant with IEEE 802.3ba/ Infiniband QDR specifications
•  Enhanced EMI/EMC performance
•  Supports serial ID functionality thru EEPROM

•  Passive cable assembly supports distances up to 7meters
•  30AWG to 24AWG cable sizes available
•  RoHS compliant

- Applications :

•  Switches / Routers / HBAs/SAN,NIC cards
•  Server & Storage Devices
•  Data Center Networking
•  Fiber Channel
•  InfiniBand QDR/DDR 
•  10Gbs/40Gbs Ethernet

- Outline drawing :

- Wiring Diagram：

- Electrical Performance：

  - （Signal Integrity）

| （ITEM） |  | （REQUIREMENT） |  |  |  |  | （TEST CONDITION） |
| --- | --- | --- | --- | --- | --- | --- | --- |
| （Differe ntial Impedan ce） | Cable Impedance | 105+5/-5Ω |  |  |  |  | Rise time of 35ps (20 % - 80 %). |
|  | Paddle Card Impedance | 100±10Ω |  |  |  |  |  |
|  | Cable Termination Impedance | 100±15Ω |  |  |  |  |  |
| [Differential (Input/Output)Return loss SDD11/SDD22] |  | Return loss(f)≥ 10 0.01≤f﹤4.1 6.3-13log10(f/ 5.5) 4.1≤f≤11.1 Where f is the frequency in GHz Return loss(f) is the return loss at frequency f |  |  |  |  | 0.01GHz≤f≤11.1GH z SFF-8431 Rev.4.1 |
| [Differential Insertion Loss (SDD21 Max.)] |  | (Differential InsertionLoss Max. For TPa to TPb Excluding Test fixture ) |  |  |  |  | 10MHz≤f ≤5GHz |
|  |  | F AWG | 0.6GHz | 1.25GHz | 2.5GHz | 5.0GHz |  |
|  |  | 30AWG(1m)Max. | 2dB | 3dB | 4.5dB | 7.5dB |  |
|  |  | 30 AWG(2m)Max. | 4dB | 5dB | 7dB | 10dB |  |
|  |  | 28AWG（3m)Max. | 4dB | 5.5dB | 7.5 dB | 12dB |  |
|  |  | 26AWG(5m)Max. | 5.5dB | 7dB | 10dB | 16.0dB |  |
|  |  | 24AWG(7m&10m )Max. | 6.5dB | 10dB | 14dB | 21dB |  |
| [MDNEXT(multiple disturber near-end crosstalk)] |  | ≥26dB |  |  |  |  | 10MHz≤f ≤5GHz |
| [Insertion Loss Deviation] |  | -0.7-0.2*10-3f ≤ ILD ≤ 0.7+0.2*10-3f (f is the frequency in MHz), |  |  |  |  | 10MHz≤f ≤5GHz |

  - （Other Electrical Performance）

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| [Low Level Contact Resistance] | 70milliohms Max. From initial. | EIA-364-23:Apply a maximum voltage of 20mV And a current of 100 mA. |
| Insulation Resistance | 10Mohm(Min.) | EIA364-21:AC 300V 1minute |
| [Dielectric Withstanding Voltage] | DC 500V 1 minute disruptive discharge. | EIA-364-20:Apply a voltage of 500 VDC for 1minute between adjacent terminals And between adjacent terminals and ground. |

- （Environment Performance）

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| [Operating Temp. Range] | -20°C to +75°C | Cable operating temperature range. |
| [Storage Temp. Range (in packed condition)] | -25°C to +65°C | Cable storage temperature range in packed condition. |
| [Thermal Cycling Non-Powered] | No evidence of physical damage | EIA-364-32D, Method A, -25 to 90C, 100 cycles, 15 min. dwells |
| [Salt Spraying] | 48 hours salt spraying after shell corrosive area less than 5％. | EIA-364-26 |
| Mixed Flowing Gas | Pass electrical tests per 3.1 after stressing. (For connector only) | EIA-364-35 Class II,14 days. |
| Temp. Life | No evidence of physical damage | EIA-364-17C w/ RH, Damp heat 90℃ at 85% RH for 500 hours then return to ambient |
| Cable Cold Bend | 4H,No evidence of physical damage | Condition: -20℃±2℃, mandrel diameter is 6 times the cable diameter. |

- Mechanical and Physical Characteristics

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| Vibration | Pass electrical tests per 3.1 after stressing. | Clamp & vibrate per EIA-364-28E, TC-VII, test condition letter – D, 15 minutes in X, Y & Z axis. |
| Cable Flex | No evidence of physical damage | Flex cable 180° for 20 cycles (±90° from nominal position) at 12 cycles per minute with a 1.0kg load applied to the cable jacket. Flex in the boot area 90º in each direction from vertical. Per EIA-364-41C |
| Cable Plug Retention in Cage | 90N Min. No evidence of physical damage | Force to be applied axially with no damage to cage. Per SFF 8661 Rev 2.1 Pull on cable jacket approximately 1 ft behind cable plug. No functional damage to cable plug below 90N. Per SFF-8432 Rev 5.0 |
| Cable Retention in Plug | 90N Min. No evidence of physical damage | Cable plug is fixtured with the bulk cable hanging vertically. A 90N axial load is applied (gradually) to the cable jacket and held for 1 minute. Per EIA-364-38B |
| Mechanical Shock | Pass electrical tests Per 3.1 after stressing. | Clamp and shock per EIA-364-27B, TC-G,3 times in 6 directions, 100g, 6ms. |
| Cable Plug Insertion | 40N Max.(QSFP+) 18N Max.(SFP+) | Per SFF8432 Rev 5.0. |
| Cable plug Extraction | 30N Max. (QSFP28) 12.5N Max. (SFP28) | Measure without the aid of any cage kick-out springs. Place axial load on de-latch to de-latch plug. Per SFF-8432 Rev 5.0. |
| Durability | 50 cycles,No evidence of physical damage | EIA-364-09, perform plug &unplug cycles:Plug and receptacle mate rate: 250times/hour. 50times for QSFP28/SFP28 module (CONNECTOR TO PCB) |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City