25Gbps SFP28 Passive High Speed Cable Specification

| Signature | (Drafted By) | （Approved by ） | （ Customer Approved by ） |
| --- | --- | --- | --- |

- Description ：

SFP28 direct attach cable is a low-cost, high-efficiency high-speed interconnection solution with a single channel maximum transmission rate of up to 25Gbps. It is suitable for stacking and connection of short-distance switch equipment.

- Features：

• Single channel data rate of 25Gbps

• Compliant with SFP28 MSA and SFF-8432 compliant

• Compliant with IEEE 802.3 and FCoE

• Enhanced EMI / EMC performance

• Support serial ID function through EEPROM

• Supports cable customization up to 5 meters

• 30AWG to 26AWG cable available

• RoHS and halogen-free options

- Applications :

• Switch / Router / HBA

• Servers and storage devices

• Data center network

• Base station / BBU / RRU

• Fiber Channel

• 25Gbs Ethernet

- Outline drawing :

- Wiring Diagram：

- Electrical Performance：

  - Signal Integrity

| （ITEM） |  | （REQUIREMENT） |  |  |  |  |  |  | （TEST CONDITION） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| （Differe ntial Impedan ce） | Cable Impedance | 105+5/-10Ω |  |  |  |  |  |  | Rise time of 25ps (20 % - 80 %). |
|  | Paddle Card Impedance | 100±10Ω |  |  |  |  |  |  |  |
|  | Cable Termination Impedance | 100±15Ω |  |  |  |  |  |  |  |
| [Differential (Input/Output)Return loss SDD11/SDD22] |  | 16.5-2√f 0.05≤f﹤4.1 Return_loss(f)≥ 10.66-14log10(f/ 5.5) 4.1≤f≤19 Where f is the frequency in GHz Return loss(f) is the return loss at frequency f |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
| [Differential to common-mode (Input/Output)Return loss SCD11/SCD22] |  | 22-(20/25.78)f 0.01≤f﹤12.89 Return_loss(f)≥ 15-(6/25.78)f 12.89≤f≤19 Where f is the frequency in GHz Return_loss(f) is the Differential to common-mode return loss at frequency f |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
| [Common-mode to Common-mode (Input/Output)Return loss SCC11/SCC22] |  | Return_loss(f)≥2dB 0.2≤f≤19 Where f is the frequency in GHz Return_loss(f) is the common-mode to common-mode return loss at frequency f |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
| [Differential Insertion Loss (SDD21 Max.)] |  | (Differential InsertionLoss Max. For TPa to TPb Excluding Test fixture ) |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
|  |  | F AWG | 1.25GHz | 2.5GHz | 5.0GHz | 7.0GHz | 10Ghz | 12.89Ghz |  |
|  |  | 30(1m) | 4.5dB | 5.4dB | 6.3dB | 7.5dB | 8.5dB | 10.5dB |  |

|  | Max. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30/28( 3m)Ma x. | 7.5dB | 9.5dB | 12.2dB | 14.8dB | 18.0dB | 21.5dB |  |
|  | 26(3m) Max. | 5.7dB | 7.2dB | 9.9 dB | 11.9dB | 14.1dB | 16.5dB |  |
|  | 26/25( 5m)Ma x. | 7.8dB | 10.0dB | 13.5dB | 16.0dB | 19.0dB | 22.0dB |  |
| Differential to common-mode Conversion Loss-Differential Insertion Loss(SCD21-SDD21) | 10 0.01≤f﹤12.89 Conversion _loss(f) – IL(f)≥ 27-(29/22)f 12.89≤f﹤15.7 6.3 15.7≤f≤19 Where f is the frequency in GHz Conversion_loss(f ) is the cable assembly differential to common-mode conversion loss IL(f) is the cable assembly insertion loss |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
| [MDNEXT(multiple disturber near-end crosstalk)] | ≥26dB @12.89GHz |  |  |  |  |  |  | 10MHz≤f ≤19GHz |
| [Intra Skew] | 15ps/m, |  |  |  |  |  |  | 10MHz≤f ≤19GHz |

  - Other Electrical Performance

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| [Low Level Contact Resistance] | 70milliohms Max. From initial. | EIA-364-23:Apply a maximum voltage of 20mV And a current of 100 mA. |
| Insulation Resistance | 10Mohm(Min.) | EIA364-21:AC 300V 1minute |
| [Dielectric Withstanding Voltage] | NO disruptive discharge. | EIA-364-20:Apply a voltage of 300 VDC for 1minute between adjacent terminals And between adjacent terminals and ground. |

- Environment Performance:

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| [Operating Temp. Range] | -40°C to +80°C | Cable operating temperature range. |
| [Storage Temp. Range (in packed condition)] | -40°C to +80°C | Cable storage temperature range in packed condition. |
| [Thermal Cycling Non-Powered] | No evidence of physical damage | EIA-364-32D, Method A, -25 to 90C, 100 cycles, 15 min. dwells |
| [Salt Spraying] | 48 hours salt spraying after shell corrosive area less than 5％. | EIA-364-26 |
| Mixed Flowing Gas | Pass electrical tests per 3.1 after stressing. (For connector only) | EIA-364-35 Class II,14 days. |
| Temp. Life | No evidence of physical damage | EIA-364-17C w/ RH, Damp heat 90℃ at 85% RH for 500 hours then return to ambient |
| Cable Cold Bend | 4H,No evidence of physical damage | Condition: -20℃±2℃, mandrel diameter is 6 times the cable diameter. |

- Mechanical and Physical Characteristics

| （ITEM） | （REQUIREMENT） | （TEST CONDITON） |
| --- | --- | --- |
| Vibration | Pass electrical tests per 3.1 after stressing. | Clamp & vibrate per EIA-364-28E, TC-VII, test condition letter – D, 15 minutes in X, Y & Z axis. |
| Twist | No evidence of physical | Twist cable 180° (±90° from nominal |

| Twist | damage | position) for 100 cycles at 30 cycles per minute with a 0.5kg load applied to the cable jacket. Clamp position: 300mm |
| --- | --- | --- |
| Cable Flex | No evidence of physical damage | Flex cable 180° for 20 cycles (±90° from nominal position) at 12 cycles per minute with a 1.0kg load applied to the cable jacket. Flex in the boot area 90º in each direction from vertical. Per EIA-364-41C |
| Cable Plug Retention in Cage | 90N Min. No evidence of physical damage | Force to be applied axially with no damage to cage. Per SFF 8661 Rev 2.1 Pull on cable jacket approximately 1 ft behind cable plug. No functional damage to cable plug below 90N. Per SFF-8432 Rev 5.0 |
| Cable Retention in Plug | 90N Min. No evidence of physical damage | Cable plug is fixtured with the bulk cable hanging vertically. A 90N axial load is applied (gradually) to the cable jacket and held for 1 minute. Per EIA-364-38B |
| Mechanical Shock | Pass electrical tests Per 3.1 after stressing. | Clamp and shock per EIA-364-27B, TC-G,3 times in 6 directions, 100g, 6ms. |
| Cable Plug Insertion | 18N Max.(SFP28) | Per SFF-8432 Rev 5.0 |
| Cable plug Extraction | 12.5N Max. (SFP28) | Measure without the aid of any cage kick-out springs. Place axial load on de-latch to de-latch plug. Per SFF-8432 Rev 5.0 |
| Durability | 50 cycles,No evidence of physical damage | EIA-364-09, perform plug &unplug cycles:Plug and receptacle mate rate: 250times/hour. 50times for QSFP28/SFP28 module (CONNECTOR TO PCB) |