10G SFP+ Direct Attach Cable

General Description

SFP+ Direct Attach Cables are compliant with the SFF-8431, SFF-8432 and SFF-8472 specifications. Various choices of wire gauge are available from 30 to 24 AWG with various choices of cable length (up to 7m).

PRODUCT FEATURES

- Compliant with SFF-8431, 8432 and 8472.

- Up to 10.3125Gbps data rate per channel

- Up to 7m transmission

- Operating temperature:0℃ to +70℃

- Single 3.3V power supply

- RoHS compliant

BENEFITS

- Cost-effective copper solution

- Lowest total system power solution

- Lowest total system EMI solution

- Optimized design for Signal Integrity

PRODUCT APPLICATIONS

- Ethernet for 2x40/100GBASE-SR4

- HPC Interconnects

- Proprietary Interconnections

GENERAL PRODUCT CHARACTERISTICS

| Number of Lanes | Tx & Rx |
| --- | --- |
| Channel Data Rate | 10.3125 Gbps |
| Operating Temperature | 0 to + 70°C |
| Storage Temperature | -40 to + 85°C |
| Supply Voltage | 3.3 V nominal |
| Electrical Interface | 20 pins edge connector |
| Management Interface | Serial, I2C |

HIGH SPEED CHARACTERISTICS

| Parameter | Symbol | Min | Typ | Max | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| - Differential Impedance | Zd | 90 | 100 | 110 | Ω |  |
| - Differential Input Return Loss | SDDXX | <-12+2* SQRT (f) with f in GHz |  |  | dB | 0.01~4.1GHz |
|  |  | <-6.3+13* Log10/(f/5.5) with f in GHz |  |  | dB | 4.1~11.1GHz |
| - Common Mode Output Return Loss | SCCXX | < -7+1.6*f with f in GHz |  |  | dB | 0.01~2.5GHz |
|  |  |  |  | -3 | dB | 2.5~11.1GHz |
| - Difference Waveform Distortion Penalty | dWDPc |  |  | 6.75 | dB |  |
| - VMA Loss | L |  |  | 4.4 | dB |  |
| - VMA Loss to Crosstalk Ratio | VCR | 32.5 |  |  | dB |  |

MECHANICAL SPECIFICATIONS

The connector is compatible with the SFF-8432 specification.

| Length (m) | Cable AWG |
| --- | --- |
| 1 | 30 |
| 3 | 30 |
| 5 | 24 |
| 7 | 24 |

Pin Function Definition

| Pin | Logic | Symbol | Description |
| --- | --- | --- | --- |
| 1 |  | VeeT | Module Transmitter Ground |
| 2 | LVTTL-O | Tx_Fault | Module Transmitter Fault |
| 3 | LVTTL-I | Tx_Disable | Transmitter disable; Turns off transmitter laser output |
| 4 | LVTTL-I/O | SDA | 2-wire Serial Interface Data Line (Same as MOD-DEF2 in INF-8074i) |
| 5 | LVTTL-I/O | SCL | 2-wire Serial Interface Clock (Same as MOD-DEF1 in INF-8074i) |
| 6 |  | Mod_ABS | Module Absent, connected to VeeT or VeeR in the module |
| 7 | LVTTL-I | RS0 | Rate Select 0, optionally controls SFP+ module receiver |
| 8 | LVTTL-O | Rx_LOS | Receiver Loss of Signal Indication (In FC designated as Rx_LOS and in Ethernet designated as Signal Detect) |
| 9 | LVTTL-I | RS1 | Rate Select 1, optionally controls SFP+ module transmitter |
| 10 |  | VeeR | Module Receiver Ground |
| 11 |  | VeeR | Module Receiver Ground |
| 12 | CML-O | RD- | Receiver Inverted Data Output |
| 13 | CML-O | RD+ | Receiver Non-Inverted Data Output |
| 14 |  | VeeR | Module Receiver Ground |
| 15 |  | VccR | Module Receiver 3.3 V Supply |
| 16 |  | VccT | Module Transmitter 3.3 V Supply |
| 17 |  | VeeT | Module Transmitter Ground |
| 18 | CML-I | TD+ | Transmitter Non-Inverted Data Input |
| 19 | CML-I | TD- | Transmitter Inverted Data Input |
| 20 |  | VeeT | Module Transmitter Ground |

Regulatory Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1(>2000 Volts) |
| Electromagnetic Interference(EMI) | FCC Class B | Compliant with Standards |
|  | CENELEC EN55022 Class B |  |
|  | CISPR22 ITE Class B |  |
| RF Immunity(RFI) | IEC61000-4-3 | Typically Show no Measurable Effect from a 10V/m Field Swept from 80 to 1000MHz |
| RoHS Compliance | RoHS Directive 2011/65/EU and it's Amendment Directives 6/6 | RoHS 6/6 compliant |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,

Donghu High-tech Zone, Wuhan City