800G OSFP to 800G OSFP Active Copper Cable (ACC)

PRODUCT FEATURES

- Hot Pluggable OSFP form factor with finned top heat sink design

- Maximum Aggregate Data Rate: 800Gb/s (8 x 100G/Per Lane)

- Based on 8-channels of 100G-PAM4 modulation

- Built-in EQ Linear Driver chip

- 5meter length max

- 1.5 Watts max per end

- SFF-8665 compliant

- CMIS compliant I2C management interface

- Commercial case temperature range of 0°C to 70°C

- IEEE 802.3ck 400GBase-CR4, 200Gbase-CR2 and 100GBase-CR1 standards

- EIA-364

- DAC Firmware supports both InfiniBand and Ethernet

PRODUCT APPLICATIONS

- Infiniband 2x400Gb/s Quantum-2

- InfiniBand Spectrum-4 Ethernet switch-to-switch and switch-to-DGX-H100

- Data storage and communication industry

- Switch / router / HBA

- Enterprise network

- Data Center Network

PRODUCT P/N

| PN | Data Rate | Temp | Length (meter) | Remark |
| --- | --- | --- | --- | --- |
| JH-800G-OSFP-A1-AOC | 800Gbps | 0-70℃ | 3~5 |  |

Description

JH-800G-OSFP-A1-AOC is an 800G OSFP (Octal Small Formfactor Pluggable) to 800G OSFP Active Copper Cable (ACC). It’s with an equalizer integrated circuit to extend the length while maintaining very low-latency and very low-power at 1.5 Watts per end. The DAC supports both InfiniBand and Ethernet and is automatically enabled depending on the protocol of the switch attached to.

JH-800G-OSFP-A1-AOC cable assemblies is compliant with the OSFP-MSA and IEEE 802.3ck, it’s a high performance & cost effective I/O solutions for LAN, HPC and SAN. The high speed cable assemblies meet and exceed 800Gigabit Ethernet, InfiniBand EDR /HDR and temperature requirements for performance and reliability.

Absolute Maximum Ratings

１

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Storage Temperature | Ts | -20 | 85 | ºC |
| Case Operating Temperature | Tc | 0 | 70 | ºC |
| Humidity (non-condensing) | Rh | 5 | 95 | % |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | Tc | 0 |  | 70 | ºC |
| Baud Rate per Lane (PAM4) | fd |  | 53.125 |  | GBaud/s |
| Humidity | Rh | 5 |  | 85 | % |

Electrical Specifications

| Parameter | Min | Typ | Max | Units | Note |
| --- | --- | --- | --- | --- | --- |
| Characteristic impedance | 90 | 100 | 110 | Ω |  |
| Time propagation delay | -- | -- | 4.5 | ns/m | Informative Reference |

SI Requirements

| Test Items | Specification | Notes |
| --- | --- | --- |
| SDD21&SDD12 | (Please reference only, when the length is more than 1.5m) ≤19.75 dB Min. @26.56 GHz; ≥ 11.0 dB max. @26.56GHz; | From 0.01 GHz to 26.56GHz |
| ERL | Minimum cable assembly ERL(*) : ≥ 8.25dB | / |
| SCD12-SDD12 SCD21-SDD21 | ≥ 10 0.05GHz≤f<12.89GHz ≥ 14-0.3108f 12.89GHz≤f≤40GHz | ( up to 40GHz) |

Electrical Pinout

Electrical pin list and description

| Pin | Symbol | Description | Pin | Symbol | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Ground | 31 | GND | Ground |
| 2 | Tx2p | Transmitter Non-Inverted Data Input | 32 | Rx2p | Receiver Non-Inverted Data Output |
| 3 | Tx2n | Transmitter Inverted Data Input | 33 | Rx2n | Receiver Inverted Data Output |
| 4 | GND | Ground | 34 | GND | Grounds |
| 5 | Tx4p | Transmitter Non-Inverted Data Input | 35 | Rx4p | Receiver Non-Inverted Data Output |
| 6 | Tx4n | Transmitter Inverted Data Input | 36 | Rx4n | Receiver Inverted Data Output |
| 7 | GND | Ground | 37 | GND | Ground |
| 8 | Tx6p | Transmitter Non-Inverted Data Input | 38 | Rx6p | Receiver Non-Inverted Data Output |
| 9 | Tx6n | Transmitter Inverted Data Input | 39 | Rx6n | Receiver Inverted Data Output |

| 10 | GND | Ground | 40 | GND | Ground |
| --- | --- | --- | --- | --- | --- |
| 11 | Tx8p | Transmitter Non-Inverted Data Input | 41 | Rx8p | Receiver Non-Inverted Data Output |
| 12 | Tx8n | Transmitter Inverted Data Input | 42 | Rx8n | Receiver Inverted Data Output |
| 13 | GND | Ground | 43 | GND | Ground |
| 14 | SCL | 2-wire serial interface clock | 44 | INT / RSTn | Module Interrupt / Module Reset |
| 15 | VCC | +3.3V Power | 45 | VCC | +3.3V Power |
| 16 | VCC | +3.3V Power | 46 | VCC | +3.3V Power |
| 17 | LPWn / PRSn | Low-Power Mode / Module Present | 47 | SDA | 2-wire Serial interface data |
| 18 | GND | Ground | 48 | GND | Ground |
| 19 | Rx7n | Receiver Inverted Data Output | 49 | Tx7n | Transmitter Inverted Data Input |
| 20 | Rx7p | Receiver Non-Inverted Data Output | 50 | Tx7p | Transmitter Non-Inverted Data Input |
| 21 | GND | Ground | 51 | GND | Ground |
| 22 | Rx5n | Receiver Inverted Data Output | 52 | Tx5n | Transmitter Inverted Data Input |
| 23 | Rx5p | Receiver Non-Inverted Data Output | 53 | Tx5p | Transmitter Non-Inverted Data Input |
| 24 | GND | Ground | 54 | GND | Ground |
| 25 | Rx3n | Receiver Inverted Data Output | 55 | Tx3n | Transmitter Inverted Data Input |
| 26 | Rx3p | Receiver Non-Inverted Data Output | 56 | Tx3p | Transmitter Non-Inverted Data Input |
| 27 | GND | Ground | 57 | GND | Ground |
| 28 | Rx1n | Receiver Inverted Data Output | 58 | Tx1n | Transmitter Inverted Data Input |
| 29 | Rx1p | Receiver Non-Inverted Data Output | 59 | Tx1p | Transmitter Non-Inverted Data Input |
| 30 | GND | Ground | 60 | GND | Ground |

Mechanical Dimensions （mm）

| Length (m) | Cable AWG |
| --- | --- |
| 0.5 | 30 |
| 1 | 30 |
| 1.5 | 28 |
| 2 | 28 |
| 2.5 | 28 |
| 3 | 28 |
| 4 | 26 |
| 5 | 26 |

Regulatory    Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1(>2000 Volts) |
| Electromagnetic Interference( EMI) | FCC Class B | Compliant withStandards |
|  | CENELEC EN5 5 0 2 2 Class B |  |
|  | CISPR2 2 ITE Class B |  |
| RF Immunity( RFI) | IEC61000-4-3 | Typically Show no Measurable Effect from a10V/m Field Swept from80 to 1000MHz |
| RoHS Compliance | RoHS Directive 2011/65/EU andit's AmendmentDirectives(EU)2015/863 | RoHS (EU) 2015/863 compliant |

Contact:

Wuhan Jiuhong Optics Technology Co., Ltd.

TEL: 86-13476016116

E-mail：grace@jh-optics.com

Web：www.jh-optics.com

Address：

Building 1, 4th Floor, Yunqi Free Trade Innovation Port, Donghu Comprehensive Bonded Zone,Donghu High-tech Zone, Wuhan City