Product Specification

200G QSFP56 to 2*100G QSFP56 AOC Transceivers

FEATURES

- Supports IBTA InfiniBand HDR

- 200GbE to 2x100GbE data rate

- 50 Gb/s PAM4 modulation per lane

- SFF-8665 compliant QSFP56 port

- Support SFF-8636 or CMIS 5.0 compliant I2C management

- Single 3.3V power supply

- 4.5W power dissipation each end, with retiming

- Built-in DSP (QSFP56)

- Operating case temp Commercial:

- Hot pluggable

- RoHS compliant

- 0°C to +70 °C

APPLICATIONS

- 200 Gb/s InfiniBand HDR systems

- 200GBASE-SR4 and 100GBASE-SR2 compliant

Ordering information

| Part Number | Product Description |
| --- | --- |
| JH-200G-QSFP56-A2-AOC | 200G QSFP56 to 2*100G QSFP56 AOC Transceivers, C-TEMP xx=01, cable length=1m xx=03, cable length=3m xx=05, cable length=5m xx=0A, cable length=10m xx=0F, cable length=15m xx=19, cable length=25m xx=32, cable length=50m xx=64, cable length=100m |

ABSOLUTE MAXIMUM RATINGS

| Parameter | Symbol | Conditions | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TStorage |  | -40 | +85 | °C |
| Relative Humidity | RH |  | 0 | +85 | % |

RECOMMENDED OPERATING CONDITIONS (T=25°C, unless noted)

| Parameter | Symbol | Conditions | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Case Temperature | TC |  | 0 |  | 70 | °C |
| Power Supply Voltage | VCC |  | 3.135 | 3.3 | 3.465 | V |
| Power Consumption |  |  |  |  | 4.5 | W |
| Signaling Rate each Channel |  |  |  | 26.5625 |  | Gbps |
| Data Rate Accuracy |  |  | -100 |  | 100 | ppm |
| Error Bit Rate |  |  |  |  | 2.4E-4 |  |

Notes: PRBS31Q@26.5625Gbd PAM4

PIN ASSIGNMENT

PIN DESCRIPTION  - 200G QSFP56 END

| PIN | Logic | Symbol | Name/Description | Note |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground |  |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 |  | GND | Ground |  |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |  |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output |  |
| 7 |  | GND | Ground |  |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver |  |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data |  |
| 13 |  | GND | Ground |  |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | CML-O | Rx3n | Receiver Inverted Data Output |  |
| 16 |  | GND | Ground |  |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground |  |
| 20 |  | GND | Ground |  |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground |  |
| 24 | CML-O | Rx4n | Receiver Inverted Data Output |  |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 |  | GND | Ground |  |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter |  |
| 30 |  | Vcc1 | +3.3 V Power Supply |  |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground |  |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Output |  |
| 35 |  | GND | Ground |  |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output |  |
| 38 |  | GND | Ground |  |

PIN DESCRIPTION  - 100G QSFP56 END

| PIN | Logic | Symbol | Name/Description | Note |
| --- | --- | --- | --- | --- |
| 1 |  | GND | Ground |  |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |  |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |  |
| 4 |  | GND | Ground |  |
| 5 | CML-I | Tx4n | NC |  |
| 6 | CML-I | Tx4p | NC |  |
| 7 |  | GND | Ground |  |
| 8 | LVTTL-I | ModSelL | Module Select |  |
| 9 | LVTTL-I | ResetL | Module Reset |  |
| 10 |  | VccRx | ﹢3.3V Power Supply Receiver |  |
| 11 | LVCMOS-I/O | SCL | 2-Wire Serial Interface Clock |  |
| 12 | LVCMOS-I/O | SDA | 2-Wire Serial Interface Data |  |
| 13 |  | GND | Ground |  |
| 14 | CML-O | Rx3p | NC |  |
| 15 | CML-O | Rx3n | NC |  |
| 16 |  | GND | Ground |  |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | CML-O | Rx1n | Receiver Inverted Data Output |  |
| 19 |  | GND | Ground |  |
| 20 |  | GND | Ground |  |
| 21 | CML-O | Rx2n | Receiver Inverted Data Output |  |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 |  | GND | Ground |  |
| 24 | CML-O | Rx4n | NC |  |
| 25 | CML-O | Rx4p | NC |  |
| 26 |  | GND | Ground |  |
| 27 | LVTTL-O | ModPrsL | Module Present |  |
| 28 | LVTTL-O | IntL | Interrupt |  |
| 29 |  | VccTx | +3.3 V Power Supply transmitter |  |
| 30 |  | Vcc1 | +3.3 V Power Supply |  |
| 31 | LVTTL-I | LPMode | Low Power Mode |  |
| 32 |  | GND | Ground |  |
| 33 | CML-I | Tx3p | NC |  |
| 34 | CML-I | Tx3n | NC |  |
| 35 |  | GND | Ground |  |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Output |  |
| 38 |  | GND | Ground |  |

# Electrical Interface

OUTLINE DIMENSIONS

Unit mm

Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |