Product Specification

100Gb/s QSFP28 to 2xQSFP28 Active Optical Cable

- Features

- Supports 100G to 25G Ethernet interoperability

- Aggregate 2 discrete QSFP28 50G channels into single parallel QSFP28 100G interface

- Maximum link length of 70m links on OM3 multimode fiber or 100m links on OM4 multimode Fiber

- Electrically hot-pluggable

- Electrical interface compliant to QSFP28 connector and QSFP28 connectors

- Case operating temperature range: 0°C to 70°C

- Applications

- Network Switch Manufacturers supporting 100/25G

- Server Manufacturers supporting 100/25G

- Data Centers with 25G requirements with Aggregation Architectures

Ordering information

| Part Number | Product Description |
| --- | --- |
| JH-100G-QSFP28-A2-AOC | 100G QSFP28 to 2xQSFP28 Active Optical Cable Transceivers, C-TEMP XX=01，cable length=1m XX=03，cable length=3m XX=05，cable length=5m XX=0A，cable length=10m XX=19，cable length=25m XX=32，cable length=50m XX=64，cable length=100m |

- General Description

This product QSFP28 to 2xQSFP28 active optic cables are a high performance, low power consumption, long reach interconnect solution supporting 100G to 25G Ethernet.

100G QSFP28

- Pin Function Definition

Pin Definition- QSFP28

| PIN | Logic | Symbol | Name/Description |
| --- | --- | --- | --- |
| 1 |  | GND | Ground |
| 2 | CML-I | Tx2n | Transmitter Inverted Data Input |
| 3 | CML-I | Tx2p | Transmitter Non-Inverted Data output |
| 4 |  | GND | Ground |
| 5 | CML-I | Tx4n | Transmitter Inverted Data Input |
| 6 | CML-I | Tx4p | Transmitter Non-Inverted Data output |
| 7 |  | GND | Ground |
| 8 | LVTLL-I | ModSelL | Module Select |
| 9 | LVTLL-I | ResetL | Module Reset |
| 10 |  | VccRx | +3.3V Power Supply Receiver |
| 11 | LVCMOSS-I/O | SCL | 2-Wire Serial Interface Clock |
| 12 | LVCMOSS-I/O | SDA | 2-Wire Serial Interface Data |
| 13 |  | GND | Ground |
| 14 | CML-O | Rx3p | Receiver Non-Inverted Data output |
| 15 | CML-O | Rx3n | Receiver Inverted Data Input |
| 16 |  | GND | Ground |
| 17 | CML-O | Rx1p | Receiver Non-Inverted Data output |
| 18 | CML-O | Rx1n | Receiver Inverted Data Input |
| 19 |  | GND | Ground |
| 20 |  | GND | Ground |
| 21 | CML-O | Rx2n | Receiver Inverted Data Input |
| 22 | CML-O | Rx2p | Receiver Non-Inverted Data output |
| 23 |  | GND | Ground |
| 24 | CML-O | Rx4n | Receiver Inverted Data Input |
| 25 | CML-O | Rx4p | Receiver Non-Inverted Data output |
| 26 |  | GND | Ground |
| 27 | LVTLL-O | ModPrsL | Module Present |
| 28 | LVTLL-O | IntL | Interrupt |
| 29 |  | VccTx | +3.3V Power Supply transmitter |
| 30 |  | Vccl | +3.3V Power Supply |
| 31 | LVTLL-I | LPMode | Low Power Mode |
| 32 |  | GND | Ground |
| 33 | CML-I | Tx3p | Transmitter Non-Inverted Data output |
| 34 | CML-I | Tx3n | Transmitter Inverted Data Input |
| 35 |  | GND | Ground |
| 36 | CML-I | Tx1p | Transmitter Non-Inverted Data output |
| 37 | CML-I | Tx1n | Transmitter Inverted Data Input |
| 38 |  | GND | Ground |

- Recommended Power Supply Filter

- Performance Specifications

6.1 Absolute Maximum Ratings

It has to be noted that the operation in excess of any individual absolute maximum ratings might cause permanent damage to this module

| Parameter | Symbol | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| Storage Temperature | Tstg | -40 | +85 | °C |
| Operating relative humidity (Non- condensing) | RH | 0 | 85 | % |
| Input Voltage |  | Vcc-0.3 | Vcc+0.3 | V |
| Supply Voltage | Vcc | -0.3 | 3.6 | dBm |

6.2 Recommended Operating Conditions and Power Supply Requirements

| Parameter | Symbol | Min. | Type | Max. | Unit | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | Tc | 0 |  | +70 | ℃ |  |
| Power Supply Voltage | Vcc | +3.135 | 3.3 | +3.465 | V |  |
| Data Rate | BR |  | 103.12 |  | Gb/s | QSFP28 Bit Rate |
| Power Consumption Max (per cable) | P |  |  | 2.5 | W | End-QSFP28 |
| Supply Current (per cable) | ICC |  |  | 0.76 | A | End-QSFP28 |
| Data Rate Accuracy | - | -100 |  | 100 | ppm |  |
| Control Input Voltage High | - | - |  | 2 | V |  |
| Control Input Voltage Low | - | 0.8 |  | - | V |  |
| Link Distance(OM4 MMF) | D | 0 |  | 100 | m |  |

  - Electrical Specification

| Parameter | Symbol | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Differential input impedance | Zin | 90 | 100 | 110 | ohm |
| Differential Output impedance | Zout | 90 | 100 | 110 | ohm |
| Differential input voltage amplitude | ΔVin | 300 |  | 1100 | mVp-p |
| Differential output voltage amplitude | ΔVout | 500 |  | 800 | mVp-p |
| Pre-FEC Bit Error Ratio | BR |  |  | 5E-5 |  |
| Post-FEC Bit Error Ratio | BR |  |  | 1E-12 |  |
| Input Logic Level High | VIH | 2.0 |  | VCC | V |
| Input Logic Level Low | VIL | 0 |  | 0.8 | V |
| Output Logic Level High | VOH | VCC-0.5 |  | VCC | V |
| Output Logic Level Low | VOL | 0 |  | 0.4 | V |

- Mechanical Dimensions

100G QSFP28

(Unit: mm)

- Digital Diagnostic Monitor Accuracy

The following characteristics are defined over recommended operating conditions

| Parameter | Accuracy | Unit |
| --- | --- | --- |
| Internally measured transceiver temperature | +/-3 | deg.C |
| Internally measured transceiver supply voltage | +/-3 | % |
| Measured Tx bias current | +/-10 | % |
| Measured Tx output power | +/-3 | dB |
| Measured Rx received average optical power | +/-3 | dB |