1.25G SFP 1310nm 10KM LC DDM

Features：

■Up to 1.25Gbps data rate

■Duplex LC receptacle optical interface compliant

■Single +3.3V power supply

■DDM function implemented

■External calibration
■Hot-pluggable

■Receiver Loss of Signal Output

■AC coupling of PECL signals

■Serial ID module on MOD(0-2)

■International Class 1 laser safety certified

■Transmitter disable input

■Operating temperature range: 0℃~+70℃ or  -40~85℃

■CWDM module Operating temperature range:0℃~+70℃

■RoHS Compliant

Applications：

■Gigabit Ethernet

■Gigabit Fiber Channel

■Switch to switch interface

■Switched backplane applications

Standard:

■Compliant with SFP MSA (INF-8074i)

■Compliant with SFF-8472 v9.3

■Compliant with IEEE802.3z Gigabit Ethernet

■Compliant with ITU-T G.695

■Compliant with FC-PI v2.0

Absolute Maximum Ratings

| Parameter | Symbol | Uint. | Min | Max |
| --- | --- | --- | --- | --- |
| Storage Temperature Range | Ts | °C | -40 | +85 |
| Relative Humidity | RH | % | 5 | 95 |
| Power supply Voltage | Vcc | V | -0.5 | 4 |

Recommended Operating Conditions

| Parameter | Symbol | Uint | Min | Typ | Max |
| --- | --- | --- | --- | --- | --- |
| OperatingTemperature | Tc | °C | 0/-40 |  | 70/85 |
| Power Supply Voltage | Vcc | V | 3.135 | 3.3 | 3.465 |
| Data Rate | - | Gb/s | - | 1.25/1.0625 | - |

Specifications (tested under recommended operating conditions, unless otherwise noted)

| Parameter |  |  | Symbol | Uint | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electrical Characteristics |  |  |  |  |  |  |  |  |
| Supply Current |  | Tx Section | Icc | mA | - | - | 300 | 1 |
|  |  | Rx Section |  |  |  |  |  |  |
| Single Ended Data Input Swing |  |  |  | mV | 150 |  | 1100 |  |
| Single Ended Data Output Swing |  |  |  | mV | 300 |  | 600 |  |
| TX_fault /LOS output (TTL) |  |  | VOH | V | 2.0 |  | Vcc |  |
|  |  |  | VOL |  | 0 |  | 0.8 |  |
| TX_disable input (TTL) |  |  | VOH | V | 2.0 |  | Vcc |  |
|  |  |  | VOL |  | 0 |  | 0.8 |  |
| Optical transmitter Characteristics |  |  |  |  |  |  |  |  |
| Launch Optical Power |  |  | Po | dBm | -9 | -6 | -3 | 10km 1310nm FP-LD |
| Center Wavelength Range |  |  | λc | nm | 1270 | 1310 | 1355 | FP-LD |
| Extinction Ratio |  |  | EX | dB | 9 |  |  |  |
| Spectral Width |  |  | ∆λ | nm |  |  | 4 | FP-LD |
| Side Mode Suppression Ratio |  |  | SMSR | dB | 30 |  |  | DFB-LD |
| Contributed Total Jitter added at TP2 |  |  | TJ | UI |  |  | 0.284 | 3 |
| Relative Intensity Noise |  |  | RIN | dB/H z |  |  | -120 | 4 |
| Eye Diagram |  |  | Complies with IEEE802.3z eye masks when filtered |  |  |  |  |  |
| Dispersion Penalty |  |  |  | dB |  |  | 1 |  |
|  |  |  |  |  |  |  | 2 | CWDM |
| Optical Rise/Fall Time |  |  | Trise/Tfal | PS |  |  | 260 | 5 |
| Optical receiver Characteristics |  |  |  |  |  |  |  |  |
| Receiver Sensitivity |  |  | S | dBm |  |  | -22 | 1310nm 10km |
| Overload Input Optical Power |  |  | P in | dBm | -3 |  |  | 10km |
| LOS | Optical Dessert |  |  | dB |  |  | S |  |
|  | Optical Assert |  |  |  | -35 |  |  |  |
|  |  |  |  |  | -45 |  |  |  |
| LOS Hysteresis |  |  |  | dB | 0.5 | 3 | 5 |  |

Note1. The supply current includes SFP module’s supply current and test board working current.

Note2. The CWDM transmitter center wavelengths “λ”are: 1471, 1491, 1511, 1531, 1551, 1571, 1591, 1611nm

Note3. TP refers to the compliance point specified in IEEE802.3z, section 38.2.1.

Note4. RIN is the laser noise, integrated over a specified bandwidth, measured relative to average optical power with 12dB

return loss. For multimode application, the RIN is better than -117dB/Hz.

Note5. Optical transition time is the time interval required for the rising or falling edge of an optical pulse to transition

between the 20% and 80% amplitudes relative to the logical 1 and 0 levels

Note6. Measured with a PRBS 2 23 -1 test pattern, @1.25Gb/s, EX=10dB, BER<10 -12 .

Note7. The LOS Hysteresis to minimize “chatter” on the output line. In principle, Hysteresis alone does

not guarantee chatter-free operation

EEPROM Serial ID Memory Contents

Accessing Serial ID Memory uses the 2 wire address 1010000X(A0). Memory Contents of Serial ID areshown in Table 2

Table 2Serial ID Memory Contents

| Data Address | Size (Bytes) | Name of Field | Contents(Hex) | Description |
| --- | --- | --- | --- | --- |
| BASE ID FIELDS |  |  |  |  |
| 0 | 1 | Identifier | 03 | SFP |
| 1 | 1 | Ext. Identifier | 04 | SFP function is defined by serial ID only |
| 2 | 1 | Connector | 07 | LC Connector |
| 3~10 | 8 | Transceiver |  | Transceiver Codes |
| 11 | 1 | Encoding | 03 | NRZ |
| 12 | 1 | BR, Nominal | 0C | 1.25Gbit/s |
| 13 | 1 | Reserved | 00 |  |
| 14 | 1 | Length (9µm) km |  | Transceiver transmit distance |
| 15 16 | 1 | Length (9µm) 100m |  |  |
| 16 | 1 | Length (50µm) 10m |  |  |
| 17 | 1 | Length(62.5µm)10m |  |  |
| 18 | 1 | Length (Copper) | 00 | Not compliant |
| 19 | 1 | Reserved | 00 |  |
| 20~35 | 16 | Vendor name |  | (ASCII) |
| 36 | 1 | Reserved | 00 |  |
| 37~39 | 3 | Vendor OUI | 00 00 00 |  |
| 40~55 | 16 | Vendor PN |  | Transceiver part number |
| 56~59 | 4 | Vendor rev | 20 20 20 20 |  |
| 60~61 | 2 | Wavelength |  | Transceiver wavelength |
| 62 | 1 | Reserved | 00 |  |
| 63 | 1 | CC_BASE | Check Sum (Variable) | Check code for Base ID Fields |
| EXTENDED ID FIELDS |  |  |  |  |
| 64~65 | 2 | Options | 00 1A | TX_DISABLE, TX_FAULT and Loss of Signal implemented. |
| 66 | 1 | BR,max | 00 |  |
| 67 | 1 | BR,min | 00 |  |
| 68~83 | 16 | Vendor SN | 42 30 30 39 38 32 32 20 20 20 20 20 20 20 20 20 | Serial Number of transceiver (ASCII).For example“B009822” |
| 84~91 | 8 | Date code | 30 32 31 30 30 35 20 20 | Manufactory date code. For example “021005”. |
| 92 | 1 | Diagnostic Monitoring Type | 58 | Digital diagnostic monitoring implemented, “externally calibrated” isimplemented, RX measurement type is “Average Power”. |
| 93 | 1 | Enhanced Options | B0 | Optional Alarm/Warning flags implemented for all monitored quantities, Optional Soft TX_FAULT monitoring implemented, Optional Soft RX_LOS monitoring implemented. |
| 94 | 1 | SFF_8472 Compliance | 01 | Includes functionality described in Rev9.3 SFF-8472. |
| 95 | 1 | CC_EXT | Check Sum (Variable) | Check sum for Extended ID Field |
| VENDOR SPECIFIC ID FIELDS |  |  |  |  |
| 96~127 | 32 | Vendor Specific | Read only | Depends on customer information |
| 128~25555 | 128 | Reserved | Read only | Filled by zero |

Diagnostic Monitor Functions

Diagnostic Monitor Functions interface uses the 2 wire address 1010001X (A2). Memory contents ofDiagnostic Monitor Functions are shown in Table 3

Table 3 Memory contents of Diagnostic Monitor Function

| Data Address | Field Size (bytes) | Name | Contents and Description |
| --- | --- | --- | --- |
| Alarm and Warning Thresholds |  |  |  |
| 00-01 | 2 | Temperature High Alarm | Set to 85 o C |
| 02-03 | 2 | Temperature Low Alarm | Set to -5 o C |
| 04-05 | 2 | Temperature High Warning | Set to 75 o C |
| 06-07 | 2 | Temperature Low Warning | Set to 0 o C |
| 08-09 | 2 | Vcc High Alarm | Set to 3.6 V |
| 10-11 | 2 | Vcc Low Alarm | Set to 3.0 V |
| 12-13 | 2 | Vcc High Warning | Set to 3.5 V |
| 14-15 | 2 | Vcc Low Warning | Set to 3.1 V |
| 16-17 | 2 | Bias High Alarm | 2×IBias (25 o C ) +20 |
| 18-19 | 2 | Bias Low Alarm | 25%×IBias (25 o C) |
| 20-21 | 2 | Bias High Warning | 2×IBias (25 o C ) +10 |
| 22-23 | 2 | Bias Low Warning | 50%×IBias (25 o C |
| 24-25 | 2 | TX Power High Alarm | Manufacture measurement plus 2dB |
| 26-27 | 2 | TX Power Low Alarm | Manufacture measurement minus 2dB |
| 28-29 | 2 | TX Power High Warning | Manufacture measurement plus 1dB |
| 30-31 | 2 | TX Power Low Warning | Manufacture measurement minus 1 dB |
| 32-33 | 2 | RX Power High Alarm | Maximum input optical power |
| 34-35 | 2 | RX Power Low Alarm | Minimum input optical power |
| 36-37 | 2 | RX Power High Warning | Maximum input power minus 3dB |
| 38-39 | 2 | RX Power Low Warning | Manufacture measurement plus 3dB |
| 40-55 | 16 | Reserved |  |
| Calibration Constants |  |  |  |
| 56-59 | 4 | RX Power Calibration Data4 | Single precision floating-point numbers (variousvalues at each device) |
| 60-63 | 4 | RX Power Calibration Data3 |  |
| 64-67 | 4 | RX Power Calibration Data2 | Single precision floating-point numbers (variousvalues at each device) |
| 68-71 | 4 | RX Power Calibration Data1 |  |
| 72-75 | 4 | RX Power Calibration Data0 |  |
| 76-77 | 2 | Bias Calibration Data1 | 00 01 (fixed) |
| 78-79 | 2 | Bias Calibration Data0 | 00 00 (fixed) |
| 80-81 | 2 | TX Power Calibration Data1 | 00 01 (fixed) |
| 82-83 | 2 | TX Power Calibration Data0 | 00 00 (fixed) |
| 84-85 | 2 | Temperature Calibration Data1 | 00 01 (fixed) |
| 86-87 | 2 | Temperature Calibration Data0 | 00 00 (fixed) |
| 88-89 | 2 | Vcc Calibration Data1 | 00 01 (fixed) |
| 90-91 | 2 | Vcc Calibration Data0 | 00 00 (fixed) |
| 92-94 | 3 | Reserved | 00 00 00 (fixed) |
| 95 | 1 | Check Sum | Checksum of bytes 0-94 |
| Real Time Diagnostic Monitor Interface |  |  |  |
| 96-97 | 2 | Measured Temperature | Yield to a 16-bit A/D value (see Table 3.1) |
| 98-99 | 2 | Measured Vcc | Yield a 16-bit A/D value (see Table 3.1) |
| 100-101 | 2 | Measured Bias | Yield a 16-bit A/D value (see Table 3.1) |
| 102-103 | 2 | Measured TX Power | Yield a 16-bit A/D value (see Table 3.1) |
| 104-105 | 2 | Measured RX Power | Yield a 16-bit A/D value (see Table 3.1) |
| 106-109 | 4 | Reserved |  |
| 110 | 1 | Logic Status | See Table 3.2 |
| 111 | 1 | AD Conversion Updates | See Table 3.2 |
| 112-129 | 8 | Alarm and Warning Flags | See Table 3.3 |
| Vendor Specific |  |  |  |
| 120-127 | 8 | Vendor Specific | Don’t Access |
| 128-247 | 120 | User writable EEPROM |  |
| 245-255 | 8 | Vendor Specific | Don’t Access |

The measured values located at bytes 96-105(in the 2 wire address 0xA2) are raw A/D values (16-bit

integers) of transceiver temperature, supply voltage, laser bias current, laser optical output power and

received power. All the measured values are “Externally Calibrated”, and then it is necessary to convert raw

A/D values to real world units by the manner as shown in Table 3.1

Table 3.1 Real Time Diagnostic Monitor Values

| Byte | Name | Description |
| --- | --- | --- |
| 96 | Temperature MSB | Internally measured transceiver temperature. Comply with ExternalCalibration of SFF-8472. |
| 97 | Temperature LSB |  |
| 98 | Vcc MSB | Internally measured supply voltage. Comply with External Calibration ofSFF-8472 |
| 99 | Vcc LSB |  |
| 100 | Laser Bias MSB | Measured Laser bias current. Comply with External Calibration ofSFF-8472 |
| 101 | Laser Bias LSB |  |
| 102 | Tx Power MSB | Measured Tx power. Comply with External Calibration of SFF-8472. |
| 103 | Tx Power LSB |  |
| 104 | Rx Power MSB | Measured Tx power. Comply with External Calibration of SFF-8472 |
| 105 | Rx Power LSB |  |

This transceiver implements two optional status bytes, “Logic States” at byte 110(0xA2)” and “A/D

Updated” at byte 111(0xA2) as shown in Table 3.2. “A/D Updated” status bits allow the user to verify if an

update from the analog-digital conversion has occurred of the measured values, temperature, Vcc, laser bias,

Tx power and Rx power. The user writes the byte to 0x00. Once a conversion is completed for a given value,

its bit will change to ‘1’.

Table 3.2 Logic Status and AD Conversion Updates

| Byte | Bit | Name | Description |
| --- | --- | --- | --- |
| 110 | 7 | Tx Disable State | Optional digital State of the Tx Disable input pin. |
| 110 | 6 | Soft Tx Disable Control | Not supported (set to 0). |
| 110 | 5 | Reserved | Set to 0. |
| 110 | 4 | Rx Rate Select State | Not supported (set to 1). |
| 110 | 3 | Soft Rate Select Control | Not supported (set to 0). |
| 110 | 2 | Tx Fault | Optional digital state of the Tx Fault output pin |
| 110 | 1 | LOS | Optional digital state of the LOS output pin |
| 110 | 0 | Power on Logic | Bit will be 0 when the analog monitoring is active |
| 111 | 7 | Temp A/D Valid | Indicates A/D value in Bytes 96/97 is valid. |
| 111 | 6 | Vcc A/D Valid | Indicates A/D value in Bytes 98/99 is valid. |
| 111 | 5 | Laser Bias A/D Valid | Indicates A/D value in Bytes 100/101 is valid. |
| 111 | 4 | Tx Power A/D Valid | Indicates A/D value in Bytes 102/103 is valid. |
| 111 | 3 | Rx Power A/D Valid | Indicates A/D value in Bytes 104/105 is valid. |
| 111 | 2 | Reserved | Set to 0 |
| 111 | 1 | Reserved | Set to 0 |
| 111 | 0 | Reserved | Set to 0 |

Each of the measured values has a corresponding high alarm, low alarm, high warning and low warning

threshold level at location 00-39(x0A2) written as the data format of a corresponding valued shown in Table

3.3.Alarm and warning flags at bytes 112-119(0xA2) are defined as follows.

(1) Alarm flags indicate conditions likely to result (or have resulted) in link failure and cause for

immediate action.

(2) Warning flags indicate conditions outside the guaranteed operating specification of transceiver but not

necessarily causes of immediate link failures

Table 3.3 Alarm and Warning Flags

| Byte | Bit | Name | Description |
| --- | --- | --- | --- |
| 112 | 7 | Temperature High Alarm | Set when temperature monitor value exceeds high alarmlevel. |
| 112 | 6 | Temperature Low Alarm | Set when temperature monitor value exceeds low alarm level |
| 112 | 5 | Vcc High Alarm | Set when Vcc monitor value exceeds high alarm level. |
| 112 | 4 | Vcc Low Alarm | Set when Vcc monitor value exceeds Low alarm level |
| 112 | 3 | Laser Bias High Alarm | Set when laser bias monitor value exceeds high alarm level. |
| 112 | 2 | Laser Bias Low Alarm | Set when laser bias monitor value exceeds low alarm level. |
| 112 | 1 | Tx Power High Alarm | Set when Tx power monitor value exceeds high alarm level |
| 112 | 0 | Tx Power Low Alarm | Set when Tx power monitor value exceeds low alarm level. |
| 113 | 7 | Rx Power High Alarm | Set when Rx power monitor value exceeds high alarm level |
| 113 | 6 | Rx Power Low Alarm | Set when Rx power monitor value exceeds low alarm level |
| 113 | 5-0 | Reserved | All bits set to 0 |
| 114 | 7-0 | Reserved | All bits set to 0 |
| 115 | 7-0 | Reserved | All bits set to 0 |
| 116 | 7 | Temperature High warning | Set when temperature monitor value exceeds high warning level. |
| 116 | 6 | Temperature Low warning | Set when temperature monitor value exceeds low warning level |
| 116 | 5 | Vcc High warning | Set when Vcc monitor value exceeds high warning level. |
| 116 | 4 | Vcc Low warning | Set when Vcc monitor value exceeds Low warning level. |
| 116 | 3 | Laser Bias High warning | Set when laser bias monitor value exceeds high warning level. |
| 116 | 2 | Laser Bias Low warning | Set when laser bias monitor value exceeds low warning level. |
| 116 | 1 | Tx Power High warning | Set when Tx power monitor value exceeds high warning level |
| 116 | 0 | Tx Power Low warning | Set when Tx power monitor value exceeds low warning level. |
| 117 | 7 | Rx Power High warning | Set when Rx power monitor value exceeds high warning level |
| 117 | 6 | Rx Power Low warning | Set when Rx power monitor value exceeds low warning level |
| 117 | 5-0 | Reserved | All bits set to 0. |
| 118 | 7-0 | Reserved | All bits set to 0. |
| 119 | 7-0 | Reserved | All bits set to 0. |

Pin Description

Pin Connections

| Pin | Name | Function/Description | Engagement order | Notes |
| --- | --- | --- | --- | --- |
| 1 | VeeT | Transmitter Ground | 1 |  |
| 2 | TX Fault | Transmitter Fault Indication | 3 | 1 |
| 3 | TX Disable | Transmitter Disable-Module disables on high or open | 3 | 2 |
| 4 | MOD_DEF2 | Module Definition 2-Two wire serial ID interface | 3 | 3 |
| 5 | MOD_DEF1 | Module Definition 1-Two wire serial ID interface | 3 | 3 |
| 6 | MOD_DEF0 | Module Definition 0-Two wire serial ID interface | 3 | 3 |
| 7 | Rate Select | Not Connected | 3 |  |
| 8 | LOS | Loss of Signal | 3 | 4 |
| 9 | VeeR | Receiver Ground | 1 |  |
| 10 | VeeR | Receiver Ground | 1 |  |
| 11 | VeeR | Receiver Ground | 1 |  |
| 12 | RD- | Inverse Received Data out | 3 | 5 |
| 13 | RD+ | Received Data out | 3 | 5 |
| 14 | VeeR | Receiver Ground | 1 |  |
| 15 | VccR | Receiver Power —— +3.3V±5% | 2 | 6 |
| 16 | VccT | Transmitter Power —— +3.3 V±5% | 2 | 6 |
| 17 | VeeT | Transmitter Ground | 1 |  |
| 18 | TD+ | Transmitter Data In | 3 | 7 |
| 19 | TD- | Inverse Transmitter Data In | 3 | 7 |
| 20 | VeeT | Transmitter Ground | 1 |  |

Note1. TX Fault is open collector/drain output which should be pulled up externally with a 4.7K – 10KΩresistor on the host

board to supply <VccT+0.3V or VccR+0.3V. When high, this output indicates a laser fault of some kind. Low indicates normal

operation. In the low state, the output will be pulled to <0.8V.

Note2. TX Disable input is used to shut down the laser output per the state table below. It is pulled up within the module with

a 4.7 – 10K resistor.

Low (0 – 0.8V): Transmitter on

Between (0.8V and 2V): Undefined

High (2.0 – VccT): Transmitter Disabled

Open : Transmitter Disabled

Note3. Mod-Def 0, 1, 2. These are the module definition pins. They should be pulled up with a 4.7 - 10K resistor on the host

board to supply less than VccT+0.3V or VccR+0.3V.

Mod-Def 0 is grounded by the module to indicate that the module is present.

Mod-Def 1 is clock line of two wire serial interface for optional serial ID.

Mod-Def 2 is data line of two wire serial interface for optional serial ID.

Note4. LOS (Loss of signal) is an open collector/drain output which should be pulled up externally with a 4.7 – 10K resistor

on the host board to supply <VccT+0.3V or VccR+0.3V. When high, this output indicates the received optical power is below

the worst case receiver sensitivity (as defined by the standard in use). Low indicates normal operation. In the low state, the

output will be pulled to <0.8V.

Note5. RD-/+: These are the differential receiver outputs. They are AC coupled 100Ω differential lines which should be

terminated with 100Ω differential at the user SERDES. The AC coupling is done inside the module and thus not required on

the host board.

Note6. VccR and VccT are the receiver and transmitter power supplies. They are defined as 3.3V±5% at the SFP connector

pin. The in-rush current will typically be no more than 30mA above steady state supply current after 500ns.

Note7. TD-/+: These are the differential transmitter inputs. They are AC coupled differential lines with 100Ω differential

termination inside the module. The AC coupling is done inside the module and is thus not required on host board.

Block Diagram

Required Host Board Components

The MSA power supply noise rejection filter is required on the host PCB to meet data sheet performance.The MSA filter incorporates an inductor which should be rated 400mADC and 1Ωserial resistance or better.It should not be replaced with a ferrite. The required filter is illustrated in Figure 3. The MSA also specifiesthat 4.7K to 10KΩ pull-up resistors for TX_FAULT, LOS, and MOD_DEF0,1,2 are required on the hostPCB. Figure is the suggested transceiver/host interface

Typical Application Circuit

Package Outline

PCB Layout Recommendation

Regulatory Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the ElectricalPins | MIL-STD-883E Method 3015.7 | Class 1 (>1.5kV) – Human Body Model |
| Electrostatic Discharge (ESD) Immunity | IEC61000-4-2 | Class 2(>4.0kV) |
| Electromagnetic Interference (EMI) | CISPR22 ITE Class B FCC Class B CENELEC EN55022 VCCI Class 1 | Compliant with standard |
| Immunity | IEC61000-4-3 Class2 | Typically show no measurable effect from a 3V/m field swept from 80 to 1000MHz applied tothe transceiver without a chassis enclosure. |
| Eye Safety | FDA CDRH 21-CFR 1040 Class 1 NO.:0322110-02 |  |
|  | UL NO.:E239070 |  |