2.5Gbps SFP Bi-Directional Transceiver,120km Reach

1550nm TX / 1490 nm RX

Features

Dual data-rate of 2.5Gbps/2.488Gbps operation

1550nm DFB laser and APD photo detector for 120km transmission Compliant with SFP MSA and SFF-8472 with duplex LC receptacle Digital Diagnostic Monitoring:

Internal Calibration or External Calibration Compatible with SONET OC-24-LR-1 Compatible with RoHS

+3.3V single power supply Operating case temperature:

Standard :	0 to +70°C

Applications Gigabit Ethernet Fiber Channel

Switch to Switch interface Switched backplane applications Router/Server interface

Other optical transmission systems

Description

The SFP-BIDI transceivers are high performance, cost effective modules supporting dual data-rate of 2.5Gbps/2.488Gbps and 120km transmission distance with SMF.

The transceiver consists of three sections: a DFB laser transmitter, a APD photodiode integrated with a trans-impedance preamplifier (TIA) and MCU control unit. All modules satisfy class I laser safety requirements.

The transceivers are compatible with SFP Multi-Source Agreement (MSA) and SFF-8472. For further

MCU

Vcc

Temp

ADC/DAC

EEPROM

APD+TIA

Post Amp

LD

LD Driver

information, please refer to SFP MSA.

TD+/-

TxFault TxDis

LOS RD+/-

SCL SDA

Absolute Maximum Ratings

Table 1 - Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | -0.5 | 4.5 | V |
| Storage Temperature | Ts | -40 | +85 | °C |
| Operating Humidity | - | 5 | 85 | % |

Recommended Operating Conditions

Table 2 - Recommended Operating Conditions

| Parameter |  |  | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature |  | Standard | Tc | 0 |  | +70 | °C |
| Power Supply Voltage |  |  | Vcc | 3.13 | 3.3 | 3.47 | V |
| Power Supply Current |  |  | Icc |  |  | 300 | mA |
| Data Rate | Gigabit Ethernet |  |  |  | 2.5 |  | Gbps |
|  | Fiber Channel |  |  |  | 2.488 |  |  |

Optical and Electrical Characteristics

2.5G T1550nm/R1490nm 120km:	:	(DFB and APD,	120km Reach)

Table 3 - Optical and Electrical Characteristics

| Parameter |  | Symbol |  | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |
| Centre Wavelength |  |  | λc | 1530 | 1550 | 1570 | nm |  |
| Spectral Width (-20dB) |  |  | ∆λ |  |  | 1 | nm |  |
| Side Mode Suppression Ratio |  |  | SMSR | 30 |  |  | dB |  |
| Average Output Power |  |  | Pout | 0 |  | 5 | dBm | 1 |
| Extinction Ratio |  |  | ER | 9 |  |  | dB |  |
| Optical Rise/Fall Time (20%~80%) |  |  | tr/tf |  |  | 0.26 | ns |  |
| Data Input Swing Differential |  |  | VIN | 400 |  | 1800 | mV | 2 |
| Input Differential Impedance |  |  | ZIN | 90 | 100 | 110 | Ω |  |
| TX Disable | Disable |  |  | 2.0 |  | Vcc | V |  |
|  | Enable |  |  | 0 |  | 0.8 | V |  |
| TX Fault | Fault |  |  | 2.0 |  | Vcc | V |  |
|  | Normal |  |  | 0 |  | 0.8 | V |  |
| Receiver |  |  |  |  |  |  |  |  |
| Centre Wavelength |  |  | λc | 1470 | 1490 | 1510 | nm |  |
| Receiver Sensitivity |  |  |  |  |  | -31 | dBm | 3 |
| Receiver Overload |  |  |  | -3 |  |  | dBm | 3 |
| LOS De-Assert |  |  | LOSD |  |  | -24 | dBm |  |
| LOS Assert |  |  | LOSA | -30 |  |  | dBm |  |
| LOS Hysteresis |  |  |  | 1 |  | 4 | dB |  |
| Data Output Swing Differential |  |  | Vout | 400 |  | 1800 | mV | 4 |
| LOS |  |  | High | 2.0 |  | Vcc | V |  |
|  |  |  | Low |  |  | 0.8 | V |  |

Timing and Electrical

Table 4 - Timing and Electrical

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Tx Disable Negate Time | t_on |  |  | 1 | ms |
| Tx Disable Assert Time | t_off |  |  | 10 | µs |
| Time To Initialize, including Reset of Tx Fault | t_init |  |  | 300 | ms |
| Tx Fault Assert Time | t_fault |  |  | 100 | µs |
| Tx Disable To Reset | t_reset | 10 |  |  | µs |
| LOS Assert Time | t_loss_on |  |  | 100 | µs |
| LOS De-assert Time | t_loss_off |  |  | 100 | µs |
| Serial ID Clock Rate | f_serial_clock |  |  | 400 | KHz |
| MOD_DEF (0:2)-High | VH | 2 |  | Vcc | V |
| MOD_DEF (0:2)-Low | VL |  |  | 0.8 | V |

Diagnostics

Table 5 – Diagnostics Specification

| Parameter | Range | Unit | Accuracy | Calibration |
| --- | --- | --- | --- | --- |
| Temperature | 0 to +70 | °C | ±3°C | Internal / External |
| Voltage | 3.0 to 3.6 | V | ±3% | Internal / External |
| Bias Current | 0 to 100 | mA | ±10% | Internal / External |
| TX Power | -9 to 0 | dBm | ±3dB | Internal / External |
| RX Power | -23 to -3 | dBm | ±3dB | Internal / External |

Digital Diagnostic Memory Map

The transceivers provide serial ID memory contents and diagnostic information about the present operating conditions by the 2-wire serial interface (SCL, SDA).

The diagnostic information with internal calibration or external calibration all are implemented, including received power monitoring, transmitted power monitoring, bias current monitoring, supply voltage monitoring and temperature monitoring.

The digital diagnostic memory map specific data field defines as following.

Pin Definitions

Pin Diagram

Pin Descriptions

| Pin | Signal Name | Description | Plug Seq. | Notes |
| --- | --- | --- | --- | --- |
| 1 | VEET | Transmitter Ground | 1 |  |
| 2 | TX FAULT | Transmitter Fault Indication | 3 | Note 1 |
| 3 | TX DISABLE | Transmitter Disable | 3 | Note 2 |
| 4 | MOD_DEF(2) | SDA Serial Data Signal | 3 | Note 3 |
| 5 | MOD_DEF(1) | SCL Serial Clock Signal | 3 | Note 3 |
| 6 | MOD_DEF(0) | TTL Low | 3 | Note 3 |
| 7 | Rate Select | Not Connected | 3 |  |
| 8 | LOS | Loss of Signal | 3 | Note 4 |
| 9 | VEER | Receiver ground | 1 |  |
| 10 | VEER | Receiver ground | 1 |  |
| 11 | VEER | Receiver ground | 1 |  |
| 12 | RD- | Inv. Received Data Out | 3 | Note 5 |
| 13 | RD+ | Received Data Out | 3 | Note 5 |
| 14 | VEER | Receiver ground | 1 |  |
| 15 | VCCR | Receiver Power Supply | 2 |  |
| 16 | VCCT | Transmitter Power Supply | 2 |  |
| 17 | VEET | Transmitter Ground | 1 |  |
| 18 | TD+ | Transmit Data In | 3 | Note 6 |
| 19 | TD- | Inv. Transmit Data In | 3 | Note 6 |
| 20 | VEET | Transmitter Ground | 1 |  |

Notes:

Plug Seq.: Pin engagement sequence during hot plugging.

- TX Fault is an open collector output, which should be pulled up with a 4.7k~10kΩ resistor on the host board to a voltage between 2.0V and Vcc+0.3V. Logic 0 indicates normal operation; Logic 1 indicates a laser fault of some kind. In the low state, the output will be pulled to less than 0.8V.

- TX Disable is an input that is used to shut down the transmitter optical output. It is pulled up within the module with a 4.7k~10kΩ resistor. Its states are:

Low (0 to 0.8V):	Transmitter on

(>0.8V, < 2.0V):	Undefined

High (2.0 to 3.465V):	Transmitter Disabled Open:	Transmitter Disabled

- Mod-Def 0,1,2. These are the module definition pins. They should be pulled up with a 4.7k~10kΩ resistor on the host board. The pull-up voltage shall be VccT or VccR.

Mod-Def 0 is grounded by the module to indicate that the module is present Mod-Def 1 is the clock line of two wire serial interface for serial ID

Mod-Def 2 is the data line of two wire serial interface for serial ID

- LOS is an open collector output, which should be pulled up with a 4.7k~10kΩ resistor. Pull up voltage between 2.0V and Vcc+0.3V. Logic 1 indicates loss of signal; Logic 0 indicates normal operation. In the low state, the output will be pulled to less than 0.8V.

- RD-/+: These are the differential receiver outputs. They are internally AC-coupled 100 differential lines which should be terminated with 100Ω (differential) at the user SERDES.

- TD-/+: These are the differential transmitter inputs. They are internally AC-coupled, differential lines with 100Ω differential termination inside the module.

Recommended Interface Circuit