10Gb/s 100KM SFP+ 1550nm Transceivers

# Features

The  1550nm  cooled  EML  laser  based  10Gigabit SFP+ Transceiver is designed to transmit and receive serial optical data over single mode optical fiber with 100KM.

They   are   compliant   with   SFF-8431,SFF-8432, 10GFC  Rev  4.0,  and  10GBASE-ZR.  The  transmitter converts serial  CML  electrical data  into  serial  optical data compliant with the  IEEE  802.3ae standard. The receiver  converts  serial  optical  data  into  serial  CML electrical data.Digital diagnostics functions are available via a 2-wire serial interface, as specified in SFF-8472.

l Compliant to SFP+ MSA

l Fully RoHS Compliant

l All metal housing for

superior EMI performance

l Operating data rate up to 11.3Gbps

l Cooled EML DFB Laser

l High sensitivity APD photodiode and TIA

l LC duplex connector

l Hot pluggable 20pin connector

l Low power consumption <1.8 W

l -5℃ to 70℃  operating wide temperature range

l Single +3.3V±5% power supply

l Digital Monitoring

SFF-8472 Rev 10.2 compliant

l Real time monitoring of： Transmitted optical power Received optical power

Laser bias current

Applications

l 10GBASE-ZR

l 10G Fiber Channel

# Specifications

(Tc=-5  ℃ to 70  ℃ and Vcc= 3.14 to 3.46V)

| Parameter | Symbol | Unit | Min | Typ | Max | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Nominal Wavelength | λ | nm | 1530 |  | 1565 |  |
| Side Mode Suppression Ratio | SMSR | dB | 30 |  |  |  |
| Spectral Width(-20dB) | Δλ | nm |  |  | 0.3 |  |
| Optical Output Power | Pav | dBm | 0 |  | 5 |  |
| Extinction Ratio | ER | dB | 8.2 |  |  |  |
| Average Launch Power of OFF Transmitter | POFF | dBm |  |  | -30 |  |
| Relative Intensity Noise | RIN | dB/Hz |  |  | -128 |  |
| Receiver |  |  |  |  |  |  |
| Center Wavelength | λC | nm | 1260 |  | 1620 |  |
| Receiver Sensitivity@10.3125Gb/s | RSENSE | dBm |  |  | -26 | 1 |
| Receiver Sensitivity at 1600ps/nm @ 10.3125Gb/s | RSENSE | dBm |  |  | -22 | 1.2 |
| Overload |  | dBm | -7 |  |  |  |
| Optical Return Loss |  | dB | 27 |  | - |  |
| LOS Assert | LOSA | dBm | -36 |  |  |  |
| LOS De-Assert LOS | LOSD | dBm |  |  | -27 |  |
| LOS Hysteresis |  | dB | 0.5 |  | 6 |  |

Note  1:  Measured  at   1528-1600nm,ER>8.2dBm,  PRBS  231-1  and  BER better than or equal to 10E-12;

2: loopback using 100KM fiber (SMF-28).

Ordering Information

| Part No. | Specifications |  |  |  |  |  | Application |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Package | Laser | Optical Power | Detector | Sensitivity | Temp | Reach | Other |  |
| JH-10G-ZR-100 | SFP+ | 1550nm EML | 0 ~+5dBm | APD | < -26dBm | -5~70℃ | 100KM | DDM | 10GBASE-ZR/ZW 10G Fiber Channel |

# Block diagram

Laser Driver

Controller

MCU

FLASH

Post Amplifier

TOSA

ROSA

TIA

S F P

+ C O N N E C T O R

## Figure 1.Transceiver functional diagram

Absolute Maximum Ratings

| Parameter | Symbol | Unit | Min | Max |
| --- | --- | --- | --- | --- |
| Storage Temperature Range | Ts | oC | -40 | 85 |
| Relative Humidity | RH | % | 0 | 95 |
| Recommended Operating Conditions |  |  |  |  |
| Parameter | Symbol | Unit Min | Typ | Max |
| Operating Case Temperature Range | Tc | oC -5 |  | 70 |
| Power Supply Voltage | Vcc | V 3.14 | 3.3 | 3.46 |
| Bit Rate | BR | Gb/s |  | 11.3 |
| Bit Error Ratio | BER |  |  | 10-12 |
| Max Supported Link Length | L | Km |  | 100 |

Electric Ports Definition

| Parameter | Symbol | Unit | Min | Typ | Max Note |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | V | 3.14 | 3.3 | 3.46 |
| Power consumption | Por | W |  |  | 1.8 |
| Transmitter |  |  |  |  |  |
| Input Differential Impedance | RIN | Ω | 80 | 100 | 120 |
| Differential Data Input | VIN | mVp-p | 180 |  | 700 |
| Transmit Disable Voltage | VDIS | V | 2 |  | VCCHOST |
| Transmit Enable Voltage | VEN | V | VEE |  | VEE+0.8 |
| Transmit Fault Assert Voltage | VFA | V | 2 |  | VCCHOST |
| Transmit Fault De-Assert Voltage | VFDA | V | VEE |  | VEE+0.4 |
| Receiver |  |  |  |  |  |
| Differential Data Output | VOD | mVp-p | 300 |  | 850 |
| Output Rise Time | tRISE | pS | 25 |  |  |
| Output Fall Time | tFALL | pS | 25 |  |  |
| LOS Fault | VLOSFT | V | 2 |  | VCCHOST |
| LOS Normal | VLOSNR | V | VEE |  | VEE+0.4 |

# Pin function definitions

## Figure 2.Pin function definitions

Table 1: Transceiver pin descriptions

| Pin Number | Symbol | Name | Description |
| --- | --- | --- | --- |
| 1,17,20 | VeeT | Transmitter Signal Ground | These pins should be connected to signal ground on the host board. |
| 2 | TX Fault | Transmitter Fault Out (OC) | Logic “1” Output = Laser Fault (Laser off before t_fault) Logic “0” Output = Normal Operation |

| This pin is open collector compatible, and should be pulled up to Host Vcc with a 10kΩ resistor. |  |  |  |
| --- | --- | --- | --- |
| 3 | TX Disable | Transmitter Disable In (LVTTL) | Logic “1” Input (or no connection) = Laser off Logic “0” Input = Laser on This pin is internally pulled up to VccT with a 10 kΩ resistor. |
| 4 | SDA | Module Definition Identifiers | Serial ID with SFF 8472 Diagnostics Module Definition pins should be pulled up to Host Vcc with 10 kΩ resistors. |
| 5 | SCL |  |  |
| 6 | MOD-ABS |  |  |
| 7 | RS0 | Receiver Rate Select (LVTTL) Transmitter Rate Select (LVTTL) | These pins have an internal 30kΩ pull-down to ground. A signal on either of these pins will not affect module performance. |
| 9 | RS1 |  |  |
| 8 | LOS | Loss of Signal Out (OC) | Sufficient optical signal for potential BER < 1x10- 12 = Logic “0” Insufficient optical signal for potential BER < 1x10- 12 = Logic “1” This pin is open collector compatible, and should be pulled up to Host Vcc with a 10kΩ resistor. |
| 10,11,14 | VeeR | Receiver Signal Ground | These pins should be connected to signal ground on the host board. |
| 12 | RD- | Receiver Negative DATA Out (CML) | Light on = Logic “0” Output Receiver DATA output is internally AC coupled and series terminated with a 50Ω resistor. |
| 13 | RD+ | Receiver Positive DATA Out (CML) | Light on = Logic “1” Output Receiver DATA output is internally AC coupled and series terminated with a 50Ω resistor. |
| 15 | VccR | Receiver Power Supply | This pin should be connected to a filtered +3.3V power supply on the host board. See Figure 3.Recommended power supply filter |
| 16 | VccT | Transmitter Power Supply | This pin should be connected to a filtered +3.3V power supply on the host board. See Figure 3.Recommended power supply filter |
| 18 | TD+ | Transmitter Positive DATA In (CML) | Logic “1” Input = Light on Transmitter DATA inputs are internally AC coupled and terminated with a differential 100Ω resistor. |
| 19 | TD- | Transmitter Negative DATA In (CML) | Logic “0” Input = Light on Transmitter DATA inputs are internally AC coupled and terminated with a differential 100Ω resistor. |

Typical Application Circuit

Package Outline

Regulatory Compliance

| Feature | Test Method | Performance |
| --- | --- | --- |
| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883C Method 3015.7 | Class 1 (> 1500 Volts) |
| Electrostatic Discharge (ESD) to the Duplex LC Receptacle | Variation of IEC 61000-4-2 | Typically, no damage occurs with 15 kV when the duplex LC connector receptacle is contacted by a Human Body Model probe. |
| Electrostatic Interference (EMI) | CISPR22 ITE Class B EN55022 Class B FCC Class B | Compliant with standards |
| Immunity | IEC61000-4-3 Class 2 EN55024 | Typically show no measurable effect from a 3V/m field swept from 80 to 1000MHz applied to the transceiver without a chassis enclosure. |
| RoHS Compliance |  | Less than 1000 ppm of cadmium, lead, mercury, hexavalent chromium, polybrominated biphenyls, and polybrominated biphenyl ethers. |