Product Specification

10Gb/s 80Km SFP+ BIDI Transceivers

Features

- Compliant to SFP+ MSA

- Fully RoHS Compliant

- All metal housing for superior EMI performance

- Operating data rate up to 11.3Gbps

- 1490nm/1550nm Laser

- High sensitivity Pin photodiode and TIA

- LC single connector

- Hot pluggable 20pin connector

- Low power consumption <1W

- 0℃ to 70℃  operating wide temperature range

- Single +3.3V±5% power supply

- Digital Monitoring SFF-8472 Rev 10.2 compliant

Applications

- 10GBASE-LR/LW

- 10G Fiber Channel

- SFP+ MSA (SFF-8472)

- IEEE802.3ae

The laser based 10Gigabit SFP+ Transceiver is designed to transmit and receive serial optical data over single mode optical fiber with 80Km.

They are compliant with SFF-8431,SFF-8432, 10GFC Rev 4.0,. The transmitter converts serial CML electrical data into serial optical data compliant with the IEEE 802.3ae standard. The receiver converts serial optical data into serial CML electrical data.Digital diagnostics functions are available via a 2-wire serial interface, as specified in SFF-8472.

| Part No. | Specifications |  |  |  |  |  |  |  |  | Application |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Package | Data rate | Laser | Optical Power | Detector | Sensitivity | Temp | Reach | Other |  |
| JH-10G-WDM -A-80L-45 | SFP+ | Up to 11.3G | 1490nm/ 1550nm | 0~4dBm | PIN | < -23dBm | 0~70℃ | 80km | DDM | 10GBASE-LR/LW |

Pin function definitions

Figure 1.Pin function definitions

Table 1: Transceiver pin descriptions

| Pin Number | Symbol | Name | Description |
| --- | --- | --- | --- |
| 1,17,20 | VeeT | Transmitter Signal Ground | These pins should be connected to signal ground on the host board. |
| 2 | TX Fault | Transmitter Fault Out (OC) | Logic “1” Output = Laser Fault (Laser off before t_fault) Logic “0” Output = Normal Operation This pin is open collector compatible, and should be pulled up to Host Vcc with a 10kΩ resistor. |
| 3 | TX Disable | Transmitter Disable In (LVTTL) | Logic “1” Input (or no connection) = Laser off Logic “0” Input = Laser on This pin is internally pulled up to VccT with a 10 kΩ resistor. |
| 4 | SDA | Module Definition Identifiers | Serial ID with SFF 8472 Diagnostics Module Definition pins should be pulled up to Host Vcc with 10 kΩ resistors. |
| 5 | SCL |  |  |
| 6 | MOD-ABS |  |  |
| 7 | RS0 | Receiver Rate Select (LVTTL) Transmitter Rate Select (LVTTL) | These pins have an internal 30kΩ pull-down to ground. A signal on either of these pins will not affect module performance. |
| 9 | RS1 |  |  |
| 8 | LOS | Loss of Signal Out (OC) | Sufficient optical signal for potential BER < 1x10-12 = Logic “0” Insufficient optical signal for potential BER < 1x10-12 = Logic “1” This pin is open collector compatible, and should be pulled up to Host Vcc with a 10kΩ resistor. |
| 10,11,14 | VeeR | Receiver Signal Ground | These pins should be connected to signal ground on the host board. |
| 12 | RD- | Receiver Negative DATA Out (CML) | Light on = Logic “0” Output Receiver DATA output is internally AC coupled and series terminated with a 50Ω resistor. |
| 13 | RD+ | Receiver Positive DATA Out (CML) | Light on = Logic “1” Output Receiver DATA output is internally AC coupled and series terminated with a 50Ω resistor. |
| 15 | VccR | Receiver Power Supply | This pin should be connected to a filtered +3.3V power supply on the host board. See Figure 3.Recommended power supply filter |
| 16 | VccT | Transmitter Power Supply | This pin should be connected to a filtered +3.3V power supply on the host board. See Figure 3.Recommended power supply filter |
| 18 | TD+ | Transmitter Positive DATA In (CML) | Logic “1” Input = Light on Transmitter DATA inputs are internally AC coupled and terminated with a differential 100Ω resistor. |
| 19 | TD- | Transmitter Negative DATA In (CML) | Logic “0” Input = Light on Transmitter DATA inputs are internally AC coupled and terminated with a differential 100Ω resistor. |

Absolute Maximum Ratings

| Parameter | Symbol | Unit | Min | Max |
| --- | --- | --- | --- | --- |
| Storage Temperature Range | Ts | oC | -40 | 85 |
| Relative Humidity | RH | % | 0 | 95 |
| Maximum Supply Voltage | Vcc3 | V | -0.5 | 3.6 |

General Specifications

| Parameter | Symbol | Min | Typ | Max | Units | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| Bit Rate | BR |  |  | 11.3 | Gb/s | 1 |
| Bit Error Ratio | BER |  |  | 10E-12 |  | 2 |
| Max. Supported Link Length | LMAX |  |  | 80 | km | 1 |

Notes:

1. 10GBASE-LR, 10GBASE-LW, 1200-SM-LL-L 10GFC.

2. Tested with a 231-1 PRBS.

Recommended Operating Conditions

| Parameter | Symbol | Unit | Min | Typ | Max |
| --- | --- | --- | --- | --- | --- |
| Operating Case Temperature Range | Tc | oC | 0 |  | 70 |
| Power Supply Voltage | Vcc | V | 3.14 | 3.3 | 3.46 |
| Bit Rate | BR | Gb/s |  |  | 11.3 |
| Bit Error Ratio | BER |  |  |  | 10-12 |
| Max Supported Link Length | L | Km |  |  | 80 |

Electric Ports Definition

| Parameter | Symbol | Unit | Min | Typ | Max | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | V | 3.14 | 3.3 | 3.46 |  |
| Module Power | Icc | mW |  |  | 1000 |  |
| Transmitter |  |  |  |  |  |  |
| Input Differential Impedance | RIN | Ω | 80 | 100 | 120 |  |
| Differential Data Input | VIN | mVp-p | 180 |  | 700 |  |
| Transmit Disable Voltage | VDIS | V | 2 |  | VCCHOST |  |
| Transmit Enable Voltage | VEN | V | VEE |  | VEE+0.8 |  |
| Transmit Fault Assert Voltage | VFA | V | 2 |  | VCCHOST |  |
| Transmit Fault De-Assert Voltage | VFDA | V | VEE |  | VEE+0.4 |  |
| Receiver |  |  |  |  |  |  |
| Differential Data Output | VOD | mVp-p | 300 |  | 850 |  |
| Output Rise Time | tRISE | pS | 25 |  |  |  |
| Output Fall Time | tFALL | pS | 25 |  |  |  |
| LOS Fault | VLOSFT | V | 2 |  | VCCHOST |  |
| LOS Normal | VLOSNR | V | VEE |  | VEE+0.4 |  |

| Parameter | Symbol | Min | Typ | Max | Note | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Optical Transmitter Characteristics |  |  |  |  |  |  |
| Nominal Wavelength (FLS10C411S) | λC | 1480 | 1490 | 1500 |  | nm |
| Nominal Wavelength (FLS10C412S) | λC | 1540 | 1550 | 1560 |  | nm |
| Optical Output Power | Pav | 0 |  | 4 |  | dBm |
| Extinction Ratio | ER | 3.5 |  |  |  | dB |
| Average Launch Power of OFF Transmitter | POFF |  |  | -35 |  | dBm |
| Relative Intensity Noise | RIN |  |  | -128 |  | dB/Hz |
| Optical Receiver Characteristics |  |  |  |  |  |  |
| Center Wavelength (FLS10C411S) | λC | 1540 | 1550 | 1560 |  | nm |
| Center Wavelength (FLS10C412S) | λC | 1480 | 1490 | 1500 |  | nm |
| Receiver Sensitivity@10.3Gb/s | RSENSE |  |  | -23 | 1 | dBm |
| Receiver Overload |  | 0.5 |  |  |  | dBm |
| Optical Return Loss |  | 27 |  | - |  | dB |
| LOS Assert | LOSA | -30 |  |  |  | dBm |
| LOS De-Assert LOS | LOSD |  |  | -17 |  | dBm |
| LOS Hysteresis |  | 0.5 |  |  |  | dB |

Optical Characteristics (Tc=0 oC to 70 oC and Vcc= 3.14 to 3.46)

Note1: Measured at 10.3125Gbps,ER>3.5dBm, PRBS 231-1 and BER better than or equal to 10E-12;

Typical Application Circuit

Figure 2. Typical application circuit

Mechanical Dimensions

Figure 3. Module Mechanical Dimensions

Digital Diagnostics Functions

As defined by the SFF-8472, Our SFP+ transceivers provide digital diagnostic functions via a 2-wire serial interface, which allows real-time access to the following operating parameters:

- Transceiver temperature

- Laser bias current

- Transmitted optical power

- Received optical power

- Transceiver supply voltage

It also provides a sophisticated system of alarm and warning flags, which may be used to alert end-users when particular operating parameters are outside of a factory-set normal range. The operating and diagnostics information is monitored and reported by a Digital Diagnostics Transceiver Controller (DDTC) inside the transceiver, which is accessed through the 2-wire serial interface. When the serial protocol is activated, the serial clock signal (SCL pin) is generated by the host. The positive edge clocks data into the SFP+ transceiver into those segments of its memory map that are not write-protected. The negative edge clocks data from the SFP+ transceiver. The serial data signal (SDA pin) is bi-directional for serial data transfer. The host uses SDA in conjunction with SCL to mark the start and end of serial protocol activation. The memories are organized as a series of 8-bit data words that can be addressed individually or sequentially. The 2-wire serial interface provides sequential or random access to the 8 bit parameters, addressed from 000h to the maximum address of the memory.For more detailed information, including memory map definitions, please see the SFF-8472 documentation1.